| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal on Broadcast Burst Resource Indication |  |
| Date Submitted | 2010-03-04 |  |
| Source(s) | Xianming Chen, Yanfeng Guan, Huiying Fang, Lei Zhang ZTE Corporation | guan.yanfeng@zte.com.cn |
| Re: | Comment on “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, P802.16m/D4” 16.3.6.5.2.1 Non-user-specific A-MAP IE & 16.3.6.5.2.4 Assignment A-MAP IE |  |
| Abstract | This contribution proposes text on Broadcast Burst Resource Indication. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal on Broadcast Burst Resource Indication

Xianming Chen, Yanfeng Guan, Huiying Fang, Lei Zhang

ZTE Corporation

- Introduction

In the current 802.16m draft amendment [1], broadcast assignment AMAP is defined and used for broadcast burst attribute description and resource indication.

- Problem

When a unicast assignment AMAP, such as DL Basic Assignment AMAP, was not received correctly by an AMS, the corresponding burst could be re-transmitted by ABS through HARQ function.

When a broadcast assignment AMAP was not received correctly by an AMS, the corresponding burst could not be re-transmitted by ABS, as HARQ function could not be used for broadcast burst transmission.

Therefore, it is necessary to make the probability of decoding broadcast assignment AMAP correctly higher than that of decoding unicast assignment AMAP correctly.

In other hand, both broadcast assignment AMAP and NUS need be received by each AMS, and in reuse 1 frequency partition, current NUS code rate (1/12) is more robust than unicast assignment AMAP code rate (1/4 or 1/8), but current broadcast assignment AMAP code rate is the same as unicast/multicast assignment AMAP code rate (1/4 or 1/8) in D4.

In addition, CRC detection method for broadcast A-AMAP is not fully accurate, and error detection may happen.

Therefore, it is not reasonable for current broadcast burst resource indication method using broadcast assignment AMAP.

- References

[1] "DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems Advanced Air Interface", P802.16m/D4 Feb. 2010.

- Proposed Text