| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to resource allocation for UL control channel in FFR (16.3.8.3.3) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Chiwoo Lim, Hwasun Yoo, Jaeweon Cho, Hokyu Choi, Heewon Kang Samsung Electronics Co., Ltd. | chiwoo.lim@samsung.com |
| Re: | IEEE 802.16-10/0011, “IEEE 802.16 Working Group Letter Ballot#31” Target topic: “IEEE P802.16m/D4, section 16.3.8.3.3”. |  |
| Abstract | The contribution proposes the text changes to resource allocation for UL control channel in FFR of the 802.16m draft amendment, IEEE P802.16m/D4. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes to resource allocation for UL control channels 
in FFR (16.3.8.3.3)

Chiwoo Lim, Hwasun Yoo, Jaeweon Cho, Hokyu Choi, Heewon Kang
Samsung Electronics Co., Ltd.

- Introduction

The contribution proposes the text changes of the resource allocation for UL control channels in FFR to be included in the 802.16m amendment. In the current draft amendment [1], the UL control channels can be used in the reuse 1 partition and the power-boosted reuse 3 when FFR is used in an UL AAI subframe. However, there is no reason to use both partitions. It can be a restriction because each FFR region may have different DLRUs according to FFR configuration. Moreover, signaling for UL control channel can be complex.

- References

[1] IEEE P802.16m/D4, “P802.16m DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”, 2010-02-03

- Proposed changes for 802.16m draft amendment

---------------------------------------------------  Text Start  ---------------------------------------------------

[Remedy-1: Modify the text on page 596 line 16-18 as following text]

If FFR is used in an UL AAI subframe, the UL control channels can be used in the reuse 1 partition and the power-boosted reuse 3 partition.

IF FFR is used in an UL AAI subframe, the UL control channels shall be located in a frequency partition. A frequency partition can be either reuse 1 or power-boosted reuse 3 and called UL primary frequency partition. If reuse 1 exists, it shall be a UL primary frequency partition; otherwise, power-boosted reuse 3 is UL primary frequency partition.

[Remedy-2: Modify the text on page 596 line 23 as following text]

The number of bandwidth request channels in frequency partition FPi in a UL AAI subframe is Nbwr.

The number of bandwidth request channels in UL primary frequency partition in a UL AAI subframe is Nbwr.

[Remedy-3: Modify the text on page 596 line 40-49 as following text]

Let UL_FEEDBACK_SIZE distributed LRUs in frequency partition FPi be reserved for feedback channels in the units of LRU. The number of feedback channels in frequency partition FPi, is LFB,FPi.

LFB,FPi = Nfb∙UL_FEEDBACK_SIZE                                         (267)

Let UL_FEEDBACK_SIZE distributed LRUs in UL primary frequency partition be reserved for feedback channels in the units of LRU. The number of feedback channels in UL primary frequency partition, is LFB.

LFB = Nfb∙UL_FEEDBACK_SIZE                                           (XXX)

[Remedy-4: Modify the term on page 596 line 56 as following term]

3∙LFB,FPi -1

3∙LFB -1

[Remedy-5: Modify the term on page 597 line 14 as following term]

LFB,FPi -1

LFB -1

[Remedy-6: Modify the term on page 597 line 21 as following term]

LHFB,FPi

LHFB

[Remedy-7: Modify the term on page 597 line 50 as following term]

LHFB,FPi -1

LHFB -1

---------------------------------------------------  Text End  ---------------------------------------------------