| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to the Bandwidth request channels(16.3.8.3.3.1) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Chiwoo Lim, Hwasun Yoo, Jaeweon Cho, Hokyu Choi, Heewon Kang Samsung Electronics Co., Ltd. | chiwoo.lim@samsung.com |
| Re: | IEEE 802.16-10/0011, “IEEE 802.16 Working Group Letter Ballot#31” Target topic: “IEEE P802.16m/D4, section 16.3.8.3.3.1”. |  |
| Abstract | The contribution proposes the text changes to the Bandwidth request channels of the 802.16m draft amendment, IEEE P802.16m/D4. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes to the Bandwidth request channels (16.3.8.3.3.1)

Chiwoo Lim, Hwasun Yoo, Jaeweon Cho, Hokyu Choi, Heewon Kang
Samsung Electronics Co., Ltd.

- Introduction

The contribution proposes the text changes of the bandwidth request channels to be included in the 802.16m amendment. In the current draft amendment [1], there is no description of construction way for the bandwidth request channels when frame structure is supporting the WirelessMAN-OFDMA with FDM-based uplink PUSC zone. In addition, there is no description for tile indexing.

- References

[1] IEEE P802.16m/D4, “P802.16m DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”, 2010-02-03

- Proposed changes for 802.16m draft amendment

---------------------------------------------------  Text Start  ---------------------------------------------------

[Remedy-1: Modify the text on page 596 line 28-32 as following text]

In MZone, the bandwidth request channels are of same size as LRUs, i.e. three 6-by-6 tiles. In LZone with PUSC, the bandwidth request channels consist of three 4-by-6 tiles. The bandwidth request channels use LRUs constructed from the tile permutation specified in 16.3.8.3.2.

The bandwidth request channel is of same size as a LRU, i.e. three 6-by-6 tiles. The bandwidth request channel uses a LRU constructed from the tile permutation specified in 16.3.8.3.2.

When frame structure is supporting the WirelessMAN-OFDMA with FDM based uplink PUSC zone, the bandwidth request channels consist of three 4-by-6 tiles. A DLRU consists of two bandwidth request channels and the DLRU is constructed from 16.3.8.3.5.3. A bandwidth request channel is formed by three consecutive tiles. One bandwidth request channel is formed by tile (0), tile (1), and tile (2). Another bandwidth request channel is formed by tile (3), tile (4), and tile (5).

[Remedy-2: Insert the text on page 601 line34 as following text]

When frame structure is supporting the WirelessMAN-OFDMA with FDM-based uplink PUSC zone, a LRU consists of six tiles and the tile indexing is same as the Wireless MAN-OFDMA.

---------------------------------------------------  Text End  ---------------------------------------------------