| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Femto ABS Indicator in S-SFH (16.3.6.5.1.2) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Jin Lee, Hyunwoo Lee, Jinsoo Choi, Youngsoo Yuk, Kiseon Ryu, Ronny Kim, JinSam Kwak LG Electronics, Inc. | +82-31-450-2067 [jin1.lee, hyunwoolee.lee, js.choi, sixs, ksryu, ronnykim, jinsam.kwak] @lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot #31 |  |
| Abstract | This contribution is to purpose Femto ABS indicator in S-SFH |  |
| Purpose | Update P802.16m/D4 by discussion and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Femto ABS Indicator in S-SFH

Jin Lee, Hyunwoo Lee, Jinsoo Choi, Youngsoo Yuk, Kiseon Ryu, Ronny Kim, JinSam Kwak

LG Electronics, Inc.

- Introduction

- According to D4 [1], macro ABS and non macro ABS are classified by hard partitioning of SA-preamble sequence. Soft portioning information is carried in SP3 IE to identify public ABS (OSG Femto ABS, relay, hotzone ABS, etc) and CSG Femto ABS. In other words, AMS is not able to discover Femto ABS itself even by obtaining SP3 IE in S-SFH. AAI_SCD-MSG which will be transmitted later may aid AMS to detect the type of Femto ABS (e.g., OSG/CSG-closed/CSG-open Femto ABS). This may cause significant delay for AMS to access Femto ABS especially to its CSG Femto ABS. Also, if soft portioning of SA-preamble sequence is used to categorize CSG Femto ABS, this may lead complexity in order to convert CSG-Closed Femto ABS to CSG-Open Femto ABS for interference mitigation defined in 16.4.11.

- Therefore, this contribution proposes all types of Femto ABS can be identified at the time of transmitting SP3 by including 1 bit indicator as SFH contents.

- Rationale of modifications

- According to the current D4 [1], it is believed that AMS should wait to receive AAI_SCD-MSG to identify whether the detected preamble indicates Femto ABS or not since the AMS can only categorize public ABS with the aid of SP3 IE. In other words, soft partitioning information via SP3 IE is not sufficient for AMS to identify whether the current detected preamble indicates Femto ABS or not since public ABS represents OSG Femto ABS, relay and hotzone ABS etc.

- Additionally, AMS can not interpret ranging channel information related to Femto ABS in SP1 IE until before acquiring the AAI_SCD-MSG because the different structures of ranging channel are used between Femto ABS and others. Using soft preamble portioning for CSG Femto ABS (CSG-Closed or CSG-Open) may not be efficient to give opportunity for CSG-Closed Femto ABS to convert its cell type for the purpose of interference mitigation.

- To resolve the aforementioned problems, we propose 1 bit indicator be used commonly in both public and CSG Femto ABS. The indicator is included in SP3 IE along with currently defined soft partitioning information for non macro ABS.

- The table1, 2 below depict how AMS can detect cell type by adding 1 bit indicator in SP3. The 1 bit indicator can be used in common for public and CSG Femto ABS but it has different meaning in each case.

- For example, if the detected preamble is within the range of ‘Public’ and the Femto ABS indicator is set to ‘0’, the AMS can identify the cell is OSG Femto ABS. In case the detected preamble is within the range of ‘CSG’ and the Femto ABS indicator is set to ‘1’, it indicates CSG-Closed Femto ABS.

- Further, AAI_SCD-MSG may need to transmit the information to classify non Femto ABSs.

| Classification | SA-Preamble Hard Partitioning | SA-Preamble Soft Partitioning | Femto ABS indicator in SP3 |
| --- | --- | --- | --- |
| Cell Type | Macro |  |  |
|  | Non Macro | Public | OSG Femto ABS |
|  |  |  | Non Femto ABS |
|  |  | CSG Femto ABS | CSG-Open Femto ABS |
|  |  |  | CSG-Closed Femto ABS |

- Table 1. Cell Type Categorization

Femto ABS indicator

| Cell Type | - Femto ABS indicator : 0 | - Femto ABS indicator :1 |
| --- | --- | --- |
| Public | - OSG Femto ABS | - Non Femto ABS |
| CSG Femto ABS | - CSG-Open Femto ABS | - CSG-Closed Femto ABS |

- Table 2. Usage of ‘Femto ABS indicator’

- Proposed Texts

---------------------------------------------------------------Text Start ----------------------------------------------------------

Remedy

[Include blue texts as suggested below: Page 454, Table 813]

Table 813 –S-SFH SP3 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| ….. |  |  |
| SA-Preamble sequence soft partitioning information | 4 | Specifies the partition information of SA-Preamble sequence for non-macro ABS as public and CSG femto BS |
| If ( public ABS ) { |  | Indicates the public ABS, when 86<=Idx<=z in the 16.3.6.1.2 where z is the SA-Preamble sequence soft partitioning information. |
| Femto ABS Indicator | 1 | 0b0 : OSG Femto ABS 0b1 : non-femto ABS |
| } else if ( CSG femto ABS ) { |  | Indicates the CSG femto ABS, when z+1<=Idx<=255 in the 16.3.6.1.2 where z is the SA-Preamble sequence soft partitioning information. |
| Femto ABS Indicator | 1 | 0b0 : CSG-Open Femto ABS 0b1 : CSG-Closed Femto ABS |
| } else { |  |  |
| Reserved | 1 |  |
| } |  |  |
| ….. |  |  |

---------------------------------------------------------------Text End -----------------------------------------------------------

- Reference

[1] IEEE P802.16m/D4, “Part 16: Air Interface for Broadband Wireless Access Systems : Advanced Air Interface,” February 2010.