| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Cell bar in Femto ABS (16.4.7.3) |  |
| Date Submitted | 2026-07-28 |  |
| Source(s) | Jin Lee, Inuk Jung, Kiseon Ryu, Yoongsoo Yuk, Ronny Kim and JinSam Kwak LG Electronics | {jin1.lee, cooper, ksryu, sixs, ronnykim, samji }@lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Comments on IEEE P802.16m/D4 for IEEE 802.16 WG Letter Ballot #31 |  |
| Abstract | The contribution proposes text changes IEEE P802.16m/D4 regarding CSGID transmission for snooping |  |
| Purpose | Discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Cell bar in Femto ABS (16.4.7.3)

Jin Lee, Inuk Jung, Kiseon Ryu, Youngsoo Yuk, Ronny Kim and JinSam Kwak

LG Electronics

- Introduction

- By definition of CSG-Open Femto ABS, when resources in CSG-Open Femto ABS are not being consumed by CSG users, those resources are made available to the service provider’s other users (Non CSG member but authorized AMSs) or roaming users. Then, when most of its resources are being used by CSG AMS, CSG-Open Femto ABS shall protect non member AMS from network entry/HO.  Additionally, cell bar shall also be used to protect any AMS (re)entry when a Femto ABS is going to be out of service.

- Therefore, we propose Cell bar which indicates prevention of network entry for non member AMS in a CSG -
Open Femto ABS when a certain condition is met. The CSG-Open Femto ABS should convert to a CSG-Closed Femto ABS and transmit cell bar set to 1 in case of out of service.

- Rationale of modifications

- Table 1 describes the usage of cell bar for each cell type.

- Cell bar bit set to 0 in CSG-Closed Femto ABS shall be ignored by its non member AMS. By definition of CSG-Closed Femto ABS, NE from non member AMS shall always be protected.

- When most of resources are being used by memberAMS, CSG-Open Femto ABS should cell bar set to 1 in order to protect non member AMS from NE/HO

- If a CSG-Open Femto ABS is going to disable the air interface, it should communicate with a network entity to see if it can convert to a CSG-Closed Femto ABS. If allowed, the CSG-Open Femto ABS converts to a CSG-Closed Femto ABS and sets cell bar to 1.

| Cell bar Cell Type | - Macro | - OSG | - CSG-Open | - CSG-Closed |
| --- | --- | --- | --- | --- |
| - cell bar : 0 | - Allow NE from any AMS | - Allow NE from any AMS | - Allow NE from any AMS | - Allow NE only from member AMS |
| - cell bar : 1 | - Prevent NE from any AMS | - Prevent NE from any AMS | - Prevent NE from non member AMS | - Prevent NE from any AMS |

- Table 1. Usage of cell bar

- Purposed Texts

-------------------------------------------------- Begin Proposed Text Changes----------------------------------------------

Remedy 1 :

[Include the following texts at the end of the section 16.4.7.3, in line 47 page 699]

16.4.7.3 Femto ABS Access Restrictions

AMS should not attempt to access or initiate handover to a CSG-closed femto ABS, which is not contained in the CSG whitelist, except in case of emergency call or manual Femto BS selection. (see 16.4.7.2) Cell bar set to 0 in CSG-Closed Femto ABS shall be ignored by its non member AMSs. If cell bar bit in CSG-Open femto ABS is set to 1, non member AMS shall not attempt network entry or handover to the CSG-Open femto ABS.

Remedy 2 :

[Modify texts as suggested below, in line 8 page 705]

When a Femto ABS is going to disable the air interface, it shall set the cell bar bit to 1 in order to prevent AMS (re)entry and should broadcast the air interface is going down through AAI_SON-ADV with Action type 4 with the appropriate Reason bit 0b00 and 0b01, as specified in section <<16.2.3.6>>, repeatedly until it disable the air interface. In case of a CSG-Open Femto ABS, it should convert to a CSG-Closed Femto ABS before transmitting cell bar bit to 1 to prevent any AMS (re)entry.

-------------------------------------------------- End Proposed Text Changes------------------------------------------------