| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | CSGID transmission for snooping (16.4.8.1.2) |  |
| Date Submitted | 2026-07-28 |  |
| Source(s) | Jin Lee, Kiseon Ryu, Ronny Kim, Youngsoo Yuk, and JinSam Kwak LG Electronics | {jins978, ksryu, ronnykim, sixs, samji }@lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Comments on IEEE P802.16m/D4 for IEEE 802.16 WG Letter Ballot #31 |  |
| Abstract | The contribution proposes text changes IEEE P802.16m/D4 regarding CSGID transmission for snooping |  |
| Purpose | Discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

CSGID transmission for snooping (16.4.8.1.2)

Jin Lee, Kiseon Ryu, Ronny Kim, Youngsoo Yuk and JinSam Kwak

LG Electronics

- Introduction

The contribution proposes the text changes in D4 to describe how CSGID(s) sent by AMS can be used. Currently, CSGID(s) is sent for CSG membership detection while AMS performs network entry to Femto ABS. However, the CSGID(s) information can be also used for macro ABS to recommend CSG Femto ABS belonging to the CSGID(s) to detect its CSG AMS(s) by observing UL signaling.

Therefore, this contribution proposes CSGID(s) transmission during NE to macro ABS as well in case the macro ABS has a capability in order to direct CSG Femto ABS to monitor UL signal from accessible AMS.

- Rationale of modifications

- According to D4 [1], overlay macro ABS can help CSG Femto ABS to detect accessible (membership based) AMS as the procedure (called snooping) defined in 16.4.8.1.2. To do so, the overlay macro ABS needs to know which Femto ABSs can perform snooping for the AMS served by overlay macro ABS.

- Currently, AMS may send CSGIDs in AAI_NBR-REQ, which is used to retrieve neighbor lists or system information of the neighbors. However, if AMS enables to send CSGIDs during network entry to ‘snooping capable’ macro ABS, AMS can discover its CSG Femto ABS even before AAI_NBR-REQ is sent. (Currently, there is no way to initiate transmission of CSGIDs by ABS) Snooping capable macro ABS can be identified during basic capability negotiation process.

- Fig1. CSGID transmission to trigger snooping

- Purposed Texts

-------------------------------------------------- Begin Proposed Text Changes----------------------------------------------

Remedy 1 :

[Include the following texts at the end of the section 16.2.3.5, in line 54 page 71]

16.2.3.5 AAI_SBC-RSP

- Snooping Capability: Snooping capability shall only be set to 1 when snooping, which the ABS is capable of recommending CSG Femto ABS to monitor UL signal of accessible AMSs is supported.

Remedy 2 :

[Modify table 682 as suggested below, in line 50 page 75]

16.2.3.7 AAI_REG-REQ

Table 682 – Parameters for AAI_REG-REQ

| Name | Value | Usage |
| --- | --- | --- |
| …… | …… | …… |
| CSGID(s) | One or more identifiers for one or more CSG(s) | It should be included for macro ABS to recommend Femto ABS to monitor UL signal of the AMS |

Remedy 3 :

[Modify texts as suggested below, in line 33 page 701]

16.4.8.1.2 AMS scanning of neighbor Femto ABSs

When the AMS performs network entry with macro ABS, it may include CSGID(s) in AAI_REG-REQ if ‘snooping capability’ is set to 1 during capability negotiation. If the overlapped macro ABS receives the CSGID(s) from the AMS, Alternatively, it overlapped Macro ABS may recommend CSG Femto ABS to monitor UL signaling of accessible AMS which is served by the Macro ABS when a certain condition is met.

-------------------------------------------------- End Proposed Text Changes------------------------------------------------