| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text Related to FPEH for the IEEE 802.16m/D4 (16.2.2.2.1) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Eunjong Lee, Kiseon Ryu, Youngsoo Yuk, Ronny Yongho Kim LG Electronics | E-mail: eunjong.lee@lge.com, kiseon.ryu@lge.com, youngsoo.yuk@lge.com, ronnyyoungho.kim@lge.com |
| Re: | IEEE 802.16 Working Group Letter Ballot #30a |  |
| Abstract | The contribution proposes the text of MPDU formats to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m DRAFT amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text Related to FPEH for the IEEE 802.16m/D4 (16.2.2.2.1)

Eunjong Lee, Kiseon Ryu, Youngsoo Yuk and Ronny Yongho Kim

LG Electronics

- Introduction

This contribution proposes that the size of Length field in FPEH have 11 or 14 bits.

- Discussions

The MAC PDU length in 16m can be greater than 2047 bytes by MLEH (MAC PDU Length Extended Header). With this reason, the length field in MEH also has size of 11 or 14 bits and this is indicated by LI bit for each connection. However, the length field in current FPEH has only 11bit size.

Therefore, we propose to define the ‘LI’ field in FPEH to indicate whether the size of length in FPEH have 11 or 14 bits.

- References

[1] IEEE 802.16m-07/002r8, “802.16m System Requirements Document (SRD)”

[2] IEEE 802.16m-09/0034, “The Draft IEEE 802.16m System Description Document”

[3] IEEE P802.16 Rev2 / D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” January. 2009.

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

[5] IEEE 802.16m-09/0010r2, “IEEE 802.16m Amendment Working Document”

[6] IEEE P802.16m/D4, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Text proposal for the 802.16m DRAFT amendment

------------------------------------------------------Start of the Text---------------------------------------------------------

[Modify the text at line 567 on page 49 as follows:]

16.2.2.2.1 Fragmentation and packing extended header (FPEH)

Table 664 - FPEH Format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| FPEH (){ |  |  |
| Type | 4 | FPEH Type |
| FC | 2 | Fragmentation Control bits (see Table 664) |
| If (MAC Header == AGMH) { |  |  |
| SN | 10 | SN is maintained per connection. For non ARQ connection, 'SN' represents the MAC PDU Payload Sequence Number and the 'SN' value increments by one (modulo 1024) for each MAC PDU. For ARQ connection, 'SN' represents the ARQ block sequence number. |
| AFP | 1 | ARQ feedback poll indicator 0 = No ARQ feedback poll 1 = ARQ feedback poll for the connection indicated in AGMH or MEH if multiplexing is used |
| RI | 1 | ARQ Re-arrangement information indicator 0= no ARQ re-arrangement 1= ARQ re-arrangement |
| If (RI == 1) { |  |  |
| LSI | 1 | Last ARQ sub-block indicator 0 = indicating the last ARQ sub-block from the single ARQ block is not included in this MAC PDU 1 = indicating the last ARQ sub-block from the single ARQ block is included in this MAC PDU |
| SSN | 8 | SUB-SN of the first ARQ sub-block |
| } |  |  |
| } |  |  |
| Do { |  |  |
| End | 1 | Indication of more information 0 = Indicating another 'Length' and 'End' fields are followed 1 = Indicating no more 'Length' and 'End' fields are followed |
| LI | 1 | Length Indicator LI = 0 indicates that size of Length field is 11 bits LI = 1 indicates that size of Length field is 14 bits |
| If (End == 0) { |  |  |
| If (LI == 1){ |  |  |
| Length | 14 | This field indicates the length of SDU or SDU fragment in a MAC PDU payload. If a MAC PDU payload consists of 'N' SDU/SDU fragments, N-1 'Length' fields are present to represent the length of SDU/SDU fragment. No multiplexing: Length of last SDU/SDU fragment = Length of MAC PDU (given by length field in GMH/CMH & MLEH) - Length of GMH/CMH – Extended header group length (if present) - Length of PN& EKS (3bytes, if present) - Length of ICV ( if present) – sum of length of SDU/SDU fragment s #1 to #N-1 With multiplexing: Length of last SDU/SDU fragment = Length of connection payload corresponding to this FPEH (see table 666, MEH format) – sum of length of SDU/SDU fragments #1 to #N-1 |
| }else{ |  |  |
| Length | 11 | This field indicates the length of SDU or SDU fragment in a MAC PDU payload. If a MAC PDU payload consists of 'N' SDU/SDU fragments, N-1 'Length' fields are present to represent the length of SDU/SDU fragment. No multiplexing: Length of last SDU/SDU fragment = Length of MAC PDU (given by length field in GMH/CMH & MLEH) - Length of GMH/CMH – Extended header group length (if present) - Length of PN& EKS (3bytes, if present) - Length of ICV ( if present) – sum of length of SDU/SDU fragment s #1 to #N-1 With multiplexing: Length of last SDU/SDU fragment = Length of connection payload corresponding to this FPEH (see table 666, MEH format) – sum of length of SDU/SDU fragments #1 to #N-1 |
| } |  |  |
| } |  |  |
| } while (!End) |  |  |
| Padding | Variable | For byte alignment |
| } |  |  |

-----------------------------------------------------------End of the Text---------------------------------------------------------