| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | UL Feedback channel allocation for unpaired DL carrier in MC operation (16.2.8.2.8) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Eunjong Lee, Youngsoo Yuk, Kiseon Ryu, Ronny Yongho Kim LG Electronics | E-mail: eunjong.lee@lge.com, kiseon.ryu@lge.com, youngsoo.yuk@lge.com, ronnyyoungho.kim@lge.com |
| Re: | IEEE 802.16 Working Group Letter Ballot #30a |  |
| Abstract | The contribution proposes the text of multi-carrier operation to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m DRAFT amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

UL Feedback channel allocation for unpaired DL carrier 
in MC operation (16.2.8.2.8)

Eunjong Lee, Youngsoo Yuk, Kiseon Ryu and Ronny Yongho Kim

LG Electronics

- Introduction

In this contribution, we would like to discuss how to allocate the UL feedback channel for unpaired DL carrier, i.e., secondary carrier that DL is activated but UL is not activated.

- Discussions

In the current D4, the ABS can activate only the DL of a secondary carrier through the ‘Activation of DL/UL’ or ‘Deactivation of DL/UL’ field in the AAI_CM-CMD message. The meaning for these two fields is as follows.

- Activation of DL/UL (1 bit): if this field is set to ‘0’, both DL/UL are activated. Else, DL is activated but UL is not activated.

- Deactivation of DL/UL (1 bit): if this field is set to ‘0’, both DL/UL are deactivated. Else, UL is deactivated but DL is kept active.

The DL data transmission may require UL feedback information, such as channel quality feedback, MIMO feedback or HARQ ACK/NACK feedback. In order to efficiently transmit such UL feedback information, 16m has defined the several UL feedback channels (16.3.9.3), e.g., FFBCH and HFBCH. The ABS allocates or deallocates the UL feedback channel through one of Assignment A-MAP IEs defined in Section 16.3.6.5.2.4. However, UL feedback channel allocation of the current 16m has focused on a single carrier supported AMS and there don’t have any details regarding MC operation. In fact, if a carrier is both DL/UL activated carrier, a MC supporting ABS or AMS follows the same mechanism defined in 16.3.9.3 and there are no problem. However, since current MC operation in 16m supports a secondary carrier that DL is activated but UL is not activated, we need to define how to allocate the UL feedback channel for unpaired DL carrier.

Therefore, we propose that the ABS allocate the UL feedback channel for an AMS in multicarrier aggregation mode as follows (Note: the unpaired DL carrier is used only for an AMS in MC aggregation mode) and some examples for our proposal are illustrated in Appendix.

- If both DL and UL of a carrier have been activated, an AMS reports the feedback through Feedback Channel at the UL region of the corresponding carrier, as defined in Section 16.3.6.5.2.4.

- However, if a secondary carrier is one of unpaired DL carrier(s), an ABS should inform AMS of ‘Feedback Carrier Index’ for the carrier through the AAI_CM-CMD message when directing secondary carrier activation/deactivation. The AMS receiving the AAI_CM-CMD message with Feedback Carrier Index can report feedback through Feedback Channel at UL region of the carrier which is indicated by Feedback Carrier Index, when the ABS allocates an UL Feedback Channel to the AMS through the corresponding DL carrier.

- The mechanism for the feedback channel allocation of an ABS or UL feedback channel usage of an AMS is the same as defined in section 16.3.6.5.2.4 or 16.3.9.3. For an AMS in MC aggregation mode, an ABS may allocate one or more UL feedback channel(s) over a carrier.

Benefits from our proposed mechanism are as follows.

- No change in physical operation, such as A-MAP IE or UL feedback Channel design.

- Minimum signaling overhead (Feedback Carrier Index is only transmitted when necessary)

- Load balancing and flexible scheduling for the UL feedback channel of unpaired DL carrier(s)

- References

[1] IEEE 802.16m-07/002r8, “802.16m System Requirements Document (SRD)”

[2] IEEE 802.16m-09/0034, “The Draft IEEE 802.16m System Description Document”

[3] IEEE P802.16 Rev2 / D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” January. 2009.

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

[5] IEEE 802.16m-09/0010r2, “IEEE 802.16m Amendment Working Document”

[6] IEEE P802.16m/D4, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Text proposal for the 802.16m DRAFT amendment

-----------------------------------------------------------------------Start of the Text------------------------------------------------------------

[Modify and delete the texts at line 55 on page 234 as follows:]

16.2.8.2.8 DL CINR report operation

An ABS may assign FastFeedback channels to each carrier of an AMS. When FastFeedback channel is assigned, the AMS reports CINR for a carrier over the assigned FastFeedback channel of the corresponding carrier. ABS may also direct AMS to report CINRs of active carriers through FastFeedback channel(s) on the primary carrier at the feedback region. When measurement/report MAC control messages are used for DL CINR report operation, the messages are transmitted on the AMS's primary carrier. The measurement/report MAC control message may contain CINR reports(s) for all carriers or for each one or more carrier(s) of the AMS.

[Insert the section at line xx on page xxx as follows:]

16.2.8.2.x UL Feedback channel allocation

An ABS may assign UL Feedback channels to each active carrier of an AMS as defined in section 16.3.6.5.2.4.

When UL Feedback channel is assigned, the ABS allocates the Feedback channel through the corresponding DL carrier and the AMS reports feedback for a carrier over the UL Feedback channel of the corresponding carrier. However, in case of unpaired DL carrier, the ABS may direct AMS to report feedback for the corresponding DL carrier through Feedback Channel at UL region of the carrier which is indicated by the Feedback Carrier Index of the AAI_CM-CMD message. The carrier indicated by Feedback Carrier Index shall be one of both DL/UL activated carrier(s) and it may be primary carrier. If a carrier indicated by Feedback Carrier Index is deactivated, the ABS may update the Feedback Carrier Index for unpaired DL carrier through the AAI_CM-CMD message when deactivating the secondary carrier.

For an AMS in MC aggregation mode, the ABS may allocate one or more UL feedback channels over a carrier.

[Insert the texts at line 58 on page 197 and line 22 on page 198 as follows:]

16.2.8.2.11. 4.1 Carrier Management Command (AAI_CM-CMD) MAC control message

Table 749—AAI_CM-CMD MAC Control Message Format

| Field | Size (bit) | Description |
| --- | --- | --- |
| AAI_Carrier Management Command message format(){ |  |  |
| Control Message Type | 8 | AAI_CM-CMD |
| Action code | 1 | 0: Secondary carrier management 1: Primary carrier change |
| If (Action code == 0){ |  | This message is for secondary carrier activation and/or deactivation |
| Indication Type | 2 | For Activation and/or Deactivation Bit #0: ‘0’: No action, ‘1’: Activation Bit #1: ‘0’: No action, ‘1’: Deactivation |
| If (Indication Type #0 == 1) |  |  |
| Num of target carrier | 3 | The number of newly activated carrier(s) |
| For(i=0; i< Num of target carrier; i++){ |  |  |
| Target carrier index | 3 | Target carrier index for activation |
| Activation of DL/UL | 1 | ‘0’: Both DL/UL are activated ‘1’: DL is activated but UL is not activated |
| If (Activation of DL/UL == 1){ |  |  |
| Feedback Carrier Index | 3 | Logical carrier index for UL feedback channel This shall be one of AMS’s active carrier(s). It may be primary carrier of an AMS. |
| } |  |  |
| Ranging indicator | 1 | Ranging indicator for target carrier ‘0’: No ranging is required for the target carrier ‘1’: Periodic ranging is required for the target carrier |
| } |  |  |
| } |  |  |
| If (Indication Type #1 == 1){ |  |  |
| Num of target carrier | 3 | The number of deactivated carrier(s) |
| For(i=0; i< Num of target carrier; i++){ |  |  |
| Target carrier index | 3 | Target carrier index for deactivation |
| Deactivation of DL/UL | 1 | ‘0’: Both DL/UL are deactivated ‘1’: UL is deactivated but DL is kept active |
| Feedback Carrier FLAG | 1 | Feedback Carrier Index presence indicator, ‘0’: not present ‘1’: present. If the Deactivation of DL/UL is set to ‘1’, this value shall be always set to ‘1’. If the target carrier is both DL/UL activated carrier and has been linked to any unpaired DL carrier, Feedback Carrier for the linked DL carrier is updated to the following Feedback Carrier Index. In this case, this flag shall be set to ‘1’. |
| If (Feedback Carrier FLAG == 1){ |  |  |
| Feedback Carrier Index | 3 | Logical carrier index for UL feedback channel This shall be one of AMS’s active carrier(s). It may be primary carrier of an AMS. |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| If (Action code == 1){ |  | This message is for primary carrier change. |
| Physical carrier index of Target carrier | 6 | Physical carrier index of target carrier for primary carrier change If the AMS supports multicarrier operation, the carrier shall be one of the assigned carriers. |
| Action Time | 3 | LSB bits of Superframe number at the time to switch to the target carrier. This value shall be set to the value more than the retransmission timer for AAI_CM-CMD message |
| Next state of serving primary carrier | 1 | ‘0’: Serving carrier will be deactivated after primary carrier change. If the AMS does not support carrier aggregation, this field shall be always set to ‘0’. ‘1’: Serving carrier is kept active after primary carrier change |
| Ranging indicator | 1 | Ranging indicator for target carrier ‘0’: No ranging is required for the target carrier ‘1’: Periodic ranging is required for the target carrier |
| } |  |  |
| } |  |  |

--------------------------------------------------------------------End of the Text-------------------------------------------------------------------------

- Appendix

In this section, we illustrate some examples for the feedback channel allocation of an ABS and the feedback channel usage of an AMS in the following cases.

- Case 1. Both DL/UL of an inactive secondary carrier are activated.

- Case 2. DL of an inactive secondary carrier is activated but UL is not activated.

- Case 3. UL of an active secondary carrier is deactivated but DL is not deactivated.

- Case 4. Both DL/UL of an active secondary carrier which is linked with unpaired DL carrier are deactivated. In this case, an ABS has to update Feedback Channel Index for unpaired DL carrier.

Figure 1. An example for case 1

Figure 2. An example for case 2

Figure 3. An example for case 3

Figure 4. An example for case 4