| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | UL Feedback channel allocation for unpaired DL carrier in MC operation (16.2.8.2.8) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Eunjong Lee, Youngsoo Yuk, Kiseon Ryu, Ronny Yongho Kim LG Electronics | E-mail: eunjong.lee@lge.com, kiseon.ryu@lge.com, youngsoo.yuk@lge.com, ronnyyoungho.kim@lge.com |
| Re: | IEEE 802.16 Working Group Letter Ballot #30a |  |
| Abstract | The contribution proposes the text of multi-carrier operation to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m DRAFT amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

UL Feedback channel allocation for unpaired DL carrier 
in MC operation (16.2.8.2.8)

Eunjong Lee, Youngsoo Yuk, Kiseon Ryu and Ronny Yongho Kim

LG Electronics

- Introduction

In this contribution, we would like to discuss how to allocate the UL feedback channel for unpaired DL carrier, i.e., secondary carrier that DL is activated but UL is not activated.

- Discussions

In the current D4, the ABS can activate only the DL of a secondary carrier through the ‘Activation of DL/UL’ or ‘Deactivation of DL/UL’ field in the AAI_CM-CMD message. The meaning for these two fields is as follows.

- Activation of DL/UL (1 bit): if this field is set to ‘0’, both DL/UL are activated. Else, DL is activated but UL is not activated.

- Deactivation of DL/UL (1 bit): if this field is set to ‘0’, both DL/UL are deactivated. Else, UL is deactivated but DL is kept active.

The DL data transmission may require UL feedback information, such as channel quality feedback, MIMO feedback or HARQ ACK/NACK feedback. In order to efficiently transmit such UL feedback information, 16m has defined the several UL feedback channels (16.3.9.3), e.g., FFBCH and HFBCH. The ABS allocates or deallocates the UL feedback channel through one of Assignment A-MAP IEs defined in Section 16.3.6.5.2.4. However, UL feedback channel allocation of the current 16m has focused on a single carrier supported AMS and there don’t have any details regarding MC operation. In fact, if a carrier is both DL/UL activated carrier, a MC supporting ABS or AMS follows the same mechanism defined in 16.3.9.3 and there are no problem. However, since current MC operation in 16m supports a secondary carrier that DL is activated but UL is not activated, we need to define how to allocate the UL feedback channel for unpaired DL carrier.

Therefore, we propose that the ABS allocate the UL feedback channel for an AMS in multicarrier aggregation mode as follows (Note: the unpaired DL carrier is used only for an AMS in MC aggregation mode) and an example for our proposal is illustrated in Appendix.

- If both DL and UL of a carrier have been activated, an AMS reports the feedback through Feedback Channel at the UL region of the corresponding carrier, as defined in Section 16.3.6.5.2.4.

- However, when the ABS assigns the feedback channel for an asymmetric DL carrier of fully configured carrier(s), the UL feedback channel shall be allocated at the same UL region used for the primary carrier.

- The mechanism for the feedback channel allocation of an ABS or UL feedback channel usage of an AMS is the same as defined in section 16.3.6.5.2.4 or 16.3.9.3. For an AMS in MC aggregation mode, an ABS may allocate one or more UL feedback channel(s) over a carrier.

Benefits from our proposed mechanism are as follows.

- No change in physical operation, such as A-MAP IE or UL feedback Channel design.

- References

[1] IEEE 802.16m-07/002r8, “802.16m System Requirements Document (SRD)”

[2] IEEE 802.16m-09/0034, “The Draft IEEE 802.16m System Description Document”

[3] IEEE P802.16 Rev2 / D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” January. 2009.

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

[5] IEEE 802.16m-09/0010r2, “IEEE 802.16m Amendment Working Document”

[6] IEEE P802.16m/D4, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Text proposal for the 802.16m DRAFT amendment

-----------------------------------------------------------------------Start of the Text------------------------------------------------------------

[Modify and delete the texts at line 55 on page 234 as follows:]

16.2.8.2.8 DL CINR report operation

An ABS may assign FastFeedback channels to each carrier of an AMS. When FastFeedback channel is assigned, the AMS reports CINR for a carrier over the assigned FastFeedback channel of the corresponding carrier. ABS may also direct AMS to report CINRs of active carriers through FastFeedback channel(s) on the primary carrier at the feedback region. When measurement/report MAC control messages are used for DL CINR report operation, the messages are transmitted on the AMS's primary carrier. The measurement/report MAC control message may contain CINR reports(s) for all carriers or for each one or more carrier(s) of the AMS.

[Insert the section at line xx on page xxx as follows:]

16.2.8.2.x UL feedback channel allocation

An ABS may assign UL feedback channels to each active carrier of an AMS as defined in section 16.3.6.5.2.4.

When UL feedback channel is assigned, the ABS allocates the feedback channel through the corresponding DL carrier and the AMS reports fast feedback and HARQ feedback for the carrier over the UL feedback channel of the corresponding carrier. When only DL of fully configured carrier has been activated, UL feedback channel assigned by the ABS is located at the UL region defined in the SFH on the primary carrier.

For an AMS in MC aggregation mode, the ABS may allocate multiple UL feedback channels over a primary carrier.

--------------------------------------------------------------------End of the Text-------------------------------------------------------------------------

- Appendix

In this section, we illustrate an example for the feedback channel allocation of an ABS and the feedback channel usage of an AMS in case of the unpaired DL carrier.

Figure 2. An example for case 2