| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Modifications to AAI_SCN-REQ Message (16.2.3) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Jaesun Cha, Soojung Jung, Eunkyung Kim, Sungcheol Chang, Hyun Lee, Chulsik Yoon ETRI | jscha@etri.re.kr, *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot #31 on P802.16m/D4 |  |
| Abstract | This contribution proposes to modify some parameters in the AAI_SCN-REQ message. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Modifications to AAI_SCN-REQ Message (16.2.3)

Jaesun Cha, Soojung Jung, Eunkyung Kim, Sungcheol Chang, Hyun Lee, Chulsik Yoon

ETRI

# Introduction

For multicarrier scanning operation, carrier information shall be negotiated through AAI_SCN-REQ/RSP messages. But, the current definition is somewhat ambiguous.

An AMS may include three types of carrier information in the AAI_SCN-REQ message as follows;

- Carriers at neighboring ABS: these are the carriers at neighbor ABS to be scanned and they may be included in the AAI_SCN-REQ message if the neighbor ABS is a multicarrier ABS.

- Carriers at serving ABS: these are the inactive carriers at the serving ABS to be scanned and they may be included in the AAI_SCN-REQ message if the serving ABS is a multicarrier ABS.

- Carrier for scanning: if an AMS is a multicarrier AMS and it is going to perform scanning using one of active carriers, then the carrier index of that carrier shall be included in the AAI_SCN-REQ message. If the AMS has additional inactive transceiver, then it will not transmit the AAI_SCN-REQ message because it can perform autonomous scanning using the inactive transceiver. Therefore, if the multicarrier AMS transmits the AAI_SCN-REQ message, then information on the carrier used for scanning shall be included in the AAI_SCN-REQ message.

# Text Proposal

---------------------------------------------------------Start of the Text----------------------------------------------------------

[Modify Table 688 as follows]

Table 688 – Parameters for AAI_SCN-REQ message

| Name | Value | Usage |
| --- | --- | --- |
| Scan duration | Duration (in units of AAI subframes) of the requested scanning period. |  |
| Scan Purpose | 0b0: scan BS(s) which is in the list of the AAI_NBR-ADV message 0b1: scan BS(s) which is not in the list of the AAI_NBR-ADV message |  |
| Interleaving Interval | The period of AMS’s normal operation which is interleaved between Scan Durations. |  |
| Scan Iteration | The requested number of iterating scanning interval by an AMS. |  |
| Recommended Start Frame indication | Indicates whether Recommended start super frame number field and Recommended start frame field are included |  |
| N_Recommended_BS_Index_indication | Indicates whether N_Recommended_BS_Index field is included in this message |  |
| N_Recommended_BS_Full_indication | Indicates whether N_Recommended_BS_Full field is included in this message. |  |
| N_Recommended_Preamble_ Index_indication | Indicates whether N_Recommended_Preamble_Index field is included in this message |  |
| Recommended start super frame number | Represents the 6 least significant bits of the absolute super frame number for which the AMS recommends the first Scanning Interval to start. This field is set to 0 if an AMS has no preferred value. | Shall be included when Recommended Start Frame indication is set to 1 |
| Recommended start frame number | Represents recommended start frame number within the super frame number which is indicated by Recommended start super frame number field. The value indicates: 0b00: the 1st frame in the super frame 0b01: the 2nd frame in the super frame 0b10: the 3rd frame in the super frame 0b11: the 4th frame in the super frame | Shall be included when Recommended Start Frame indication is set to 1 |
| N_Recommended_BS_Index | This is the number of neighboring BS to be scanned, which are included in AAI_NBR-ADV message. When an AMS receives AAI_SCN-RSP message from BS in response to AAI_SCN-REQ message, the AMS shall check whether Configuration Change Count stored by the AMS is the same as one included in AAI_SCN-RSP message sent by the ABS. If an AMS detects mismatch of Configuration Change Counts, it may retransmit AAI_SCN-REQ message to the ABS. | Shall be included when N_Recommended_BS_Index_ indication is set to 1 |
| Configuration Change Count for AAI_NBR-ADV | The value of Configuration Change Count in AAI_NBR-ADV message referred in order to compress neighbor BSID |  |
| Neighbor_BS_Index | Based on the value of N_Recommended_BS_Index, BS index corresponds to position of BS in AAI_NBR-ADV message included. | Shall be included when N_Recommended_BS_Index > 0 |
| N_Recommended_BS_Full | Number of neighboring BS to be scanned, which are using full 48-bit BSID. | Shall be included when N_Recommended_BS_Full_ indication is set to 1 |
| Recommended BS ID | Based on the value of N_Recommended_BS_Full, identifiers of the BSs the MS plans to scan is included. | Shall be included when N_Recommended_BS_Full > 0 |
| N_Recommended_Preamble_Index | Number of preambles to be scanned | Shall be included when N_Recommended_Preamble_ Index_Indication is set to 1 |
| Preamble Index | Based on the value of N_Recommended_BS_Full, preamble indexidentifiers of the BSs the MS plans to scan is included. | Shall be included when N_Recommended_Preamble_ Index > 0 |
| N_Recommended_Carrier_Index | Number of carriers to be scanned at each neighboring ABS. | May be included when a neighboring ABS is a multi-carrier ABS and N_Recommended_BS_Index > 0 or N_Recommended_BS_Full > 0 |
| Recommended_Carrier_Index | Recommended physical carrier index for scan at each neighboring ABS. | Shall be included when N_ Recommended_Carrier_Index > 0 |
| N_ Recommended_Carrier_Index_at_ Serving_ABS | Number of physical carrier index of the serving ABS to be scanned | May be included when the serving ABS is a multi-carrier ABS |
| Recommended Carrier index at serving ABS | Recommended physical carrier index of the serving ABS to be scannedused for scanning | Shall be included when N_ Recommended_Carrier_Index _at_Serving_ABS > 0 |
| Scanning_Carrier_Index | Recommended logical carrier index of the AMS to be used for scanning. | Shall be included when the AMS supports multi-carrier scanning |
| CSGID(s) | One or more identifiers for one or more CSG(s) | Sent by AMS to aid ABS reselection |

---------------------------------------------------------End of the Text----------------------------------------------------------