| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Modifications to AAI_SCN-RSP Message (16.2.3) |  |
| Date Submitted | 2010-03-17 |  |
| Source(s) | Jaesun Cha, Soojung Jung, Eunkyung Kim, Sungcheol Chang, Hyun Lee, Chulsik Yoon ETRI | jscha@etri.re.kr, *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot #31 on P802.16m/D4 |  |
| Abstract | This contribution proposes to modify some parameters in the AAI_SCN-RSP message. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Modifications to AAI_SCN-RSP Message (16.2.3)

Jaesun Cha, Soojung Jung, Eunkyung Kim, Sungcheol Chang, Hyun Lee, Chulsik Yoon

ETRI

# Introduction

For multicarrier scanning operation, carrier information shall be negotiated through AAI_SCN-REQ/RSP messages. But, the current definition is somewhat ambiguous.

An ABS may include three types of carrier information in the AAI_SCN-RSP message as follows;

- Carriers at neighboring ABS: these are the carriers at neighbor ABS to be scanned and they may be included in the AAI_SCN-RSP message if the neighbor ABS is a multicarrier ABS.

- Carriers at serving ABS: these are the inactive carriers at the serving ABS to be scanned and they may be included in the AAI_SCN-RSP message if the serving ABS is a multicarrier ABS.

- Carrier for scanning: if an AMS is a multicarrier AMS and it is going to perform scanning using one of active carriers, then the carrier index of the carrier shall be included in the AAI_SCN-RSP message. If the AMS has additional inactive transceiver, then it will perform autonomous scanning using the inactive transceiver. Therefore, if the ABS transmits the AAI_SCN-RSP message to the multicarrier AMS, then information on the carrier used for scanning shall be included in the AAI_SCN-RSP message.

# Text Proposal

---------------------------------------------------------Start of the Text----------------------------------------------------------

[Modify Section 16.2.3.14 as follows]

16.2.3.14 AAI_SCN-RSP

An AAI_SCN-RSP message shall be transmitted by the ABS either unsolicited or in response to an AAI_SCN-REQ message sent by an AMS. An ABS may transmit AAI_SCN-RSP to start AMS scan reporting with or without scanning interval allocation. If Scan Duration field contains a non-zero value, scanning interval pattern included in the AAI_SCN-RSP message replaces previous existing scanning interval pattern. When an AAI_SCN-RSP is sent in response to an AAI_SCN-REQ message, setting Scan Duration to zero indicates the request for an allocation of scanning interval is denied.

Table 689 – Parameters for AAI_SCN-RSP message

| Name | Value | Usage |
| --- | --- | --- |
| Scan duration | Duration (in units of AAI subframes) of the requested scanning periodwhere the AMS may perform scanning. |  |
| Scan Purpose | 0b0: scan BS(s) which is in the list of the AAI_NBR-ADV message 0b1: scan BS(s) which is not in the list of the AAI_NBR-ADV message |  |
| Report mode | 0b00: No report 0b01: Periodic report 0b10: Event-triggered report 0b11: One-time scan report |  |
| Report metric | Bitmap indicator of trigger metrics that the serving ABS requests the AMS to report. The serving ABS shall indicate only the trigger metrics agreed during AAI_SBC-REQ/RSP negotiation. Each bit indicates whether reports will be initiated by trigger based on the corresponding metrics: Bit 0: BS CINR mean Bit 1: BS RSSI mean Bit 2: Relative delay Bit 3: BS RTD Bits 4-5: Reserved; shall be set to zero |  |
| Report period | The period of MS’s report of scanning result when the MS is required to report the value periodically or one-time. The period is calculated from the start of the first scan duration. If the BS sends an unsolicited AAI_SCN-RSP message without assignment of a scanning interval and the scan duration is set to zero, the period is calculated from the frame the MS receives the AAI_SCN-RSP message. For MS scanning request denied by AAI_SCN-RSP with Scan Duration set to zero, Request period is the number of frames that BS suggests to MS before transmitting next AAI_SCN-REQ |  |
| N_Recommended_BS_Index_indication | Indicates whether N_Recommended_BS_Index field is included in this message. This value is valid only if Scan duration is not equal to zero. |  |
| N_Recommended_BS_Full_indication | Indicates whether N_Recommended_BS_Full field is included in this message. This value is valid only if Scan duration is not equal to zero. |  |
| Start super frame number | Represents the 6 least significant bits of the absolute super frame number in which the first Scanning Interval starts | Included only when Scan Duration > 0 |
| Start frame number | Represents recommended start frame number in the Start super frame number. 0b00: the 1st frame in the super frame 0b01: the 2nd frame in the super frame 0b10: the 3rd frame in the super frame 0b11: the 4th frame in the super frame | Included only when Scan Duration > 0 |
| Interleaving Interval | The period of AMS’s normal operation which is interleaved between Scan Durations. | Included only when Scan Duration > 0 |
| Scan Iteration | The number of iterating scanning interval. | Included only when Scan Duration > 0 |
| N_Recommended_BS_Index | The number of neighboring BS to be scanned, which are included in AAI_NBR-ADV message. | Included only when Scan Duration > 0 and N_Recommended_BS_Index _indication > 0 |
| Configuration Change Count for AAI_NBR-ADV | The value of Configuration Change Count in AAI_NBR-ADV message referred in order to compress neighbor BSID | Included when N_Recommended_BS_Index > 0 |
| N_Recommended_BS_Full | Number of neighboring BS to be scanned, which are using full 48-bit BS ID. | Included only when Scan Duration > 0 |
| Recommended BS ID | Based on the value of N_Recommended_BS_Full, Recommended BS ID list for scan is included | Included when N_Recommended_BS_Full > 0 and N_Recommended_BS_Full _indication > 0 |
| Preamble Index of Recommended BS ID | Preamble Index of Recommended BS ID | Included when N_Recommended_BS_Full > 0 |
| N_Recommended_Carrier_Index | Number of physical carriers to be scanned at each neighboring ABS. | May be included for a multi-carrier target ABS |
| Recommended_Carrier_Index | Recommended physical carrier index for scan at each neighboring ABS. | Included when N_ Recommended_Carrier_Index > 0 |
| N_ Recommended_Carrier_Index_at_ Serving_ABS | Number of physical carrier index of the serving ABS to be scanned | May be included when the serving ABS is a multi-carrier ABS |
| Recommended Carrier index at serving ABS | Recommended physical carrier index of the serving ABS to be scannedused for scanning | May be included when the serving ABS is a multi-carrier ABS Included when N_ Recommended_Carrier_Index _at_Serving_ABS > 0 |
| Scanning_Carrier_Index | Recommended logical carrier index of the AMS to be used for scanning. | Optional. May be included when the AMS supports multi-carrier scanning |

---------------------------------------------------------End of the Text----------------------------------------------------------