| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Global carrier configuration acquisition procedure (sections 16.2.15.6) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Inuk Jung, Youngsoo Yuk, Kiseon Ryu, Ronny Yongho Kim, Jin Sam Kwak LG Electronics, Inc. Daniel Cohn Alvarion | +82-31-450-7811 inuk.jung@lge.com daniel.cohn@alvarion.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for comments for Letter Ballot #31 / Topic: Multi-carrier |  |
| Abstract | Text proposal for Handover |  |
| Purpose | For member’s review and adoption into P802.16m_D4 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Global carrier configuration acquisition procedure (sections 16.2.15.6)

Inuk Jung, Youngsoo Yuk, Kiseon Ryu, Ronny Yongho Kim, Jin Sam Kwak

LG Electronics, Inc.

Daniel Cohn

Alvarion

- Introduction

This contribution includes proposed text to provide further detail on how to acquire global carrier configuration information.

- Rationale/Motivation of modifications

All the network wide (i.e. very static) system information of ABSs is defined based on the Global carrier Configuration Information (GCI), which is unicast to the AMS during initial network entry. However, GCI is very unlikely to change, hence it shall not be unicast to the AMS at each initial network entry but shall only send if AMS never acquired GCI before, or if its GCI information is out of date.

During initial network entry, an AMS with multi-carrier support shall indicate its GCI change count to the ABS via the AAI_REG-REQ message. An AMS that never received the GCI shall set its GCI change count to 0, which shall not be used as a valid change count value for GCI. If ABS detects that AMS has a GCI out-of-date or if the AMS sends a GCI change count value of 0, the ABS shall inform this mismatching status, and shall unicast the AAI_Global-Config message to the AMS. The indication of this mismatch state is sent within the AAI_REG-RSP message. The mismatch may be informed by using a GCI mismatch flag or by including the current GCI change count of the network in the AAI_REG-RSP message. When using the CGI mismatch flag, 0 indicates that AMS’s GCI is up-to-date, whether 1 indicates that the AMS’s GCI is out-of-date. When using the current GCI change count in AAI_REG-RSP message, by comparison, the AMS may know that its GCI is out-of-date or not. In case the AMS is informed of its GCI mismatch, the AMS should not perform any procedure that refers to a carrier until it receives the up-to-date AAI_Global-Config message as shown in Fig. 1.

Figure 1. GCI acquisition procedure

- References

[1] IEEE P802.16m/D4, “Air Interface for Broadband Wireless Access Systems - Advanced Air Interface”

- Proposed AWD Text Changes

Remedy #1

On the 80216m_D4, page 297, line 13, add proposed text as follows:

--------------------------------------------------------Text Start -----------------------------------------------------

16.2.15.6 Registration

…

In AAI_REG-REQ, the AMS informs the ABS of its capability parameters to be negotiated except those capabilities already negotiated with the ABS through AAI_SBC-REQ/RSP messages. In AAI_REG-RSP, the ABS responds with the accepted capability parameters. If the AMS omits some capability parameters in the AAI_REG-REQ, the ABS considers AMS follows the default values for those parameters and, if acceptable, ABS may omit those parameters applying default value in its AAI_REG-RSP

Within the AAI_REG-REQ message, the AMS shall indicate its latest received Global carrier configuration change count. Upon receiving this message, the ABS shall check if AMS’s Global carrier configuration information is valid. As a response, the ABS shall include the current Global carrier configuration change count in the AAI_REG-RSP message. If AMS’s Global carrier configuration change count is out of date, the ABS shall unicast the AAI_Global-Config message to the AMS. Also, if AMS’s Global carrier configuration information is expired, the AMS shall not perform any procedure that refers to the AAI_Global-Config message until the valid AAI_Global-Config message is sent by the ABS.

If the AAI_Global-Config message parameters are to be updated, the ABS shall unicast the updated AAI_Global-Config message to each AMS’s served by the ABS in unicast manner. The ABS shall set Polling bit to 1 in MCEH of the AAI_Global-Config message.

--------------------------------------------------------Text End -----------------------------------------------------

Remedy #2

On the 80216m_D4, page 72, line 46 Table 682, modify table as follows:

--------------------------------------------------------Text Start -----------------------------------------------------

16.2.3.7 AAI_REG-REQ

An AAI_REG-REQ message is transmitted by AMS to negotiate general AMS capabilities and do registration during network entry.

…

Table 682— Parameters for AAI_ REG-REQ

| Name |  | Value |
| --- | --- | --- |
| … |  | … |
| Host configuration capabilities and parameters | Host-Configuration-Capability-Indicator IE | Indicates whether the AMS supports the capability of configuring host using the received parameters through the AAI_REG-RSP message. (One bit indicator) This shall be omitted if Requested-Host-Configurations IE is included in the message. |
|  | Requested-Host-Configurations IE | Includes requested host configuration options in DHCP Options format. If included, this IE indicates that the AMS supports host configuration using AAI_REG-RSP message, and Host-Configuration-Capability-Indicator IE shall be omitted. |
| Global carrier configuration change count [3 bit] |  | Indicates the AMS’s last received value of Global carrier configuration change count of the network. If set to 0, it implies that AMS never received a Global carrier configuration information. |
| … |  | … |

--------------------------------------------------------Text End -----------------------------------------------------

Remedy #3

On the 80216m_D4, page 76, line 42 Table 683, modify table as follows:

--------------------------------------------------------Text Start -----------------------------------------------------

16.2.3.8 AAI_REG-RSP

An AAI_REG-RSP message is transmitted by ABS in response to AAI_ REG-REQ message during initialization.

…

Table 683— Parameters for AAI_ REG-RSP

| Name |  | Value |
| --- | --- | --- |
| … |  | … |
| Host configuration capabilities and its parameters | IPv4-Host-Address IE | The allocated IPv4 Host Address for the AMS. Size: 4 octet |
|  | IPv6-Home-Network-Prefix IE | The allocated IPv6 Home Network Prefix for the AMS. Size: 8 octet |
|  | Additional-Host-Configurations IE | Includes additional host configuration options in DHCP Options format as detailed in [3], [4] |
| Redirection Info | ABSID, preamble index and center frequency for one or more neighbor ABS | Sent by serving ABS to aid cell reselection in the event the serving ABS is not able to allow the AMS to perform entry. |
| Global carrier configuration change count [3 bit] |  | Indicates the current value of Global carrier configuration change count of the network |

--------------------------------------------------------Text End -----------------------------------------------------

Remedy #4

On the 80216m_D4, page 146, line 1 Table 735, modify table as follows:

--------------------------------------------------------Text Start -----------------------------------------------------

16.2.3.53 AAI_Global-Config (global carrier configuration) Message

The AAI_Global-Config message provides the carrier information for all available carriers in the network. The ABS transmits the AAI_Global-Config message to an AMS right after network entry completes after transmitting the REG-RSP message.

Table 735— AAI_Global-Config MAC Control Message Format

| Field | Size (bit) | Description |
| --- | --- | --- |
| MAC Control Message Type | 8 |  |
| Global carrier configuration change count | 3 | Indicates the value of Global carrier configuration change count of the network, will be incremented each time the contents of this message change. Value range is 1-7. |
| Number of Carrier Groups | 4 | Groups of contiguous carriers |
| … | … | … |

--------------------------------------------------------Text End -----------------------------------------------------