| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Correct Context Mapping for Zone Switch procedure (sections 15.2.3.2) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Inuk Jung, Kiseon Ryu, Ronny Yongho Kim, Jin Sam Kwak LG Electronics, Inc. | +82-31-450-7811 inuk.jung@lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for comments for Letter Ballot #31 / Topic: HO |  |
| Abstract | Text proposal for Handover |  |
| Purpose | For member’s review and adoption into P802.16m_D4 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Correct Context Mapping for Zone switch procedure in IEEE802.16m systems (section 16.2.3.2)

Inuk Jung, Kiseon Ryu, Ronny Yongho Kim, Jin Sam Kwak

LG Electronics, Inc.

- Introduction

This contribution includes proposed text to provide Security support Zone Switching procedure from L to MZone and vice versa in the 802.16m D4 document [1].

- Rationale/Motivation of modifications

1. Correct context mapping

The context mapping of service flow ID’s are defined to be done automatically mapped in ascending order from the first transport connection CID. The SAID in 16 and 16m are differently defined, hence, they cannot be automatically mapped.

Thus, the re-mapping SAID per service flow ID shall be explicitly informed to the AMS using the AAI_RNG-RSP message. This may be seen as an service flow update procedure using AAI_RNG-RSP, where security related service flow parameters are additionally updated.

Such SAID mapping shall be done for both LZone to MZone and MZone to LZone zone switching. In case of MZone to LZone switching, the number (or type) of SAID used in MZone are smaller than the LZone. Thus, the mapping shall be appropriately done by the ABS.

- References

[1] IEEE P802.16m/D4, “Air Interface for Broadband Wireless Access Systems - Advanced Air Interface”

- Proposed AWD Text Changes

Remedy #1

On the 80216m_D4, page 66, line 41, modify text and Table 677 as follows:

--------------------------------------------------------Text Start -----------------------------------------------------

16.2.3.2 AAI_RNG-RSP

An AAI_RNG-RSP, to which HARQ operation is applied, shall be transmitted by the ABS in response to a received AAI_RNG-REQ. In addition, it may also be transmitted asynchronously to send corrections based on measurements that have been made on other received data or MAC messages. As a result, the AMS shall be prepared to receive an AAI_RNG-RSP at any time, not just following an AAI_ RNG-REQ transmission.

…

In case of Zone Switch procedure from LZone to MZone, the AAI_RNG-RSP message shall include the updated SAID’s for each service flow.

…

Table 677—parameters for AAI_ RNG-RSP

| Name | Value | Usage |
| --- | --- | --- |
| … | … | … |
| Neighbor station measurement report indicator | Perform Neighbor station measurement report if set to '1' | Identifies Neighbor station measurement report is required during current network entry |
| N_SF_update | 4 | Number of service flow that requires update |
| Service_flow_bitmap | 16 | Bitmap for indicating the specific service flows that are being updated. The MS will map them autonomously by referring to this bitmap |
| for(i=0; i< N_SF_update; i++) { | ─ | ─ |
| SAID | 2 | The updated SAID value |
| } | ─ |  |
| … | … | … |

--------------------------------------------------------Text End -----------------------------------------------------