| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Handling mismatching System Information during HO for IEEE 802.16m (section 16.2.6.3.3) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Inuk Jung, Kiseon Ryu, Ronny Yongho Kim, Jin Sam Kwak LG Electronics, Inc. | +82-31-450-7811 inuk.jung@lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for comments |  |
| Abstract | Text proposal for HO section 16.2.6.3.3 |  |
| Purpose | For member’s review and adoption into P802.16m_D4 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Handling mismatching System Information during HO for IEEE 802.16m (section 16.2.6.3.3)

Inuk Jung, Kiseon Ryu, Ronny Yongho Kim, Jin Sam Kwak

LG Electronics, Inc.

- Introduction

This contribution includes proposed text to provide some clarification of how to update mismatch SFH during HO in IEEE802.16m systems.

- Rationale/Motivation of modifications

The mismatch of system information between AMS and S-ABS may greatly increase the HO latency, hence the ABS shall coordinate HO appropriately.

Mismatch of neighbor SFH

AMS may fail to update its SFH during HO by either missing an updated AAI_NBR-ADV at serving ABS, or if SFH has changed between the broadcast interval of AAI_NBR-ADV. In such case, the AMS may not be able to perform network re-entry at Action Time but has to re-acquire the latest SFH.

Furthermore, the detection of such SFH mismatch occurs at either the serving ABS or the AMS, which have different impact on the HO procedure. Hence, for each case, the HO procedure shall take appropriate consideration and correctly coordinate the AMS.

- In case of AMS initiated HO, the AMS informs its current AAI_NBR-ADV change count. The ABS can compare AMS’s change count and the current change count for any SFH mismatch. If SFH mismatches, ABS may either coordinate the AMS to perform EBB HO to directly acquire SFH at target ABS, or reject HO for latter HO coordination.

        - In case of ABS initiated HO, the AMS can detect whether AA_NBR-ADV is mismatching or not by referring to the AAI_NBR-ADV change count in AAI_HO-CMD message. If AAI_NBR-ADV mismatches, the AMS shall indicate it by including its latest received AAI_NBR-ADV change count in AAI_HO-IND message. This is to cancel any coordinated seamless HO at the target ABS.

Figure 1. Case of AAI_NBR-ADV mismatch between AMS and S-ABS

- References

[1] IEEE P802.16m/D4, “Air Interface for Broadband Wireless Access Systems - Advanced Air Interface”

- Proposed AWD Text Changes

Remedy #1

On the 80216m_D4, page 207, line 44, replace current text in 16.2.6.3.3 with the proposed text as follows:

--------------------------------------------------------Text Start -----------------------------------------------------

16.2.6.3.3 HO Preparation

…

Information regarding AMS identity (e.g.STID) and security context should be pre-updated during HO preparation. Any mismatched system information between AMS and the target ABS, if detected, may be provided to the AMS by the Serving ABS during HO preparation.

For AMS initiated HO, the ABS may detect an S-SFH mismatch by referring to the AAI_NBR-ADV change count of AMS included in AAI_HO-REQ message. In such case, the ABS should include mismatching delta SFH information in AAI_HO-CMD, or it should cancel the HO. not initiate seamless handover at the target ABS. may either reject the HO procedure or coordinate AMS to perform EBB HO to update S-SFH directly at target ABS.

For ABS initiated HO, the AMS may detect an SFH mismatch by referring to the S-SFH change count included in the AAI_HO-CMD message. If S-SFH change count differs between AMS and the candidate ABS(s), AMS shall transmit AAI_HO-IND message to the serving ABS with HO event code set to 0b100 (i.e. indicating mismatch of SFH) including AMS’s last received AAI_NBR-ADV change count. Upon receiving such AAI_HO-IND message, the ABS shall should cancel any coordinated seamless the HO at the target ABS. If pre-allocated at target ABS, the serving ABS shall include an STID to be used at target ABS in the AAI_HO-CMD message. The pre-allocated STID shall be used in the target ABS by the AMS to communicate with the target ABS. The FIDs which are used to distinguish different connections are not updated during the handover procedure. Rejection of each service flow shall also be indicated in the AAI_HO-CMD message.

--------------------------------------------------------Text End -----------------------------------------------------

Remedy #2

On the 80216m_D4, page 80, line 10, add the proposed parameter to Table 685 as follows:

--------------------------------------------------------Text Start -----------------------------------------------------

16.2.3.10 AAI_HO-REQ

…

Table 685—Parameters for AAI_HO-REQ message

| Name | Value | Usage |
| --- | --- | --- |
| … | … | … |
| AAI_NBR-ADV Change count | AAI_NBR-ADV change count last received from the serving ABS |  |
| … | … | … |

--------------------------------------------------------Text End -----------------------------------------------------

Remedy #3

On the 80216m_D4, page 79, line 50, add the proposed parameter to Table 684 as follows:

--------------------------------------------------------Text Start -----------------------------------------------------

16.2.3.9 AAI_HO-IND

…

Table 684—Parameters for AAI_HO-IND message

| Name | Value | Usage |
| --- | --- | --- |
| HO Event Code | 0b000: Target ABS selection in case of multiple candidate T-ABSs. 0b001: All target ABSs in AAI_HO-CMD are unreachable. In this case, the AMS shall include a new target ABS that was not included in AAI_HO-CMD. 0b0010: AMS unable to stay connected to serving ABS until expiration of disconnect time 0b011: HO cancel 0b100: Indicate HO cancel due to mismatch of SFH | This is used to distinguish AAI_HO-IND among different scenarios |
| AAI_NBR-ADV Change count | AAI_NBR-ADV change count last received from the serving ABS | This is only included when a mismatch of AAI_NBR-ADV is detected. |
| … | … | … |

--------------------------------------------------------Text End -----------------------------------------------------