| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text on Uplink Power Status Reporting (16.3.9.4.7) for the IEEE 802.16m/D4 Amendment |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Kun Liu , Zhaohua Lu, Huiying Fang, Xiangyu Liu, Yanfeng Guan ZTE Corporation | Email: liu.kun12@zte.com.cn, fang.huiying@zte.com.cn， lu.zhaohua@zte.com.cn |
| Re: | Letter Ballot #31 on the Draft Amendment (IEEE P802.16m/D4) |  |
| Abstract | This contribution provides the proposed text for 16m Uplink Power Status Reporting. |  |
| Purpose | To be discussed and adopted into P802.16m/D4 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text on Uplink Power Status Reporting(16.3.9.4.7) for the IEEE 802.16m/D4 Amendment

Kun Liu, Zhaohua Lu, Xiangyu Liu, Huiying Fang, Yanfeng Guan

ZTE Corporation

- Introduction

This contribution proposes some modification in Uplink Power Status Reporting Section (16.3.9.4.7) of IEEE P802.16m/D4[1].

In Section 16.3.9.4.7.1, AMS shall report the base uplink transmission PSD to ABS, the base uplink transmission PSD is derived from formula (292).

PSD(base) = L + NI (292) ;

But in IEEE P802.16m/D4, AMS gets NI information through “S-SFH SP3 IE ” and “16.2.3.30 AAI_UL Noise and Interference Level Broadcast Message”. As above description, ABS already knows the NI information, it is redundant to add NI information into PSD(base), AMS only need to send L to ABS, then ,ABS can calculate the PSD(base) by itself.

- References

[1] IEEE P802.16m/D4, “Draft Amendment to IEEE Standard for Local and metropolitan area network: Air Interface for Fixed and Mobile Broadband Wireless Access Systems”, Feb. 2010.

- Text proposal

Instruction to editor;

Black text: existing D4 text

Red and strike out text: for deletion

Blue and underline text: for addition

[Remedy 1:]

[Modify text from line 3 on the page 120, under subsection 16.2.3.33, as follows:]

============================== Start of Proposed Text =================================

16.2.3.33 AAI_Uplink Power Status Report (AAI_UL_PSR) message

An AAI_UL_PSR message is used by the AMS to report uplink power status to ABS. It is defined as:

-- ASN1START

AAI_UL_PSR:: = SEQUENCE {

Tx_Power_PSD_Base		INTEGER (0..255)

Downlink Pathloss_L		INTEGER (0..255)

Tx_SIR_DownloadDownlink	INTEGER (0..1024)

}

-- ASN1STOP

Table 711—AAI_UL-PSR message Field Descriptions

| Parameters | Description |
| --- | --- |
| Tx_Power_PSD_Base: Downlink Pathloss_L | Tx_Power_PSD_Base (PSD(base))Downlink Pathloss_L is coded using 8 bits in 0.5 dB steps ranging from 52.5 dB (coded 0x00) to 180 dB(coded 0xFF). |
| Tx_SIR_ DownloadDownlink | Tx_SIR_ DownloadDownlink (SIRDL) is coded using 10 bits in 0.05 dB steps ranging from -10dB (coded 0x000) to 41.15 dB (coded 0x3ff). |

============================== End of Proposed Text =================================

[Remedy 2:]

[Modify text from line 24 on the page 660, under subsection 16.3.9.4.7.1, as follows:]

============================== Start of Proposed Text =================================

16.3.9.4.7 Uplink Power Status Reporting

16.3.9.4.7.1 Power Status Reporting Information

There are two key parameters for power status reporting: the base uplink transmission PSD the downlink pathloss L defined in section 16.3.9.4 and the SIRDL defined in section 16.3.9.4.1.

The base uplink transmission PSD is derived from equation (287) by setting SINRTarget=0 and Offset=0:

PSD(base)=L+NI				(292)

The parameter the downlink pathloss L is reported in dB and is coded using 8 bits in 0.5 dB steps ranging from 52.5 dB(coded 0x00) to 180 dB (coded 0xFF).

The reported SIRDLvalue is coded using 10 bits in 0.05 dB steps ranging from -10dB (coded 0x000) to 41.15 dB (coded 0x3ff).

After the ABS has received PSD(base) the downlink pathloss L and SIRDL from the AMS, the PSD for all uplink channels can be estimated in ABS.

The base uplink transmission PSD is derived from equation (287) by setting SINRTarget=0 and Offset=0:

PSD(base)=L+NI				(292)

============================== End of Proposed Text =================================