| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text of UL MIMO Mode Set in Group resource allocation (16.2.9) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Jeongki Kim, Wookbong Lee, Youngsoo Yuk, Ronny (Yongho) Kim, Jin Sam Kwak LG Electronics | E-mail: jeongki.kim@lge.com, wookbong.lee@lge.com, youngsoo.yuk@lge.com |
| Re: | Comments on IEEE P802.16m/D4 for IEEE 802.16 WG Letter Ballot Recirc #31 |  |
| Abstract | This contribution proposes the texts related to GRA MIMO Mode. |  |
| Purpose | To be adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text of UL MIMO Mode Set in Group resource allocation (16.2.9)

Jeongki Kim, Wookbong Lee, Youngsoo Yuk, Ronny (Yongho) Kim, Jin Sam Kwak

LG Electronics

- Introduction

According to 16m D4 [1], if UL MIMO Mode set ID in group resource allocation is 0b10, Mode 2 (CL SU-MIMO with Mt=1, TNS=2) is used as the MIMO mode of the group. However, TNS=2 cannot be used for UL CL SU-MIMO and UL CL SU-MIMO is unsuitable for UL group resource allocation because the downlink  control overhead will be increased in indication of PMI or because the pre-coding by mid-amble can be used only for TDD (not FDD). So, we propose the texts to clarify the UL MIMO mode set ID = 0b10.

- Use OL MU-MIMO with Mt=1, TNS=2 instead of CL SU-MIMO

  - An AMS can know the resource allocation size of each AMS belonging to the AMS’s group by Resource allocation bitmap (RAB) in Group resource allocation A-MAP IE.

  - Usage of group resource in Mode 3

    - An ABS allocates the first N (Indicated by RAB) LRUs (from resource offset) to the first scheduled AMS in a group and the stream index of the first scheduled AMS is 1.

    - If the resource size of the N-th scheduled AMS is different from that of the (N-1)-th scheduled AMS, then the resource of N-th AMS follows the resource of (N-1)-th AMS and the stream index of N-th AMS is 1.

    - If the resource size of the scheduled N-th AMS is the same as the scheduled (N-1)th AMS and the resource size of the scheduled (N-1)th AMS is the same as the scheduled (N-2)th AMS, then the resource of N-th AMS follows the resource of (N-1)-th AMS and the stream index of N-th AMS is 1.

    - If the resource size of the scheduled N-th AMS is the same as the scheduled (N-1)th AMS and the resource size of the scheduled (N-1)th AMS is different from that of the scheduled (N-2)th AMS, then the resource of N-th AMS is the same as the resource of (N-1)-th AMS and the stream index of N-th AMS is 2.

Figure 1

- References

[1] IEEE P802.16m/D4, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Text proposal for the 802.16m DRAFT amendment

---------------------------------------------------------Start of the Proposed Text ---------------------------------------------------------

[Remedy #1: Modify table 752 as follows:]

Table 752—UL MIMO mode set candidates

| MIMO Mode set ID | UL Group MIMO mode set | SM Restriction |
| --- | --- | --- |
| 0b00 | Mode 0 | N/A |
| 0b01 | Mode 0, Mode 1 | Mt=2 |
| 0b10 | Mode 2, Mode 3 | Mt=1 |
| 0b11 | Reserved | - |

[Remedy #2: Modify table 755 as follows:]

Table 755—MIMO Bitmap Information for UL

| MIMO Mode Set | Existence of Second Bitmap | Length of Bit per Scheduled AMS | MIMO Mode Indication |
| --- | --- | --- | --- |
| 0b00 | No | - | OL SU-MIMO (SFBC with non-adaptive precoder) |
| 0b01 | Yes | 1 | 0b0: OL SU-MIMO (SFBC with non-adaptive precoder) with Mt=2 0b1: OL SU-MIMO (SM with non-adaptive precoder) with Mt=2 |
| 0b10 | No | - | CL SUOL MU-MIMO with Mt=1, TNS=2 |

[Remedy #3: Modify the section 16.2.9.4 Normal Operation as follows:]

16.2.9.4 Normal Operation

An ABS shall provide contiguous allocations to the AMSs belonging to a group, which have the corresponding bit in the user bitmap set to '1'. The order of resource allocation shall be the same as the order in which they appear in the group's user bitmap. If there is no transmission for a given AMS in a certain AAI subframe, then the ABS shall set the corresponding bit in the user bitmap to '0'.

When an AMS receives a Group Resource Allocation IE in which the corresponding bit in user bitmap is set to '1', then the AMS shall decode the remaining bitmaps to determine other attributes of the allocated resource. An AMS shall determine the location of its allocation by counting the resource allocation sizes of other AMSs appearing before it in the user bitmap. If an AMS does not receive the Group Resource Allocation IE in any of the AAI subframes of a frame in which the IE was expected, then the AMS shall assume no allocations for the group in that frame.

In case that UL MIMO mode set ID is 0b10 (Mode3), the scheduled N-th AMS in user bitmap may use the same as the resource of the scheduled (N-1)-th AMS by the following rules. The first N (Indicated by RAB) LRUs from the value of resource offset is allocated to the first scheduled AMS and the stream index of the AMS is 1. If the resource size of the N-th scheduled AMS is different from that of the (N-1) th scheduled AMS, then the resource of N-th AMS follows the resource of (N-1) th AMS and the stream index of N-th AMS is 1. If the resource size of the scheduled N-th AMS is the same as the scheduled (N-1) th AMS and the resource size of the scheduled (N-1)-th AMS is the same as the scheduled (N-2) th AMS, then the resource of N-th AMS follows the resource of (N-1) th AMS and the stream index of N-th AMS is 1. If the resource size of the scheduled N-th AMS is the same as the scheduled (N-1) th AMS but the resource size of the scheduled (N-1) th AMS is different from that of the scheduled (N-2) th AMS, then the resource of N-th AMS is the same as the resource of (N-1) th AMS and the stream index of N-th AMS is 2.

The ACID corresponding to a resource allocated to an MS in a given frame is not explicitly signaled in the Group Resource Allocation A-MAP IE. The ACIDs for an AMS implicitly cycle in the ACID range defined in section 16.2.9.3.1.2. The ACID for an allocation shall be determined using the following formula.

ACID = Mod(Initial_ACID + Mod( floor( Frame_Number / Allocation_Period), N_ACID), 16)(3)

where,

ACID = ACID used for current allocation

Initial_ACID = Initial ACID parameter signaled in the Group Configuration MAC Control Message

Periodicity = Periodicity of group resource allocation in terms of number of frames signaled in the Group Configuration MAC Control Message

N_ACIDs = Number of ACIDs assigned to the AMS for group resource allocation, signaled in the Group Configuration MAC Control Message

Frame_Number = Current frame number, which is given by Equation (4) below

The Frame_Number parameter is determined as

Frame_Number = Superframe_Number * 4 + Frame_Offset(4)

Where Superframe_Number is the current superframe number and Frame_Offset is the offset of the current frame with respect to the start of the corresponding superframe and . The Frame_Number parameter corresponds to the frame in which the Group Resource Allocation A-MAP IE is transmitted.

If the calculated ACID is still performing retransmissions of a previous packet, then the previous packet shall be dropped and the ACID shall be freed for the new allocation. The packet drop shall happen in the frame in which Group Resource Allocation A-MAP IE is received.

-----------------------------------------------------------End of the Proposed Text   --------------------------------------------------------