| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Acknowledgement scheme of signaling header and extended header (P802.16m/D4-16.2.2.2.4) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Lei Zhang, Yang Liu, Yuqin Chen Nan Li, Mary Chion ZTE Corporation Jaesun Cha, Soojung Jung, Eunkyung Kim, Sungcheul Chang ETRI Chunmei Liu, Masoud Olfat Clearwire | zhang.lei13@zte.com.cn jscha@etri.kr chunmei.liu@clearwire.com masoud.olfat@clearwire.com |
| Re: | Category: LB31 / Area: Chapter 16.2.2.2.4 (MAC Control Message ACK Extended Header) |  |
| Abstract | This contribution proposes changes to the Section 16.2.2.2.4 of IEEE P802.16m/D4 |  |
| Purpose | Discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Acknowledgement scheme of signaling header and extended header (P802.16m/D4-16.2.2.2.4)

Lei Zhang, Yang Liu, Yuqin Chen, Nan Li, Mary Chion

ZTE Corporation

Jaesun Cha, Soojung Jung, Eunkyung Kim, Sungcheul Chang

ETRI

Chunmei Liu, Masoud Olfat

Clearwire

- Introduction

In D4, the signaling header or extended header could be used to carry standalone signaling. For example, in Sleep Mode, Sleep Control Header (SCH) and Sleep Control Extended Header (SCEH) could be used by the ABS or AMS to extend Listening Window, terminate Listening Window or switch Sleep Cycle setting. In addition, the Service Specific BR without STID Header may also be used to switching Sleep Cycle ID.

Currently, in order to synchronize the Sleep Mode state machines at both AMS and ABS side, the receiver side may send a SCH/SCEH to transmitter as the acknowledgement signaling. However the length of signaling header is fixed to 6 bytes regardless of its contents. It’s a waste of bandwidth to use 6-byte signaling header just to transmit acknowledgment.

In this contribution, we propose to extend MAEH and AAI_MSG-ACK format as a normal acknowledgement method for signaling header and extended header.

- Proposed text

======================== Start of Proposed Text ========================

[Modify the sentence (from line 58 on page 52) of P802.16m/D4 as follows]

- 16.2.2.2.4 MAC Control Message ACK Extended Header

The MAEH format is defined in Table 668. This header may be used by ABS and AMS to indicate the reception of a specific, previously received MAC control message, signaling header or extended header. When an ABS or AMS receives a MAC control message or MAC control message fragment with the Polling bit set to 1 in the MCEH, the ABS or AMS shall transmit an MAEH or AAI_MSG-ACK message as an acknowledgement after receiving the complete message with the SN of the MAC control message PDU or the SN of the last received fragment if fragmented. When an ABS or AMS receives a signaling header or extended header which requires an acknowledgment (i.e. SCH, SCEH or Service Specific BR without STID Header), the ABS or AMS may transmit an MAEH as an acknowledgement.

Table 668—MAEH Format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| MAEH() { |  |  |
| Type | 4 | MAEH Type |
| MAEH sub-type | 2 | Bit #0: Indicates reception of a control message Bit #1: Indicates reception of a signaling/extended header |
| If (Bit #0 of MAEH sub-type == 1) { |  |  |
| ACK_SN | 8 | SN retrieved from the MCEH of the MAC PDU with the Polling bit set to 1 |
| Control Connection Channel ID (CCC ID) | 1 | Control Connection Channel ID (CCC ID) that the MAC management control message is received. |
| Reserved | Variable | For byte alignment |
| } |  |  |
| If (Bit #1 of MAEH sub-type == 1) { |  |  |
| ACK_header_type | 5 | Indicates the header type to be acknowledged; 0: signaling header 1: extended header Bit #1-4: header type (See Table 656 & 662) |
| } |  |  |
| Reserved | Variable | For byte alignment |
| } |  |  |

[Modify the sentence (from line 7 on page 42) of P802.16m/D4 as follows]

- 16.2.2.1.3.3 Service Specific BR without STID Header

Service Specific BR without STID Header is sent through dedicated UL resource assigned to the AMS, which needs to change the scheduling parameters of its service flow. BR type shall be included to indicate whether BR type is increment or aggregate. Service Specific BR without STID header format is defined in Table 659. If SCID change indicator is set to 1, the ABS shall send an acknowledgement (i.e. SCH, SCEH, MAEH or AAI_MSG-ACK message) to confirm the change of sleep mode configuration.

[Modify the sentence (from line 31 on page 123) of P802.16m/D4 as follows]

- 16.2.3.35 AAI_MSG-ACK

The ABS and AMS may use AAI_MSG-ACK to indicate the reception of a MAC management, signaling header or extended header. When receiving a message over management connection with Polling bit set to 1 in MCEH, the ABS and AMS shall transmit AAI_MSG-ACK as an acknowledgement to the reception of the message. When an ABS or AMS receives a signaling header or extended header which requires an acknowledgment (i.e. SCH, SCEH or Service Specific BR without STID Header), the ABS or AMS may transmit an AAI_MSG-ACK as an acknowledgement. The AAI_MSG-ACK shall include the following information:

- SN retrieved from MCEH of the received MAC PDU

- Control Connection Channel ID (CCC ID) that the MAC control message is received

Table xxx – parameters for AAI_MSG-ACK

| Name | Value | Usage |
| --- | --- | --- |
| ACK type | Bit #0: Indicates reception of a control message Bit #1: Indicates reception of a signaling/extended header | It shall be included in AAI_MSG-ACK message. |
| ACK_SN | SN retrieved from MCEH of the received MAC PDU | It shall be included when Bit #0 of ACK type is equal to 1. |
| CCC ID | Control Connection Channel ID (CCC ID) that the MAC control message is received. | It shall be included when Bit #0 of ACK type is equal to 1. |
| ACK_header_type | Indicates the header type to be acknowledged; 0: signaling header 1: extended header Bit #1-4: header type (See Table 656 & 662) | It shall be included when Bit #1 of ACK type is equal to 1. |

======================== End of Proposed Text ========================

- References

[1] IEEE P802.16m/D4. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface, Feb 2010.