| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Cleanup of Ranging Purpose Indication in AAI_RNG-REQ (P802.16m/D4-16.2.3.1) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Lei Zhang Yang Liu ZTE Corporation | zhang.lei13@zte.com.cn |
| Re: | Category: LB31 / Area: Chapter 16.2.3.1 (AAI_RNG-REQ) |  |
| Abstract | This contribution proposes changes to the Section 16.2.3.1 of IEEE P802.16m/D4 |  |
| Purpose | Discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Cleanup of Ranging Purpose Indication in AAI_RNG-REQ (P802.16m/D4-16.2.3.1)

Lei Zhang, Yang Liu

ZTE Corporation

- Introduction

This contribution proposes to clean up the current Ranging Purpose Indication usage in P802.16m/D4 by efficiently reusing some ranging purpose codes.

- Proposed text

======================== Start of Proposed Text ========================

[Modify Table-676 on page 64 of P802.16m/D4 as follows]

Table 676—Parameters for AAI_RNG-REQ

| Name | Value | Usage |
| --- | --- | --- |
| … | … | … |
| Ranging Purpose Indication | The presence of this item in the message indicates the following AMS action: If Bit#0 is set to 1, it indicates: that a) the AMS is currently attempting HO reentry. , or, b) in combination with a Paging Controller ID, indicates that the AMS is attempting network reentry from idle mode to the ABS. c) in combination with CRID, indicates that the AMS is attempting network reentry from DCR mode to the ABS. In this case b) and c) above, Bit#1 shall be 0. If Bit#1 is set to 1, it indicates that the AMS is initiating the idle mode location update process, or, in combination with CRID, it indicates that the AMS is initiating DCR mode extension. In this case, Bit#0 shall be 0. If Bit#2 is set to 1, ranging request for emergency call setup. When this bit is set to 1, it indicates AMS action of Emergency Call process. If Bit#43 is set to 1, it indicates that the AMS is attempting to perform location update due to a need to update service flow management encodings for E-MBS flows, or, in combination with CRID, indicates that the AMS is initiating location update for transmission to DCR mode from idle mode. If Bit #5 is set to 1, it indicates that the AMS is initiating location update for transmission to DCR mode from idle mode. If Bit #6 is set to 1 in combination with ID of the network entity that assigns/retains the context, it indicates that the AMS is currently attempting reentry from DCR mode. If Bit#74 is set to 1, it indicates that the AMS is currently attempting network reentry after experiencing a coverage loss. If Bit#85 is set to 1, it indicates that the AMS is currently attempting network reentry from a IEEE802.16e only Legacy BS. Bit#6~8: Reserved | It shall be included when the AMS is attempting to perform reentry, HO, location update or DCR mode extension. |
| … | … | … |

======================== End of Proposed Text ========================

- References

[1] IEEE P802.16m/D4. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface, Feb 2010.