| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on update of S-SFH IEs (P802.16m/D4-16.2.23) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Lei Zhang Yanfeng Guan Feng Xie Xianming Chen Yang Liu ZTE Corporation | zhang.lei13@zte.com.cn |
| Re: | Category: LB31 / Area: Chapter 16.2.23 (Update of S-SFH IEs) |  |
| Abstract | This contribution proposes changes to the Section 16.2.23 of IEEE P802.16m/D4 |  |
| Purpose | Discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on update of S-SFH IEs (P802.16m/D4-16.2.23)

Lei Zhang, Yanfeng Guan, Feng Xie, Xianming Chen, Yang Liu

ZTE Corporation

- Introduction

Although the procedure to update S-SFH IE is described in D4, some description in this section is still very unclear. This proposal is used to clarify some operation description regarding update of S-SFH IEs.

- Proposed text

======================== Start of Proposed Text ========================

[Modify section 16.2.23 (from line 2 on page 336) of P802.16m/D4 as follows]

- 16.2.23 Update of S-SFH IEs

The S-SFH subpacket (SP) IEs (i.e., S-SFH SP1 IE, SP 2 IE and SP3 IE) are transmitted by ABS at the scheduled period of each SP, respectively, with different periodicity (i.e., TSP1 < TSP2 < TSP3). This SP scheduling periodicity information is transmitted in the S-SFH SP3 IE. Otherwise, an AMS may implicitly know each S-SFH SP IE's scheduling period by receiving each subsequent S-SFH SP IE.

Every superframe, the ABS transmits P-SFH IE containing the S-SFH Scheduling information bitmap, S-SFH change count, S-SFH SP change bitmap, and Start super-frame offset where new S-SFH information is used. The S-SFH change count shall remain unchanged as long as all of the values (except MSB of superframe number in S-SFH SP1 IE) of S-SFH SP IEs remain unchanged. The S-SFH change count shall be incremented by 1 modulo 16 whenever any of the values (except MSB of superframe number in S-SFH SP1 IE) of S-SFH IEs changes. The changed S-SFH SP IE(s) shall be transmitted at the scheduled superframes of period corresponding to each S-SFH SP IE.

If the AMS determines detects that the "S-SFH change count" field in P-SFH has not changed, then the AMS determines that it has up to date information.

When the AMS learns the super-frame number where S-SFH contents has have changed, it uses the "Rate of change of S-SFH Info" field in S-SFH SP3 IE to determine the super-frame number until which the contents of this S-SFH remain the same. For example, when the "Rate of change of S-SFH Info" field in S-SFH SP3 IE specifies that the contents of the S-SFH IE remains same over 32 * the periodicity of S-SFH SP1 superframes.

Each bit of the S-SFH SP change bitmap indicates the changing status of the corresponding S-SFH SP IE in association with the S-SFH change count. The bit # 0 (LSB), bit #1 and bit #2 (MSB) are mapped to S-SFH SP1 IE, S-SFH SP 2 IE and S-SFH SP3 IE, respectively. If any of the values (except MSBs of superframe number in S-SFH SP1 IE) of an S-SFH SP IE are changed, the bit corresponding to the changed S-SFH SP IE is toggled.

The Start super-frame offset where new S-SFH information is used in the P-SFH IE is used to indicate which S-SFH change count superframe shall be considered for applying the system parameters in the S-SFH SPx IEs.

According to the S-SFH change count, S-SFH SP change bitmap, and Start super-frame offset where new S-SFH information, AMS knows if it is needed to decode S-SFH IE in the current superframe to update the system parameters broadcasted within the S-SFH SP IE, and the AMS may decide to disregard the S-SFH IE with the presumption of there being no change in S-SFH SP IE compared to that already stored in the AMS.

AMS shall compare values of each S-SFH change count in the last received P-SFH IE and the last stored P-SFH IE whenever it receives P-SFH IE.

a) If there is no difference of two S-SFH change counts, AMS may not decode S-SFH IE in the superframe.

b) Else, AMS shall check that change bits in S-SFH SP change bitmap are toggled as the value of the difference.

1) If the number of toggled bits in the last received S-SFH SP change bitmap is the same as the difference, AMS shall update the S-SFH SP IE(s) whose bit of the S-SFH SP change bitmap is toggled. Additionally, AMS shall update SCD S-SFH change count and S-SFH SP change bitmap.

2) Else, AMS shall update all S-SFH SP IEs, SCD S-SFH change count and S-SFH SP change bitmap.

If the S-SFH SP IE indicated by S-SFH scheduling information corresponds to the changed S-SFH SP IE as indicated in the S-SFH change bitmap, the AMS shall decode the following S-SFH SP IE and store the parameters in association with the current S-SFH change count. The AMS shall update and store all the changed S-SFH SP IEs associated the increment of S-SFH change count at the scheduled superframe for each S-SFH SP IE.

======================== End of Proposed Text ========================

- References

[1] IEEE P802.16m/D4. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface, Feb 2010.