| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Optimizations on Resource Mapping Signaling Indication and IoT Information Reporting in SFH for IEEE 802.16m/D4 Amendment |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Xiangyu Liu , Kun Liu , Yanfeng Guan , Lei Zhang, Zhaohua Lu, Ding Ning ZTE Corporation | Email: liu.kun12, guan.yanfeng, zhang.lei13@zte.com.cn |
| Re: | Comments on P802.16m/D4 |  |
| Abstract | This contribution provides the proposed text for resource mapping signaling indication and IoT Information Reporting |  |
| Purpose | To be discussed and adopted into P802.16m/D4 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Optimizations on Resource Mapping Signaling and IoT Information Reporting for IEEE 802.16m/D4 Amendment

Xiangyu Liu, Kun Liu, Yanfeng Guan, Lei Zhang, Zhaohua Lu, Ding Ning

ZTE Corporation

- Introduction

This contribution proposes some modifications on IoT information reporting (16.3.9.4.7) and resource mapping signaling indication according to the frequency partition number in IEEE P802.16m/D4[1].

For Table 812 (S-SFH SP2 IE format) in Section 16.3.6.5.1.2 (S-SFH IE), ABS shall broadcast the resource mapping signaling indication information to all AMSs. Actually, all those information is related to frequency number. However, not all the indication information is necessary when frequency number is less than 4. So, if those redundant information can be omitted and not sent according to the frequency partition number.

- References

[1] IEEE P802.16m/D4, “Draft Amendment to IEEE Standard for Local and metropolitan area network: Air Interface for Fixed and Mobile Broadband Wireless Access Systems”, Feb. 2010.

- Text proposal

Instruction to editor;

Black text: existing D4 text

Red and strike out text: for deletion

Blue and underline text: for addition

Remedy: Modify texts in line 17 and 35 on the page 452 as follows:

============================== Start of Proposed Text =================================

16.3.6.5.1.2 S-SFH IE

Table 812 S-SFH SP2 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| … | … | … |
| DFPSC | 3/2/1 | See 16.3.5.2.3 DL Frequency partitioning When FPCT is 4, the size is For 2048 FFT size, 3 bits For 1024 FFT size, 2 bits For 512 FFT size, 1 bit Otherwise, the size is zero. |
| … | … | … |
| UFPSC | 3/2/1 | See 16.3.8.2.3 UL Frequency partitioning When FPCT is 4, the size is For 2048 FFT size, 3 bits For 1024 FFT size, 2 bits For 512 FFT size, 1 bit Otherwise, the size is zero. |
| … | … | … |

16.3.6.5.1.2 S-SFH IE

Table 813 S-SFH SP3 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| … | … | … |
| IoT correction value for UL power control | 10 4 or 6 or 8 or 10 | The At most 10-bits IoT correct value is used to support the correction of at most 5 IOT values (IOT Sounding, IOT_FP0, IOT_FP1, IOT_FP2, IOT_FP3), defined in AAI_UL_NI_message, each 2 bits are expressed as the correction value: 0b00 : +1 dB 0b01 : +0.5 dB 0b10 : 0 dB 0b11 : -0.5 dB The correction value is accumulated on IoT values from the latest AAI_ULPC_NI message until the new AAI_ULPC_NI message received and applied. The bit size depends on the FPCT value: 4 bit : if FPCT = 1; 6 bit : if FPCT = 2; 8 bit : if FPCT = 3; 10 bit : if FPCT = 4; The sequence of IoT correct value is IoT Sounding followed by the specific FP IoT value sequence. The specific FP sequence is selected in ascending order with frequency partition number of none zero value according to the UFPC in Table 880-Table 882. i.e. the UFPC is “1” in Table 881, the selected FP sequence is FP1、FP2、FP3. |
| … | … | … |

============================== End of Proposed Text =================================