| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AMS scanning of Neighbor Femto ABSs (16.4.8.1.2) |  |
| Date Submitted | 2010-3-5 |  |
| Source(s) | Feng Xie, Lin Chen, Yang Liu, Zhaohua Lu, ZTE Corporation | E-mail: {xie.feng2, chen.lin23, liu.yang8, lu.zhaohua}@zte.com.cn |
| Re: | Call for LB #31 on “ P802.16m/D4”: Target topic: “16.4.8.1.2” |  |
| Abstract | This contribution proposes the texts for AMS scanning procedures for neighbor Femto ABSs (16.4.8.1.2) |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

AMS Scanning of Neighbor Femto ABSs (16.4.8.1.2)

Feng Xie, Lin Chen, Yang Liu, Zhaohua Lu

ZTE Corporation

# 1. Introduction

In current P802.16m/D4, the procedures of AMS scanning of femto ABSs need clarifications.

In this proposal, the procedures of AMS scanning of femto ABS are described.

# 2. Text Proposal

Insert the following paragraph to Line 31 of Page 701

16.4.8.1.2 AMS scanning of neighbor femto ABSs:

======================== Start of Proposed Text =====================

For intra-frequency scanning of neighbor femto ABSs, an AMS detects the existence of neighbor femto ABSs by performing intra-frequency preamble measurement. Note that the intra-frequency preamble measurement does not interrupt AMS communication with the serving ABS. If the detected CELL_ID matches with the CELL_ID range of femto ABSs indicated in S-SFH SP3 and AAI_SCD message, the AMS has detected a neighbor femto ABS and is able to further distinguish OSG, CSG-Closed, CSG-Open femto ABSs. If a CSG femto ABS is detected, the AMS can quickly exclude inaccessible CSG femto ABSs if the detected CELL_ID is not included in AMS’s CSG White List. With detected CELL_ID of accessible femto ABSs, an AMS should negotiate with the serving ABS for a scanning interval to decode the SFH and/or CSGID Info of neighbor femto ABSs. Upon receiving the BSID and/or CSGID(s) of neighbor CSG femto ABSs, AMS shall check its accessibility with the CSG White List. If the decoded BSID and/or CSGID(s) is included in CSG White List, AMS can report to the serving ABS and/or initiate HO with the decoded BSID. Detailed procedures for detecting and selecting OSG or CSG femto ABSs can be found in Figure 574.

============================== End of Proposed Text ===============