| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | E-MBS-CFG change notification (16.9) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Jeongki Kim, Youngsoo Yuk, Kiseon Ryu, Ronny (Yongho) Kim, Jin Sam Kwak LG Electronics | E-mail: jeongki.kim@lge.com, youngsoo.yuk@lge.com |
| Re: | Comments on IEEE P802.16m/D4 for IEEE 802.16 WG Letter Ballot Recirc #31 |  |
| Abstract | This contribution proposes the texts related to the method of change indication of AAI_E-MBS-CFG. |  |
| Purpose | To be adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

E-MBS-CFG change notification (16.3.6.2.3/16.9)

Jeongki Kim, Youngsoo Yuk, Kiseon Ryu, Ronny (Yongho) Kim, Jin Sam Kwak

LG Electronics

- Introduction

According to D4 [1], upon receiving the first AAI_E-MBS-CFG message AMSs subscribing E-MBS can determine whether the AMSs decode the following AAI_E-MBS-CFG messages based on E-MBS_CFG_LIFETIME in AAI_E-MBS-CFG message and E-MBS-CFG change indication in E-MBS MAP.

- M1: E-MBS_CFG_LIFETIME (in AAI_E-MBS-CFG)

  - The duration for which parameters in this AAI_E-MBS-CFG message do not change.

  - Although the parameters related to the E-MBS Zone which the AMSs belong to don’t change, the AMSs should decode the AAI_E-MBS-CFG message because the E-MBS_CFG_LIFETIME is cell specific.

- M2: E-MBS-CFG change indication (in E-MBS MAP)

  - When the parameters in AAI_E-MBS-CFG change, the ABS sets the E-MBS-CFG change indication in the previous E-MBS MAP to 1 as shown figure 1.

Figure 1

  - If AMS does not decode the last E-MBS MAP transmitted before AAI_E-MBS-CFG message is transmitted, the AMS cannot know whether the parameters in next AAI_E-MBS-CFG change or not. It may result in the unnecessary decoding of AAI_E-MBS-CFG message by AMS.

- M3: E-MBS_CFG_LIFETIME + E-MBS-CFG change indication (As defined in D4)

  - The method can solve the problem that each method has respectively.

  - If the E-MBS-CFG change indication is set to 0 in the last E-MBS MAP before the E-MBS_CFG_LIFETIME is reset, the AMS resets the E-MBS_CFG_LIFETIME to the value in the latest AAI_E-MBS-CFG which the AMS receives.

  - However, according to the time when the AMS receives the first AAI_E-MBS-CFG, the E-MBS_CFG_LIFETIME reset by each AMS is different respectively as shown in Figure 2.

Figure 2

- Proposal

We propose the next E-MBS-CFG change count in order to indicate whether the parameters related to the E-MBS Zone in AAI_E-MBS-CFG change.

Table 1. E-MBS MAP

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| E-MBS-MAP () { | - | - |
| Next AAI_E-MBS-CFG Change Count | 1 | Indicates the change version of parameters related to this E-MBS Zone in next AAI_E-MBS-CFG Incremented by one (modulo 2) by the ABS, that is toggled, whenever any of the values of parameters related to this E-MBS Zone in the AAI_E-MBS-CFG message changes. If the value of this count in a subsequent E-MBS MAP remains the same, the AMS may assume that the parameters related to this Zone in the next AAI_E-MBS-CFG message have not changed and decide to skip the decoding of the next AAI_E-MBS-CFG message. |
| …… | …… | …… |
| E-MBS_DATA_IE() | - | - |
| } |  |  |
| Padding | variable | Padding to reach byte boundary |

Figure 3

If an AMS does not receive any E-MBS MAP for the AAI_E-MBS-CFG transmission period (320ms) before the AMS receives the AAI_E-MBS-CFG, the AMS must decode the AAI_E-MBS-CFG.

- References

[1] IEEE P802.16m/D4, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Text proposal for the 802.16m DRAFT amendment

---------------------------------------------------------Start of the Proposed Text ---------------------------------------------------------

[Remedy #1: Modify “16.3.6.2.3 Enhanced Multicast Broadcast Service MAP” section as follows:]

16.3.6.2.3 Enhanced Multicast Broadcast Service MAP (E-MBS MAP)

The E-MBS MAP carries configuration information for enhanced multicast broadcast service for one E-MBS Zone. The E-MBS MAP is transmitted in the first several RUs of the E-MBS region in the beginning of the MSI. The parameters of the E-MBS region and the burst size of the E-MBS MAP is transmitted in the AAI_E-MBS-CFG MAC control message described in 16.9.3.1. AAI_E-MBS-CFG Change Indication indicates any change in the parameters of AAI_E-MBS-CFG MAC Control Message at its next transmission instance.

Table 808: E-MBS MAP Structure

| Syntax | Size (bits) | Description/Notes |
| --- | --- | --- |
| E-MBS-MAP () { | - | - |
| AAI_E-MBS-CFG Change Indication | 1 | AAI_E-MBS-CFG Change Indication indicates any change in the parameters of AAI_E-MBS-CFG MAC Control Message at its next transmission instance. If it is set (AAI_E-MBS-CFG Change Indication = 1), parameters in the AAI_E-MBS-CFG message will change and AMS shall decode the next AAI_E-MBS-CFG message. Otherwise, the AAI_E-MBS-CFG message does not change and the AMS resets the Allocation Lifetime as defined in the previous AAI_E-MBS-CFG message and may not decode the next transmission. |
| Next AAI_E-MBS-CFG Change Count | 1 | Indicates the change version of parameters related to this E-MBS Zone in next AAI_E-MBS-CFG Incremented by one (modulo 2) by the ABS, that is toggled, whenever any of the values of parameters related to this E-MBS Zone in the AAI_E-MBS-CFG message changes. If the value of this count in a subsequent E-MBS MAP remains the same, the AMS may assume that the parameters related to this Zone in the next AAI_E-MBS-CFG message have not changed and decide to skip the decoding of the next AAI_E-MBS-CFG message. |
| … | … | … |
| E-MBS_DATA_IE() | - | - |
| } |  |  |
| Padding | variable | Padding to reach byte boundary |

[Remedy #2: Modify Table 949 as follows:]

Table 949—AAI_E-MBS-CFG_Message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AAI_E-MBS-CFG_Message_Format() { |  |  |
| MAC Control Message Type | 8 | AAI_E-MBS-CFG |
| E-MBS_CFG_LIFETIME (m) | 4 | Indicates the duration of E-MBS_CFG_LIFETIME for which the E-MBS configuration information of the zone do not change. Duration of E-MBS_CFG_LIFETIME: 16(m+1) superframes |
| …… | …… | …… |

-----------------------------------------------------------End of the Proposed Text   --------------------------------------------------------