| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text for carrier switching mode for E-MBS (16.9) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Jeongki Kim, Youngsoo Yuk, Kiseon Ryu, Ronny (Yongho) Kim, Jin Sam Kwak LG Electronics | E-mail: jeongki.kim@lge.com, youngsoo.yuk@lge.com |
| Re: | Comments on IEEE P802.16m/D4 for IEEE 802.16 WG Letter Ballot Recirc #31 |  |
| Abstract | This contribution proposes the texts related to carrier switching mode for E-MBS. |  |
| Purpose | To be adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text for carrier switching mode for E-MBS (16.9)

Jeongki Kim, Youngsoo Yuk, Kiseon Ryu, Ronny (Yongho) Kim, Jin Sam Kwak

LG Electronics

- Introduction

According to D4 [1], E-MBS may be provided by the specific carrier that is not AMS’s primary carrier and an AMS with only one transceiver in connected state switches from its primary carrier to the alternative carrier in order to receive E-MBS data bursts, E-MBS configuration message, E-MBS MAP, and so on. The AMS performs the carrier switching operation by using the E-MBS connection information (i.e., MSTIDs+FIDs) that is allocated to the AMS via DSA procedure.

In real E-MBS environment, basic/default E-MBS channels are allocated to all AMSs that subscribe E-MBS and the number of basic E-MBS channels is much more than that of special E-MBS channels. In this case the carrier switching method based on the E-MBS subscription information will result in the following problems.

- Restricted unicast scheduling: The ABS will allocate the unicast resource to AMSs subscribing E-MBS during the short and specific period because the ABS cannot know the activated channel information of the AMSs as shown in Figure 1.

Figure 1

  - Especially, it will also increase the possibility of collision between E-MBS allocations and a certain periodic allocation (e.g. persistent scheduling, CLC).

- If an AMS subscribes all special channels (i.e., the AMS is a premium user in E-MBS), the unicast service cannot be provided with the AMS until the AMS leaves E-MBS.

- Proposal

So, in carrier switching mode, an AMS subscribing E-MBS can know its activated E-MBS channel information from application layers (this is implementation issue of AMS and is out of scope in this specification) and the AMS notifies the ABS of the E-MBS connection information(i.e., MSTIDs+FIDs) related to its activated E-MBS channels whenever the activated E-MBS connection changes. ABS will allocate the unicast resource to an AMS with E-MBS connection based on the activated E-MBS connection information of the AMS. The AMS with only one transceiver performs the carrier switching operation to receive the E-MBS bursts based on the activated E-MBS connection information which the AMS reports to ABS.

Figure 2

Table 1 shows the AAI_E-MBS-REP message format. In order to report the activated E-MBS connection information, the AMS shall transmit the AAI_E-MBS-REP message to ABS.

Table 1

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AAI_E-MBS-REP_Message_Format() { |  |  |
| MAC Control Message Type | 8 | AAI_E-MBS-REP |
| Action Code | 1 | 0b0: E-MBS Connection Change 0b1: E-MBS Service Termination |
| If (Action code == 0b0) { |  |  |
| Num_E-MBS_Connections | 4 | The number of activated E-MBS connections which an ABS has. |
| for (i = 0; i < Num_E-MBS_Connections; i++) { |  |  |
| activated MSTID +FID | 16 | E-MBS connections (MSTID + FID) information for activated E-MBS services of AMS |
| } |  |  |
| } |  |  |

- References

[1] IEEE P802.16m/D4, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Text proposal for the 802.16m DRAFT amendment

---------------------------------------------------------Start of the Proposed Text ---------------------------------------------------------

[Remedy #1: Modify “16.9.2.2 E-MBS Operation in Connected State” section as follows:]

16.9.2.2 E-MBS Operation in Connected State

When an AMS moves across E-MBS zone boundaries in Active Mode or Sleep Mode, the AMS performs the handover procedure described in 16.2.6.3.

When the AMS transits to a new E-MBS Zone while in Active Mode or Sleep Mode, and the E-MBS service flow management encodings of the AMS have not been updated, the AMS shall send AAI_RNG-REQ message with Ranging Purpose Indication Bit#4 is set to 1 at the target ABS and the ABS shall include MSTID and FID Update in AAI_RNG-RSP parameters to provide updated service flow management encodings for any affected E-MBS flow as part of the handover procedure.

Once an AMS has received the E-MBS allocation information in the E-MBS-MAPs, it may not listen to the downlink channels till the next transmission of desired E-MBS flow or the next E-MBS-MAP.

When EMBS data is transmitted on an alternative carrier, i.e. other than the AMS’s primary carrier where service flows are configured the AMS is redirected to relevant carrier through DSA as described in 16.9.2.1. After the E-MBS connection establishment is completed, an AMS with only one transceiver can obtain its activated E-MBS channel information from application layer (This is an AMS’s implementation issue and is out of scope in this standard.) and the AMS may notify ABS of the E-MBS connection information related to its activated E-MBS channel by using AAI_E-MBS-REP message whenever the activated E-MBS connection changes. Upon receiving the activated E-MBS connection information of AMS the ABS will allocate the unicast resource to the AMS based on the activated E-MBS connection information of the AMS. The AMS performs the carrier switching operation to receive the E-MBS bursts or unicast bursts based on the activated E-MBS connection information which the AMS reports to ABS. The AMS may also notify the ABS of the termination of E-MBS service.

During the transmission of EMBS configuration messages and the EMBS data to which AMS is subscribed, the AMS with only one transceiver may not be available for signaling exchange with ABS on the primary carrier.

The AMS with multiple transceivers may be able to receive EMBS data while communicating with ABS on primary carrier.

Other EMBS operations in idle and connection states as described in 16.9.2.2 and 16.9.2.3 also applicable to this scenario.

The AMS does monitor down link channels and follow the entire idle mode and connected mode procedures as required by other services. While receiving E-MBS data, the AMS may temporary interrupt the reception of E-MBS packets for any reason, e.g. if time critical scanning is needed, without notifying the ABS.

[Remedy #2: Add the following section at the end of Section 16.2.3:]

16.2.3.54 AAI_E-MBS-REP (E-MBS report) message

The AAI_E-MBS-REP message provides the connection information for activated E-MBS service of an AMS. The AMS transmits the AAI_E-MBS-REP message to an ABS whenever the activated E-MBS channel changes or when the E-MBS service is terminated.

Table 736 –AAI_E-MBS-REP message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AAI_E-MBS-REP_Message_Format() { |  |  |
| MAC Control Message Type | 8 | AAI_E-MBS-REP |
| Action Code | 1 | 0b0: E-MBS Connection Change 0b1: E-MBS Service Termination |
| If (Action code == 0b0) { |  |  |
| Num_E-MBS_Connections | 4 | The number of activated E-MBS connections which an ABS has. |
| for (i = 0; i< Num_E-MBS_Connections; i++) { |  |  |
| activated MSTID + FID | 16 | E-MBS connections (MSTID + FID) information for activated E-MBS services of AMS |
| } |  |  |
| } |  |  |

-----------------------------------------------------------End of the Proposed Text   --------------------------------------------------------