| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | E-MBS MAP clean-up (16.3.6) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Jeongki Kim, Youngsoo Yuk, Kiseon Ryu, Ronny (Yongho) Kim, Jin Sam Kwak LG Electronics | E-mail: jeongki.kim@lge.com, youngsoo.yuk@lge.com |
| Re: | Comments on IEEE P802.16m/D4 for IEEE 802.16 WG Letter Ballot Recirc #31 |  |
| Abstract | This contribution proposes the texts to clean up the E-MBS MAP. |  |
| Purpose | To be adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

E-MBS MAP clean-up (16.3.6)

Jeongki Kim, Youngsoo Yuk, Kiseon Ryu, Ronny (Yongho) Kim, Jin Sam Kwak

LG Electronics

- Introduction

E-MBS MAP should be updated in order to remove ambiguity of E-MBS operation. We propose the texts to clean the E-MBS MAP.

- Text proposal for the 802.16m DRAFT amendment

---------------------------------------------------------Start of the Proposed Text ---------------------------------------------------------

[Remedy #1: Modify “16.3.6.2.3 Enhanced Multicast Broadcast Service MAP” section as follows:]

16.3.6.2.3 Enhanced Multicast Broadcast Service MAP (E-MBS MAP)

The E-MBS MAP carries configuration information for enhanced multicast broadcast service for one E-MBS Zone. The E-MBS MAP is transmitted in the first several RUs of the E-MBS region in the beginning of the MSI. The parameters of the E-MBS region and the burst size of the E-MBS MAP is transmitted in the AAI_E-MBS-CFG MAC control message described in 16.9.3.1. AAI_E-MBS-CFG Change Indication indicates any change in the parameters of AAI_E-MBS-CFG MAC Control Message at its next transmission instance.

Table 808: E-MBS MAP Structure

| Syntax | Size (bits) | Description/Notes |
| --- | --- | --- |
| E-MBS-MAP () { | - | - |
| AAI_E-MBS-CFG Change Indication | 1 | AAI_E-MBS-CFG Change Indication indicates any change in the parameters of AAI_E-MBS-CFG MAC Control Message at its next transmission instance. If it is set (AAI_E-MBS-CFG Change Indication = 1), parameters in the AAI_E-MBS-CFG message will change and AMS shall decode the next AAI_E-MBS-CFG message. Otherwise, the AAI_E-MBS-CFG message does not change and the AMS resets the Allocation Lifetime as defined in the previous AAI_E-MBS-CFG message and may not decode the next transmission. |
| S-SFH SP2 update indicator | 1 | Indicates whether the changed S-SFH SP2 will be transmitted in this MSI. If this field is set to 1, the time offset which the changed S-SFH SP2 is transmitted will be included in this E-MBS MAP. |
| If (S-SFH SP2 update indicator ==1) { |  | AMS shall wake up every S-SFH SP2 transmission period until the AMS receives the changed S-SFH SP2. |
| S-SFH SP2 transmission time offset | Variable | Indicates the superframe offset at which the changed S-SFH SP2 is transmittedchanged. The size of this field depends on MSI MSI == 0b00: 1 bits MSI == 0b01: 2 bits MSI == 0b10: 3 bits MSI == 0b11: 4 bits |
| } |  |  |
| E-MBS_DATA_IE() | - | - |
| } |  |  |
| Padding | variable | Padding to reach byte boundary |

-----------------------------------------------------------End of the Proposed Text   --------------------------------------------------------