| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Classification of CSG Femto ABS w/o CSGID (16.3.6.1.2) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Feng Xie, Zhaohua Lu, Yanfeng Guan, Huiying Fang ZTE Corporation | E-mail: {xie.feng2, lu.zhaohua, guan.yanfeng, fang.huiying}@zte.com.cn |
| Re: | Call for LB #31 on “ P802.16m/D4”: Target topic: “16.3.6.1.2” |  |
| Abstract | This contribution proposes the texts for Femto ABS Types (16.4.x) and SA-Preamble (16.3.6.1.2) |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on Femto ABS Types (16.4.x)

Feng Xie, Zhaohua Lu, Yanfeng Guan, Huiying Fang

ZTE Corporation

# 1. Introduction

CSG Femto ABS can be further classified to distinguish CSG femtos with and without CSGID. For CSG femtos with CSGID, AMS needs to perform accessibility check with CSGID. For CSG femtos without CSGID, AMS performs accessibility check with BSID.

# 2. Text Proposal

Accept the following Modifications to 16.3.6.1.2 Secondary advanced preamble (SA-Preamble):

Line 31 on Page 411

======================== Start of Proposed Text =====================

SA-Preamble sequences are partitioned and each partition is dedicated to specific base station type like macrocell ABS, Macro Hotzone ABS, Femto ABS. The base station types are categorized into macro ABS and non-macro ABS cells by hard partition with 258 sequences (86 sequences per segment * 3 segments) dedicated for macro ABS. The non-macro ABS information is broadcasted in a hierarchical structure, which composes of S-SFH SP3 and AAI_SCD message. In S-SFH SP3, non-macro ABS cell type is partitioned as public and CSG femto base stations. Total 16 cases of IDcell partition for public and CSG-femto ABS are shown in Table 798, which is composed of IDcell partitions based on 30 sequence (10 sequences per segmentation) granularity. The AAI_SCD message provides finer partition information. The non-macro ABS base stations can be further categorized different types, such as: hotzone, relay, OSG and etc. The CSG femto base stations can be further categorized, such as: CSG femtos with CSGID, CSG femtos without CSGID.

============================== End of Proposed Text ===============