| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text on the E-MBS Connection Establishment (16.2.3) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Eunkyung Kim, Soojung Jung, Jaesun Cha, Sungcheol Chang, Hyun Lee, Chulsik Yoon ETRI | Voice: +82-42-860-5415 E-mail: ekkim@etri.re.kr *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | “P802.16m/D4,” in response to the IEEE 802.16 Working Group Letter Ballot #31: Announcement, IEEE 802.16-10/0011 |  |
| Abstract | E-MBS Connection Establishment on the IEEE 802.16m Amendment Draft Standard |  |
| Purpose | To discuss and adopt the proposed text in the next revision of 802.16m draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed Text on the E-MBS Connection Establishment (16.2.3)

Eunkyung Kim, Soojung Jung, Jaesun Cha, Sungcheol Chang, Hyun Lee, Chulsik Yoon

ETRI

# Instructions

This contribution provides clarification on the E-MBS Connection Establishment and some parameters thereof in IEEE 802.16m Amendment Draft Standard [1].

In the current IEEE 802.16m Amendment Draft Standard [1], MSTID and FID is provided to identify a specific E-MBS flow in the domain of an E-MBS zone. The mapping of MSTID and FID can distinguish the E-MBS connection for each E-MBS contents in an E-MBS zone. However, there is no way how to distinguish the logical E-MBS connection for each E-MBS contents among E-MBS zones. Figure 1 shows an example of zone deployment scenario. As shown the Figure 1, ABS A supports same content via zone #1 and zone #2 at the same time but let the ABS A support the content different carrier #1 and #2 respectively. As shown Figure 1, because any AMS joining the E-MBS does not have any way to distinguish the same content between zone #1 and zone #2, AMS initiated DSA-REQ is not performed properly. Therefore, in this contribution, modified E-MBS connection establishment using E-MBS contents ID is proposed to support E-MBS properly.

Figure 1 – Zone deployment scenario (Zone ID#1 and Zone ID#2 are served by ABS A simultaneously)

# Text Proposal for the IEEE 802.16m Amendment Draft Standard

Note:

The text in BLACK color: the existing text in the 802.16m Amendment Draft Standard

The text in RED color: the removal of existing 802.16m Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16m Amendment Draft Standard Text

--------------Start of Proposed Text---------------------------------------------------------------------

[add the following into the line 6, page 138 in 16.2.3.45.1 of 802.16m/D4]

- E-MBS Contents ID: Indicates an E-MBS content requesting for the connection that is being setup.

[add the following into the line 21, page 139 in 16.2.3.45.2 of 802.16m/D4]

- E-MBS Contents ID: Indicates an E-MBS content requesting for the connection that is being setup.

[add the following row into the Table 759-Service flow/convergence sublayer paramet in 16.2.12.8 of 802.16m/D4]

| E-MBS contents ID | 8 | E-MBS contents ID, which indicates an E-MBS content requesting for the connection that is being setup. To support E-MBS, it shall be included in AAI_DSA-REQ/RSP message. |
| --- | --- | --- |

--------------End of Proposed Text ---------------------------------------------------------------------

# References

- IEEE P802.16m/D4, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access systems; Advanced Air Interface,” February 2010