| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | E-MBS Service Flow Parameters in IEEE P802.16m (16.2.12) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Eunkyung Kim, Soojung Jung, Jaesun Cha, Sungcheol Chang, Hyun Lee, Chulsik Yoon ETRI 138 Gajeongno, Yusong-gu, Daejeon, 305-700, KOREA | Voice: +82-42-860-5415 E-mail: ekkim@etri.re.kr |
| Re: | “P802.16m/D4,” in response to the IEEE 802.16 Working Group Letter Ballot #31: Announcement, IEEE 802.16-10/0011 |  |
| Abstract | Service flow parameters to support E-MBS on IEEE 802.16m Amendment Draft Standard |  |
| Purpose | To discuss and adopt the proposed text in the next revision of 802.16m draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# E-MBS Service Flow Parameters in IEEE 802.16m (16.2.12)

Eunkyung Kim, Soojung Jung, Jaesun Cha, Sungcheol Chang, Hyun Lee, Chulsik Yoon

ETRI

# Instructions

IEEE 802.16m Amendment Draft Standard [1] describes the Enhanced Multicast Broadcast Service (E-MBS).

According to the IEEE 802.16m Amendment Draft Standard [1], following parameters may be included in AAI_DSA-REQ/RSP to commence E-MBS service flow.

- E-MBS Service: Indicates whether the MBS service is being requested or provided for the connection that is being setup.

- E-MBS Zone ID: Indicates an E-MBS zone where the connection for associated service flow is valid.

- E-MBS Service Flow Parameter: Mapping of Multicast STID (MSTID) and FID are included.

- Physical Carrier Index: Target carrier which the AMS switches or is redirected by ABS to, only included in ABS initiated DSA-REQ.

Therefore, in this contribution, Service flow/convergence sublayer parameters are modified to establish the E-MBS service.

# Text Proposal for the 802.16m Amendment Draft Standard

Note:

The text in BLACK color: the existing text in the 802.16m Amendment Draft Standard

The text in RED color: the removal of existing 802.16m Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16m Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Modify the Table 759-Service flow/convergence sublayer paramet in 16.2.12.8 of 802.16m/D4]

Table 759—Service flow/convergence sublayer parameters

| Fields | Size (bits) | Description |
| --- | --- | --- |
| Flow ID | 4 | An identifier of a service flow |
| Uplink/Downlink Indicator | 1 | Whether parameters are for uplink or for downlink |
| Differentiated BR timer | 6 | Grant reception timeout before contention-based BR is attempted again for the service flow. Value of range 1 ~ 64 frame(s) |
| MAC in-order delivery indicator | 1 | Indicate whether or not the order of delivery in non-ARQ connection is preserved by the MAC.0 : not preserved1 : preserved |
| GPI primary | TBD |  |
| Grant size primary | TBD | Primary grant size. If the primary grant size equals to x bytes (the newly defined bandwidth request header size), this indicates a primarily polling based service; otherwise, it is primarily granting based service. |
| GPI secondary | TBD |  |
| Grant size secondary | TBD | Secondary grant size (optional). If the secondary grant size is defined and equals to x bytes (the newly defined bandwidth request header size), this indicates a secondarily polling based; otherwise, it is a secondarily granting based service. |
| Adaptation method | TBD | —Implicit adaptation —Explicit adaptation, sustained —Explicit adaptation, one time only |
| Service Class Name | 16 ~ 1024 | Refer to 11.13.3 |
| QoS Parameter Set Type | 3 | Refer to 11.13.4 |
| Traffic Priority | 3 | Refer to 11.13.5 |
| Access Class | 2 | This parameter specifies the priority assigned to a service flow. This priority is used in prioritizing access requests as described in paragraph 16.2.11.1.1. |
| Maximum Sustained Traffic Rate | 6 | Refer to 11.13.6 |
| Maximum Traffic Burst | 6 | Refer to 11.13.7 |
| Minimum Reserved Traffic Rate | 6 | Refer to 11.13.8 |
| Vendor-Specific QoS Parameter | variable | Refer to 11.13.109 |
| Uplink Grant Scheduling Type | 3 | Refer to 11.13.11. In addition to the legacy scheduling services described in 11.13.1110, aGP Service is supported. |
| Request/Transmission Policy | 8 | Refer to 11.13.1211 |
| Tolerated Jitter | 6 | Refer to 11.13.1312 |
| Maximum Latency | 6 | Refer to 11.13.1413 |
| Fixed-length versus Variable-length SDU Indicator | 1 | Refer to 11.13.1514 |
| SDU Size | 8 | Refer to 11.13.1615 |
| Target SAID | 16 | Refer to 11.13.1716 |
| ARQ Enable | 1 | Refer to 11.13.17.1 |
| ARQ_WINDOW_SIZE | 16 | Refer to 11.13.17.2 |
| ARQ_BLOCK_LIFETIME | 16 | Refer to 11.13.17.4 |
| ARQ_SYNC_LOSS_TIMEOUT | 16 | Refer to 11.13.17.5 |
| ARQ_RX_PURGE_TIMEOUT | 16 | Refer to 11.13.17.7 |
| RECEIVER_ARQ_ACK_PROCESSING TIME | 8 | Refer to 11.13.17.9 |
| ARQ_SUB_BLOCK_SIZE | 8 |  |
| ARQ_ERROR_DETECTION_TIMEOUT | 16 |  |
| ARQ_FEEDBACK_POLL_RETRY_TIMEOUT | 16 |  |
| CS Specification | variable | Refer to 11.13.1918 |
| Unsolicited Grant Interval | 6 | Refer to 11.13.2019 |
| Unsolicited Polling Interval | 6 | Refer to 11.13.2120 |
| MBS service | 2 | Refer to 11.13.2322 |
| Global Service Class Name | variable | Refer to 11.13.2423 |
| Type of Data Delivery Services | 3 | Refer to 11.13.2524 |
| SDU Inter-arrival Interval | 16 | Refer to 11.13.2625 |
| Time Base | 16 | Refer to 11.13.2726 |
| MBS zone identifier assignmentE-MBS_Zone_ID | 8 | Refer to 11.13.29E-MBS Zone identifier, which indicates an E-MBS zone where the connection for associated service flow is valid. To support E-MBS, it shall be included in AAI_DSA-REQ/RSP message. |
| Paging Preference | 1 | Refer to 11.13.3028 |
| SN Feedback Enabled | 1 | Refer to 11.13.3129 |
| HARQ Service Flows | 1 | Refer to 11.13.3230 |
| Authorization Token | variable | Refer to 11.13.3432 |
| HARQ Channel Mapping | variable | Refer to 11.13.3533 |
| PDU SN Extended Subheader for HARQ Reordering | 2 | Refer to 11.13.3634 |
| MBS contents ID | variable | Refer to 11.13.37 |
| ROHC Parameter Payload | variable | Refer to 11.13.3836 |
| Packet Error Rate | 8 | Refer to 11.13.3937 |
| PSC assignment | 3 | Refer to 11.13.4038 |
| Emergency Indication | 1 | Indicates the associated flow is used for emergency purposes. |
| MAC Header Type | 1 | Indicates whether AGMH or CMH is presented at the start of MPDUs of the service flow 0 = AGMH (Advanced Generic MAC Header) 1 = CMH (Compact MAC header) |
| E-MBS Service Flow | 16 | Mapping of E-MBS STID+FID To support E-MBS, it shall be included in AAI_DSA-REQ/RSP message. |
| E-MBS service | 2 | 0: No available E-MBS 1: E-MBS in Serving ABS Only 2: E-MBS in a multi-ABS Zone supporting macro-diversity 3: E-MBS in a multi-ABS Zone not supporting macro-diversity |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

- IEEE P802.16m/D4, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access systems; Advanced Air Interface,” February 2010