| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comment on UL Multi-BS MIMO in IEEE P802.16m/D4 (16.5.2.1) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Kanchei(Ken) Loa, Chun-Che Chien, Youn-Tai Lee, Chun-Yen Hsu, Tsung-Yu Tsai, Yi-Ting Lin Institute for Information Industry (III) Yih Guang Jan, Tamkang University | E-mail: loa@iii.org.tw yihjan@yahoo.com |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirc #30c P802.16m/D4 : 16.5.2.1 |  |
| Abstract | This contribution proposes the text change for UL Multi-BS MIMO. |  |
| Purpose | To be discussed and adopted by TGm for the IEEE P802.16m Draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comment on UL Multi-BS MIMO

in IEEE P802.16m/D4 (16.5.2.1)

Kanchei(Ken) Loa, Chun-Che Chien, Youn-Tai Lee, Chun-Yen Hsu, Tsung-Yu Tsai, Yi-Ting Lin

Institute for Information Industry (III)

Yih Guang Jan, 
Tamkang University

# Introduction

In UL single BS precoding with Multi-BS Coordination, the precoder at the AMS is performed by combining two PMIs. One of them maximizes the transmission power to the serving BS (PMI), the other one minimizes the interference to the neighboring BS (PMImin) in [1]. However, since PMImin could be jointly determined by neighboring ABSs through backhaul network, the benefit from PMImin selection should not be confined to a dedicated neighboring ABS. To make this possible, the neighboring ABSs should be capable of selecting the PMImin such that the overall interference generated from the AMS is minimized. In this consideration, we suggest the corresponding changes in [1].

# Reference

[1] IEEE P802.16m/D4, “Part 16: Air Interface for Broadband Wireless Access Systems: Advanced Air Interface,” Feb 2010.

- Text proposal for inclusion in the 802.16m Draft 4

[Modify the following text in Section 16.5.2.1]

------------------------------------------------------------------------- Text Start --------------------------------------------------------------------------

16.5.2 UL Multi-BS MIMO

16.5.2.1 Single BS precoding with Multi-BS Coordination

16.5.2.1.1 DL Signaling

Table 939 shows the control parameters for UL multi-BS MIMO.

Table 939 – Control parameters for UL Multi-BS MIMO

| Parameter | Description | Value | Control channel (IE) | Notes |
| --- | --- | --- | --- | --- |
| PMImin | PMI minimizing interference |  | AAI_MultiBS_PMI_COM message | PMImin is negotiated by neighboring ABSs and selected from uplink SU_MIMO codebook in 16.3.10.3 |
| PCR | PMI combination ratio |  | AAI_MultiBS_PMI_COM message |  |

16.5.2.1.2 Operation procedure

Single BS precoding with Multi-BS Coordination is performed by combining two PMIs in TDD or FDD UL transmission for mitigating inter-cell interference (ICI) when CL MIMO precoding is applied for the serving and neighboring cells. One of PMIs maximizes the transmission power of the serving cell; another the other  minimizes interference generated to the one or more neighboring cells.

The operation procedure of PMI combination follows the steps below.

1) PMI combination may be triggered by an ABS in an unsolicited manner. By using UL sounding signals, the serving and neighboring ABSs may measure channels of AMSs. Upon the channel measurements, the serving and neighboring ABSs may determine PMI maximizing transmission power and PMImin minimizing interference, respectively. The neighboring ABSs may jointly determine and inform the serving ABS of PMImin through the backhaul network. Two PMIs of the predefined codebook {wi Wi}, should be selected based on the following procedure:

where Hs and HI denote information of channels from the AMS to its serving and one of corresponding neighboring ABSs, respectively, is relating in some way to the interference power in each neighboring ABS and the total interference powers.

2)The serving ABS shall inform the PMI in A-MAP IE and PMImin and PCR (PMI combination ratio) in an AAI_MultiBS_PMI_COM message to the AMS.The transmitted precoder W combining two precoders  should be generated as follows:

(313)

------------------------------------------------------------------------ Text End ---------------------------------------------------------------------------