| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Range of value for ABS_EIRP and EIRxPIR,min (16.3.6.5.1.2) |  |
| Date Submitted | 2010-03-16 |  |
| Source(s) | Jeongho Park, Hwasun Yoo, Hokyu Choi Samsung Electronics Co., Ltd. Wookbong Lee, LG Electronics Rongzhen Yang Intel | Jeongho.jh.park@samsung.com |
| Re: | IEEE 802.16-10/0011, “IEEE 802.16 Working Group Letter Ballot #31: Announcement” |  |
| Abstract | The contribution proposes the clean up text to be included in IEEE 802.16m/D5. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 5) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Range of value for ABS_EIRP and EIRxPIR,min (16.3.6.5.1.2)

Jeongho Park, Hwasun Yoo, and Hokyu Choi 
Samsung Electronics Co., Ltd.

Wookbong Lee,

LG Electronics

Rongzhen Yang

Intel

- Introduction

The contribution proposes the specific range of value for two parameters – one is ABS_EIRP in SFH SP1 and the other is EIRxPIR,min in SFH SP2. Currently they have 7 bits and 5 bits field respectively, but does not have its values.

- Backgrounds

a. BS_EIRP

According to IEEE80216-2009, BS EIRP is specified in MOB_NBR-ADV message as 8bits from -128 to 127dBm with 1dB step size.

BS EIRP represnets the equivalent isotrophic radiation power from base station and actually is expected to vary much small range. For the given 7 bits in D4, suggestion is to reduce the range such as from -64 to 63dBm.

b. EIRxPIR,min

This value represents the target received total power for initial ranging channel. And this is the value that AMS should refer to whenever AMS tries to do initial ranging. Based on this parameter, AMS set the initial total power for initial ranging channel. Therefore, this value should set depending on the performance of initial ranging channel.

D4 has two types of initial ranging channel; format 0 and format 1. This format impacts on performance. The number of receive antenna at base stations impacts on performance. So based on type of format and number of RX antenna, we can estimate the range of target signal power.

|  | Format0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 4RxCase |  |  |  |  | 2RxCase |  |  |  |  |
| RequiredSINR | -11 |  |  | dB |  | -5 |  |  | dB |  |
| Noisepowerdensity | -174 |  |  | dBm/Hz |  | -174 |  |  | dBm/Hz |  |
| Subcarrierspacing | 10940 |  |  | Hz |  | 10940 |  |  | Hz |  |
| SubcarrierspacingforRNG | 5470 |  |  | Hz |  | 5470 |  |  | Hz |  |
| NumberoftonesforRNG | 139 |  |  | tones |  | 139 |  |  | tones |  |
| Totalnoisepower | -115.19 |  |  | dBm |  | -115.19 |  |  | dBm |  |
| AssumingIoT | 0 | 10 |  | dBm |  | 0 | 10 |  | dBm |  |
| Interferencepower | -115.19 | -105.19 |  | dBm |  | -115.19 | -105.19 |  | dBm |  |
| NI | -112.18 | -104.776 |  | dBm |  | -112.18 | -104.776 |  | dBm |  |
| Signal power | -123.18 |  | -115.776 |  | dBm | -117.18 |  | -109.776 |  | dBm |

The table1 shows the format0 case. Here 0~10dB IoT range is assumed. For the given required SINR in various channels, the maximum required SINR value is -5dB and minimum required SINR valueis -11dB. Based on these number, we can easily calculate the target signal power as being described in the table.

Similary the table 2 shows the format1 case.

|  | Format1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 4RxCase |  |  | 2RxCase |  |  |
| RequiredSINR | -14 |  | dB | -8 |  | dB |
| Noisepowerdensity | -174 |  | dBm/Hz | -174 |  | dBm/Hz |
| Subcarrierspacing | 10940 |  | Hz | 10940 |  | Hz |
| SubcarrierspacingforRNG | 1367.5 |  | Hz | 1367.5 |  | Hz |
| NumberoftonesforRNG | 557 |  | tones | 557 |  | tones |
| Totalnoisepower | -115.182 |  | dBm | -115.182 |  | dBm |
| AssumingIoT | 0 | 10 | dBm | 0 | 10 | dBm |
| Interferencepower | -115.182 | -105.19 | dBm | -115.19 | -105.182 | dBm |
| NI | -112.172 | -104.776 | dBm | -112.18 | -104.768 | dBm |
| Signal power | -126.172 | -118.768 | dBm | -120.172 | -112.768 | dBm |

From all cases, we can select the minimum and maximum case as -126.2dBm and -109.7dBm. After averaging them, -117dB can be obtained and we can extend its range using 5bits with 1dB step such as -133~-102dBm.

Finally, the contribution suggests to use from -133 to -102dBm for the range of value of EIRxPIR,min.

- Textproposal

Remedy#1.<Change the line 45 in page 450 as follows:>

--------------------------------------------------TextStart---------------------------------------------------

ABS_EIRP   |   76   |  Signed Unsigned integer from 1 to 64 in units of 1dBm

---------------------------------------------------Textend---------------------------------------------------

Remedy#2.<Change the line 45 in page 452 as follows:>

---------------------------------------------------TextStart---------------------------------------------------

EIRxPIR,min   |   5   |  Unsigned integer from -133 to -102 in units of 1dBm

---------------------------------------------------Textend---------------------------------------------------