| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text for sounding channel power control in case of multiple antenna (16.3.6.5.2.4.6, 16.3.9.4.5) |  |
| Date Submitted | 2010-03-15 |  |
| Source(s) | Jeongho Park, Hokyu Choi Samsung Electronics Co., Ltd. Wookbong Lee, LG Electronics Alexei Davydov, Rongzhen Yang Intel | Jeongho.jh.park@samsung.com |
| Re: | IEEE 802.16-10/0011, “IEEE 802.16 Working Group Letter Ballot #31: Announcement” |  |
| Abstract | The contribution proposes the clean up text to be included in IEEE 802.16m/D5. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 5) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text for sounding channel power control in case of multiple antenna (16.3.6.5.2.4.6, 16.3.9.4.5)

Jeongho Park and Hokyu Choi

Samsung Electronics Co., Ltd.

Wookbong Lee,

LG Electronics

Alexei Davydov, Rongzhen Yang

Intel

- Introduction and Backgrounds

The contribution proposes the clarification for uplink sounding channel power control. D4 defines the power control algorithm for sounding channel, but it does not give description in case of multiple antenna transmission

If AMS has more than one TX antenna and receives Sounding Command A-MAP IE with ‘Antenna switching’ field of 1, then the AMS shall transmit sounding channel in a way of either decimation or cyclic-shift manner.

Tone power for sounding sequence is determined by equation (287) and (288), but total power from AMS is different according to

- Option 1) Each tone power is applied to each antenna as it is

- Option 2) Each tone power is applied to each antenna as being normalized by the number of TX Antenna

Option 1 will provides the good quality of the receved sounding sequence from each TX antenna while the quality from option 2 is worse than option 1. On the other hand, option 1 will results in sounding pollution, which means the high interference within sounding channel to other AMSs. Also, since option 1 will consume double power, it is worse in terms of AMS’s battery view.

Each option has its own pros and cons.

Therefore the contribution suggests that ABS have the controllability its ‘power normalization by the number of AMS TX antenna’ and ‘power boosting’ by signaling in Sounding Command A-MAP IE.

- Textproposal

Remedy#1.<Change the line 53~58  in page 502 as follows:>

--------------------------------------------------TextStart---------------------------------------------------

| Antenna switching | 1 | 0b0: Antenna switching 0b1: No antenna switching |
| --- | --- | --- |
| Power boosting | 1 | 0b0: No power boosting 0b1: 3dB power boosting |
| Padding | variable |  |
| } |  |  |

---------------------------------------------------Textend---------------------------------------------------

Remedy#2.<Include the following text in line 33 of page 503 >

---------------------------------------------------TextStart---------------------------------------------------

If power boosting field is set 1, 3dB tone power boosting is applied to each TX antenna.

---------------------------------------------------Textend---------------------------------------------------

Remedy#3.<Chane line 16~53 in page 659 as follows: >

---------------------------------------------------TextStart---------------------------------------------------

16.3.9.4.5 Sounding Channel Power Control

Power control for the UL sounding channel is supported to manage the sounding quality. AMS’s transmit power for UL sounding channel is controlled separately according to its sounding channel target SINR value. The power per subcarrier per TX antenna shall be maintained for the UL sounding transmission as shown in Equation (287) of 16.3.9.4.1.

In Equation (287), SINRTarget is the sounding channel target SINR, which is set according to the DL SIR of the AMS defined by parameter SIRDL. In order to maintain the UL sounding quality, the different target SINR values are assigned according to the DL SIR of each AMS; the AMS with high DL SIR applies relatively high target SINR and the AMS with low DL SIR applies relatively low target SINR.

The SINRTarget for sounding channel shall be calculated from Equation (288) using the following parameter settings: SINRMIN is the minimum SINR requirement expected by ABS which is set to SINRmin_SOUNDING soundingSinrMin,is set to the IoT control factor for sounding channel GAMMA_IOT_SOUNDING gammaIotSounding, SIRDL is the ratio of the downlink signal vs. noise and interference power, measured by the AMS, and β parameters shall be set to 0 and 1 respectively, TNS is set to 1 when antenna switching field equals 0 and TNS is set to the number of TX antenna when antenna switching field equals 1.

When calculated sounding SINRTarget is higher than soundingSinrMax SINRmax_SOUNDING, the SINRTarget shall be set to soundingSinrMax SINRmax_SOUNDING.

If power boosting field is set 1, 3dB tone power boosting is applied to each TX antenna.

All parameters necessary for sounding channel power control are transmitted through AAI_SCD message in 16.2.3.29.

---------------------------------------------------Textend---------------------------------------------------