| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments on Femto ABS Detection, Identification and Selection (16.4.7.1) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Chun-Yen Hsu, Chiu-Wen Chen, Yi-Ting Lin, Chun-Che Chien, Tsung-Yu Tsai, Youn-Tai Lee, Kanchei(Ken) Loa Institute for Information Industry Whai-En Chen, Shih-Yuan Cheng National Ilan University Yih Guang Jan, Yang-Han Lee Tamkang University | E-mail: hcy@iii.org.tw *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16m-10/0011: IEEE 802.16 Working Group Working Group Letter Ballot #31 Section 16.4.7.1 Femto ABS Detection, Identification and Selection |  |
| Abstract | This contribution suggests that the cell search method provided is for illustration only as other methods may also exist. |  |
| Purpose | To be discussed and adopted by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comments on Femto ABS Detection, Identification and Selection (16.4.7.1)

Chun-Yen Hsu, Chiu-Wen Chen, Yi-Ting Lin, Chun-Che Chien, Tsung-Yu Tsai, Youn-Tai Lee, Kanchei(Ken) Loa

Institute for Information Industry

Whai-En Chen, Shih-Yuan Cheng

National Ilan University

Yih Guang Jan, Yang-Han Lee

Tamkang University

# Introduction

Figure 584 contains certain errors. For example, AMS can identify Macro cells, CSG Femto cells and other public ABSs by IDcell (see 16.3.6.1.2). AMS cannot identify OSG Femto cells from IDcell. It is suggested to remove this Figure. Corresponding text are also modified.

# Proposed Text

Modify Section 16.4.7.1 as follows.

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

16.4.7.1 Femto ABS detection, identification and selection

The AMS selects the target Femto ABS for network entry based on the subscription types of the Femto ABS and the membership to the relevant subscribers groups or accessibility to the service providers network (ex: Cell bar).

An AMS is considered Femto-Preferred if it has a preference of selecting a Femto ABS while Macro ABS is within reach.

If an AMS detects a Cell_ID belongs to  a CSG Femto ABS which may allow limited services to mobile stations that are not subscribed and the AMS is not a member of any CSG, the AMS may continue to detect P-SFH and S-SFH and may proceed to ACCESS STATE, if the AMS cannot be served by other neighboring macro or OSG Femto ABSs.

During network entry or re-entry, the AMS shall begin scanning of the neighbor base stations through RF measurements. The detection of the Cell_ID helps categorizing the ABS type and depending on the preference of the AMS, a Macro or Femto candidate is selected. Failure in any stage of the cell search and cell selection will result in repeating the scanning and DL synchronization. Cell_ID and associated FA index, and the LSB of BS-ID (or the MSB of BS-ID if SP2 is decoded early prior to SP1) will also help the AMS to quickly  exclude  the  CSG  Femto  ABSs  to  which  it  is  not  subscribed.  Full  BSID  (48  bits)  or  the  derived CSGID is the exact identifier for the AMS to determine whether it is authorized to access to the target ABS. The AMS have the common identifiers in its White List (i.e., a local table in the AMS containing the identi-ties of all the CSG Femto base stations to which the AMS is subscribed and is authorized to access) in order to be authorized to access to the target Femto ABS.

For a legacy MS WirelessMAN-OFDMA MS or a Femto-Unaware AMS, the cell search and cell selection procedures are identical to those specified in IEEE Std 802.16-2009 and this specification, respectively, and some transparent optimizations may be made that are outside the scope of this specification.

------------------------------------------------------ End of Proposed Text ------------------------------------------------------