| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AAI_MSG-ACK Message cleanup (16.2.2.2) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Youngbin Chang, Hyunjeong Kang, Jaehyuk Jang, Youngkyo Baek, Rakesh Taori, Jungje Son Samsung Electronics | anilag@samsung.com, yb.chang@samsung.com, *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16-10/0011, IEEE 802.16 Working Group Letter Ballot Recirc #31 / Topic: Header Format (Section 16.2.2.2) |  |
| Abstract | This contribution proposes AAI_MSG-ACK message cleanup |  |
| Purpose | To be discussed and adopted by TGm for P802.16m/D5. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

AAI_MSG-ACK message cleanup

Youngbin Chang, Hyunjeong Kang, Jaehyuk Jang, Youngkyo Baek, Rakesh Taori, Jungje Son

Samsung Electronics

# Introduction

This contribution proposed the message table and ASN.1 code for AAI_MSG-ACK.

# Proposed text

[Modify the text from line 28 on page 123, as follows:]

16.2.3.35 AAIAAI_MSG-ACK

The ABS and AMS may use AAI_MSG-ACK to indicate the reception of a MAC control message. When receiving a message over control connection with ‘Polling’ bit set to 1 in MCEH, the ABS and AMS shall transmit AAI_MSG-ACK as an acknowledgement to the reception of the message. The AAI_MSG-ACK shall include the following information:

—SN retrieved from MCEH of the received MAC PDU

—Control Connection Channel ID (CCC ID) that the MAC control message is received

Table xxx - AAI_MSG-ACK

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | ACK_SN | 8 | SN retrieved from MCEH of the received MAC PDU. | NA |
| M | CCC ID | 1 | Control Connection Channel ID (CCC ID) that the MAC management message is received. | NA |

-- ASN1START

AAI-MSG-ACK ::= SEQUENCE {

messageType         OCTET STRING(SIZE(1)),

-- SN retrieved from MCEH of the received MAC PDU

ackSN               INTEGER (0 .. 256),

-- Control Connection Channel ID

cccId               INTEGER (0 .. 1)

}

-- ASN1STOP