| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | ARQ parameter cleanup in REG-REQ/RSP (16.2.3) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Youngbin Chang, Anil Agiwal, Rakesh Taori, Jungje Son Samsung Electronics | anilag@samsung.com, yb.chang@samsung.com, *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16-10/0011, IEEE 802.16 Working Group Letter Ballot Recirc #31 / Topic: MAC control message (Section 16.2.3.7, 16.2.3.8) |  |
| Abstract | This contribution proposes ARQ parameters cleanup in REG-REQ/RSP |  |
| Purpose | To be discussed and adopted by TGm for P802.16m/D5. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

ARQ parameter cleanup in REG-REQ/RSP

Youngbin Chang, Anil Agiwal, Rakesh Taori, Jungje Son

Samsung Electronics

# Introduction

This contribution proposed the ARQ parameter cleanup text for REG-REQ/RSP. ARQ parameters can be decided during registration or DSA process. If ARQ parameters are decided during registration, these parameters are commonly used all ARQ enabled flows. Some ARQ timers in ARQ section are missed in the REG message. This contribution proposes adding these missed parameters. ARQ_SN_MODULUS is default value (2^10), which does not need to negotiate during registration.

# Proposed text

[Modify ARQ parameters in the table 682 on page 74, as follows:]

Table 682—Parameters for AAI_ REG-REQ

| Name |  |  | Value |
| --- | --- | --- | --- |
| AMS capability negotiation parameters | ARQ parameters | ARQ_SN_MODULUS | the number of unique SN values; default =2^10 |
|  |  | ARQ_WINDOW_SIZE | Bit[4:0] Refer to 16.2.13.3.2the maximum number of ARQ blocks with consecutive SN in the sliding window of ARQ blocks |
|  |  | ARQ_SUB_BLOCK_SIZE | Bit[3:0] Refer to 16.2.13.3.3ARQ sub-block length when ARQ block is fragmented into ARQ sub-blocks prior to retransmission with rearrangement |
|  |  | ARQ_BLOCK_LIFETIME | Bit[15:0] Refer to 16.2.13.3.4 the maximum time interval an ARQ block shall be managed by the transmitter ARQ state machine, once initial transmission of the block has occurred |
|  |  | ARQ_RX_PURGE_TIMEOUT | Bit[15:0] Refer to 16.2.13.3.5the time interval the receiver shall wait after successful reception of a block that does not result in advancement of ARQ_RX_WINDOW_START, before advancing ARQ_RX_WINDOW_START |
|  |  | MAXIMUM_ ARQ_BUFFER_SIZE | Bit[15:0] Refer to 16.2.13.3.6 The maximum ARQ buffer size |
|  |  | ARQ_SYNC_LOSS_TIMEOUT | Bit[15:0] Refer to 16.2.13.3.7 |
|  |  | ARQ_ERROR_DETECTION_TIMEOUT | Bit[15:0] Refer to 16.2.13.3.8 |
|  |  | ARQ_FEEDBACK_POLL_RETRY_TIMEOUT | Bit[15:0] Refer to 16.2.13.3.9 |

[Modify ARQ parameters in the table 682 on page 78, as follows:]

Table 683—Parameters for AAI_ REG-RSP message

| Name |  |  | Value |
| --- | --- | --- | --- |
| AMS capability negotiation parameters | ARQ parameters | ARQ_SN_MODULUS | the number of unique SN values; default =2^10 |
|  |  | ARQ_WINDOW_SIZE | Bit[4:0] Refer to 16.2.13.3.2the maximum number of ARQ blocks with consecutive SN in the sliding window of ARQ blocks |
|  |  | ARQ_SUB_BLOCK_SIZE | Bit[3:0] Refer to 16.2.13.3.3ARQ sub-block length when ARQ block is fragmented into ARQ sub-blocks prior to retransmission with rearrangement |
|  |  | ARQ_BLOCK_LIFETIME | Bit[15:0] Refer to 16.2.13.3.4 the maximum time interval an ARQ block shall be managed by the transmitter ARQ state machine, once initial transmission of the block has occurred |
|  |  | ARQ_RX_PURGE_TIMEOUT | Bit[15:0] Refer to 16.2.13.3.5the time interval the receiver shall wait after successful reception of a block that does not result in advancement of ARQ_RX_WINDOW_START, before advancing ARQ_RX_WINDOW_START |
|  |  | MAXIMUM_ ARQ_BUFFER_SIZE | Bit[15:0] Refer to 16.2.13.3.6 The maximum ARQ buffer size |
|  |  | ARQ_SYNC_LOSS_TIMEOUT | Bit[15:0] Refer to 16.2.13.3.7 |
|  |  | ARQ_ERROR_DETECTION_TIMEOUT | Bit[15:0] Refer to 16.2.13.3.8 |
|  |  | ARQ_FEEDBACK_POLL_RETRY_TIMEOUT | Bit[15:0] Refer to 16.2.13.3.9 |