| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Simultaneous Transmit and Receive (STR) Relay (16.6.1) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Youngbin Chang, Heewon Kang, Chanho Min, Seunghee Han, Youngsoo Kim Samsung Electronics Co., Ltd. | Email: yb.chang@samsung.com, hkang@samsung.com Phone: +82-31-279-5519 |
| Re: | IEEE 802.16-10/0011, IEEE 802.16 Working Group Letter Ballot Recirc #31 / Topic: Relay Format (Section 16.2.2.2) |  |
| Abstract | The contribution proposes the simultaneous transmit and receive (STR) relaying mode for 16m relay. |  |
| Purpose | To be discussed and adopted by TGm for P802.16m/D5. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Simultaneous Transmit and Receive (STR) Relay

Youngbin Chang, Chanho Min, Heewon Kang, Seunghee Han, Youngsoo Kim

Samsung Electronics Co., Ltd.

- Introduction

This contribution includes the proposed text for the simultaneous transmit and receive (STR) relaying mode for Relay in the IEEE 802.16m AWD.

- Motivation

In 802.16j-2009, STR relay was already defined. STR relay has the following benefits compared with TTR relay.

- Increase spectral efficiency: Re-utilizes ABS-ARS link resources as ARS-AMS link resources

- Reduced overhead: No need for switching gap(R-TTI and R-RTI), relay amble, and Broadcasting information for frame configuration.

- Larger uplink coverage: Long TTI can be applied to whole UL frame which is not divided by Relay zone and Access zone.

- Minimize PHY change of ABS

- All PHY frame is same as 16m ABS-AMS frame in both ABS-ARS link and ARS-AMS link

- All HARQ timing operation is same as 16m ABS-AMS frame in both ABS-ARS link and ARS-AMS link

- Additional MIMO midamble is not necessary for ABS-ARS link

- Uplink data sub-carrier mapping and control structure change for UL relay zone is not necessary

- Minimize PHY change of AMS

- Relay zone indication for AMS and timing gap information(R_Idle_time) in SCD does not necessary

- Text Proposal for STR relaying mode

-------------------------------------------------- Start of the proposed text --------------------------------------------------

[Add the following text after section 3.133 on page 6:]

3.xxx. Simultaneous transmit and receive (STR) relaying: a relay mechanism where transmission to subordinate station(s) and reception from the superordinate station, and transmission to the superordinate station and reception from subordinate station(s) are performed simultaneously.

[Modify the following text from line 11-15 on page 713:]

Relaying in Advance WirelessMAN-OFDMA System is performed using a decode and forward paradigm. Both TDD and FDD modes for Duplexing the DL and UL are supported. In TDD deployments ARSs operate in either TTR mode, whereby the access and relay link communications are multiplexed using time division multiplexing within a single RF carrier, or STR mode, whereby the access and relay link communications are performed simultaneously if the access and relay link are isolated enough. Relay mode is decided by ARS capability negotiation phase of ARS network entry.

[Add the following text at line 8 on page 732:]

ARS shall follow the basic 16m frame structure (figure 682), In STR relay, both the ABS-ARS and ARS-AMS frame structure is same as ABS-AMS frame structure defined in 16.3.3. In TTR relay, ABS-ARS frame and AMS frame shall be multiplexed in time separated. All the following description of frame structure is all about TTR relay.

-------------------------------------------------- End of the proposed text ---------------------------------------------------