| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Carrier switching operation for E-MBS receiving |  |
| Date Submitted | 2009-03-05 |  |
| Source(s) | NanLi Kaiying Lv BoSun FengXie ZTE Corporation | E-mail: lv.kaiying@zte.com.cn |
| Re: | LB31 / Chapter 16.9.2.2.( E-MBS operation in connected state) |  |
| Abstract | This contribution proposes texts on carrier switching operation for E-MBS receiving in connected state. |  |
| Purpose | To be discussed and adopted. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Carrier switching operation for E-MBS receiving

NanLi, Kaiying Lv, BoSun,FengXie

ZTE Corporation

# 1. Problem Statement

It has been specified in 802.16m/D4 that the ABS or AMS initiates the DSA procedure with respect to multicast and broadcast connections. An AMS could establish one or more E-MBS connections on multiple carriers. The AMS receives E-MBS data after decoding the AAI_E-MBS-CFG message and E-MBS MAP on relative carrier.

For a carrier-switching enabled AMS, if its subscribed E-MBS service flows are transmitted on carriers other than its primary carrier, it performs carrier switching to receive the E-MBS service flows. However, with current description in D4, the ABS could not acquire information about when the AMS will switch to the target carrier to receive its favorite EMBS service flows. Thus the ABS cannot efficiently schedule unicast services for the AMS on its primary carrier. For example, when the AMS is going to receive EMBS service flow(s), which are conflicting with the AMS’s current PA unicast services , the ABS needs an interval to re-schedule the PA resource for the AMS.

# 2. Text Proposal

======================== Start of Proposed Text =====================

[Change text in 16.9.2.2 E-MBS Operation in Connected State as follows:]

…

When EMBS data is transmitted on an alternative carrier, i.e. other than the AMS’s primary carrier where service flows are configured the AMS is redirected to relevant carrier through DSA as described in 16.9.2.1

During the transmission of EMBS configuration messages, the EMBS MAP and the EMBS data to which AMS is subscribed, the AMS with only one transceiver may not be available for signaling exchange with ABS on the primary carrier. The AMS sends an AAI_EMBS-REV-IND message to the ABS to inform the ABS which E-MBS service it intends to receive. When the AMS changes the concurrent receiving E-MBS service(s), it sends the message with an updated MSTIDs and Flow IDs list. The ABS schedules unicast services for the AMS on its primary carrier based on this message and E-MBS service burst allocation for the indicated E-MBS service flows. After receiving an AAI_EMBS-REV-IND message, the ABS transmits an AAI_EMBS-REV-RSP message in response. An Action Time may be included in the AAI_EMBS-REV-RSP message to indicate the time the ABS allows the AMS to start carrier switching.

The AMS with multiple transceivers may be able to receive EMBS data while communicating with ABS on primary carrier.

…

[Insert the following text to 16.2.3:]

16.2.3.x AAI_EMBS-REV-IND

The AMS sends an AAI_EMBS-REV-IND message to inform the ABS which E-MBS service the AMS intends to receive. Table XXX shows the parameters for AAI_EMBS-REV-IND message:

Table XXX AAI_EMBS-REV-IND message format

| Syntax | Size (Bits) | Notes |
| --- | --- | --- |
| AAI_EMBS-REV-IND_Message_Format () { |  |  |
| Management Message Type | 8 |  |
| N | 3 | Number of EMBS service(s) the AMS intends to receive |
| for(i==0,i<N,i++) |  |  |
| { |  |  |
| MSTID + Flow ID | 16 | EMBS connection information |
| } |  |  |
| } |  |  |

The following parameters may be included：

• Action time: This is used to indicate the time the AMS leaves the primary carrier.

16.2.3.y AAI_EMBS-REV-RSP

The AAI_EMBS-REV-RSP message shall be transmitted by the ABS in response to an AAI_EMBS-REV-IND message sent by the AMS.

Table XXX  Parameters for AAI_EMBS-REV-RSP message

| Name | Value | Usage |
| --- | --- | --- |
| Management Message Type | AAI_EMBS-REV-RSP message |  |
| Action code | 0b0：confirm AMS’s switching 0b1：refuse AMS to leave the primary carrier |  |
| Action Time | Indicates the absolute frame number the AMS could start carrier switching. | May be included when action code=0b0. |

============================== End of Proposed Text ===============