| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comment on Control Parameters for DL Multi-BS Joint Processing in IEEE P802.16m/D4 (16.5.1.3.1) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Kanchei(Ken) Loa, Chun-Che Chien, Youn-Tai Lee, Chun-Yen Hsu, Tsung-Yu Tsai, Yi-Ting Lin Institute for Information Industry (III) Yih Guang Jan, Tamkang University | E-mail: loa@iii.org.tw yihjan@yahoo.com |
| Re: | IEEE 802.16 Working Group Letter Ballot #31 P802.16m/D4 : 16.1.3.1 |  |
| Abstract | This contribution proposes the text change for DL Multi-BS MIMO. |  |
| Purpose | To be discussed and adopted by TGm for the IEEE P802.16m Draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comment on Control Parameters for DL Multi-BS Joint Processing

in IEEE P802.16m/D4 (16.5.1.3.1)

Kanchei(Ken) Loa, Chun-Che Chien, Youn-Tai Lee, Chun-Yen Hsu, Tsung-Yu Tsai, Yi-Ting Lin

Institute for Information Industry (III)

Yih Guang Jan, 
Tamkang University

# Introduction

In DL multi-BS joint MIMO processing, in addition to the neighboring ABS PMI feedback, quantized phase information for each neighboring cell is required to form a concatenating PMI (CPMI), which could improve performance based on coherent transmission. According to the new version of draft in [1], the CPMI feedback in AAI_MultiBS_MIMO_FBK is mandatory under CL-MD or Co-MIMO operation. Besides, the connection between CPMI value in AAI_MultiBS_MIMO_FBK and the quantized phase parameter defined in 16.5.1.3.1 is lost. In above considerations, we suggest the corresponding changes in [1].

# Reference

[1] IEEE P802.16m/D4, “Part 16: Air Interface for Broadband Wireless Access Systems: Advanced Air Interface,” Feb 2010.

- Text proposal for inclusion in the 802.16m Draft 4

----------------------------------------------------------------------------------------------------------------------------------------------------------------

[Modify the following text in Section 16.5.1.3.1]

------------------------------------------------------------------------- Text Start --------------------------------------------------------------------------

16.5.1 DL Multi-BS MIMO

16.5.1.3 DL Multi-BS Joint MIMO Processing

16.5.1.3.1 Operation procedure

With adaptive precoding, the precoder matrix W is derived from the feedback of the AMS, with codebook-based feedback or sounding-based feedback. Two types of adaptive precoding based multi-BS joint processing are supported, CL-MD and Co-MIMO. When CL-MD is enabled, a single AMS is served jointly by multiple coordinating ABSs. When Co-MIMO is enabled, several AMSs are served jointly by the multiple coordinating ABSs through MU-MIMO scheduling and precoding.

For codebook-based feedback, the AMS(s) choose the PMIs for the serving cell and the neighboring cells based on the respective estimated channel state information. Optionally, tThe serving ABS can should also instruct the AMS(s) to feedback a 3-bit uniformly quantized phase information for each neighboring cell, such that ABS can form a concatenating PMI based on the phase information for the neighboring cells to further improve the system performance. The equation for phase quantization is where b corresponds to a 3-bit CPMI value which is defined in Table 945:

Table 945—Quantization parameters for b

| Value CPMI value | b |
| --- | --- |
| 0 0b000 | 0 |
| 1 0b001 | 1/8 |
| 2 0b010 | 2/8 |
| 3 0b011 | 3/8 |
| 4 0b100 | 4/8 |
| 5 0b101 | 5/8 |
| 6 0b110 | 6/8 |
| 7 0b111 | 7/8 |

------------------------------------------------------------------------ Text End ---------------------------------------------------------------------------