| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on AAI_Global-Config AAI_MC-REQ/RSP Messages (16.2.3.51) |  |
| Date Submitted | 2010-03-17 |  |
| Source(s) | I-Kang Fu, Kelvin Chou, Yih-Shen Chen, Alex Hsu, Paul Cheng MediaTek Inc. | IK.Fu@mediatek.com |
| Re: | P802.16m/D4 |  |
| Abstract | It is not impossible that the network may be reconfigured after deployment because of operator’s decision. This should only happen occasionally in unit of days, weeks or months. In order to response this possibility, the standard text need to be clarified. |  |
| Purpose | For TGm discussion and adoption into P802.16m/D5 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on AAI_MC-REQ/RSP Messages

I-Kang Fu, Kelvin Chou, Yih-Shen Chen, Alex Hsu, Paul Cheng

MediaTek Inc.

- Introduction

It is possible that AMS can support all the carriers available by ABS, which may happen when the number of available carrier is not high (e.g. <4). The signaling overhead can be substantially reduced by single bit indication for this situation. Meanwhile, it is also possible that ABS assign all its available carriers to AMS. The logical carrier index can be set as the physical carrier index, since this can only happen when the number of available carrier is smaller than the max. number of logical carrier index according to the current design in P802.16m/D4.

- Text Proposal

---------------------------------------------------------Start of the Text-----------------------------------------------------------

[Adopt the following text modification started from line#1, page#146 in P802.16m/D4]

16.2.3.53 AAI_Global-Config (global carrier configuration) Message

The AAI_Global-Config message provides the carrier information for all available carriers in the network. The ABS transmits the AAI_Global-Config message to an AMS right after network entry completes.

Table 735 - AAI_Global- Config MAC Control Message Format

| Field | Size (bit) | Description |
| --- | --- | --- |
| MAC Control Message Type | 8 |  |
| Number of Carrier Groups | 4 | Groups of contiguous carriers |
| For (i=0; i< Number of Carrier Groups; i++){ |  |  |
| Duplexing Mode | 1 | “0” for TDD “1” for FDD |
| Multi-Carrier Configuration Index Across the Network | 6 | Index associated to Table 7790 |
| Start Frequency Assignment Index | 6 8 | Frequency Assignment Index of the first carrier in i-th carrier group #i. |
| Number of Carriers | 6 |  |
| For(j=0;j<Number of Carriers; j++) { |  |  |
| Physical Carrier Index | 6 | Index of the physical carrier |
| Duplexing Mode | 1 | “0” for TDD “1” for FDD |
| Reference Carrier Indicator | 1 | 0b0 : Non-reference carrier 0b1 : Reference carrier of current carrier group |
| } |  |  |
| } |  |  |
| Physical Carrier Index of current carrier | 6 | The carrier that broadcasts this message; the physical carrier index refers to AAI_Global-Config message |

----------------------------------------------------------End of the Text-----------------------------------------------------------