| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on AAI_MC-REQ/RSP Messages (16.2.3.51) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | I-Kang Fu, Kelvin Chou, Yih-Shen Chen, Alex Hsu, Paul Cheng MediaTek Inc. | IK.Fu@mediatek.com |
| Re: | P802.16m/D4 |  |
| Abstract | Improper design on AAI_MC-REQ/RSP message may result in unnecessary restriction on multi-carrier AMS implementation. This contribution investigates the typical multi-carrier transceiver architectures and suggests a simple message format to ensure different architectures can all be well supported. |  |
| Purpose | For TGm discussion and adoption into P802.16m/D5 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on AAI_MC-REQ/RSP Messages

I-Kang Fu, Kelvin Chou, Yih-Shen Chen, Alex Hsu, Paul Cheng

MediaTek Inc.

- Introduction

It is possible that AMS can support all the carriers available by ABS, which may happen when the number of available carrier is not high (e.g. <4). The signaling overhead can be substantially reduced by single bit indication for this situation. Meanwhile, it is also possible that ABS assign all its available carriers to AMS. The logical carrier index can be set as the physical carrier index, since this can only happen when the number of available carrier is smaller than the max. number of logical carrier index according to the current design in P802.16m/D4.

- Text Proposal

---------------------------------------------------------Start of the Text-----------------------------------------------------------

[Adopt the following text modification started from line#19, page#117 in P802.16m/D4]

16.2.3.51 AAI_MC-REQ (multicarrier Request) Message

The Multi-Carrier Request Message (AAI_MC-REQ) is sent by an AMS to an ABS to request the list of Assigned Carriers. The AMS cannot send the AAI_MC-REQ message until it receives the AAI_Global-Config and AAI_MC-ADV message from its serving ABS. According to the multi-carrier configuration supported by serving ABS, which is indicated in AAI_MC-ADV, the AMS shall determine the subset of carriers which it can simultaneously support under its hardware capability. Then AMS will send AAI_MC-REQ message to the serving ABS and include sets of physical carrier index to inform ABS this information.

Table 733 - AAI_MC-REQ message format

| Field | Size (bits) | Notes |
| --- | --- | --- |
| MAC Control Message Type |  |  |
| Global_Support | 1 | 1: AMS can simultaneously support all the available carriers |
| If (All_Carrier_Support==0) { |  |  |
| Number of Candidate Combinations (N) | 4 |  |
| For (i=0, i<N; i++) { |  |  |
| Number of Candidate Assigned Carrier (Nc) | 3 |  |
| For (j=0, j<Nc; j++) { |  |  |
| Physical Carrier Index | 6 | The carriers AMS can simultaneously support |
| } |  |  |
| } |  |  |
| } |  |  |
| Support of data transmission over guard sub-carrier | 1 | 0= not support 1= support |

16.2.3.52 AAI_MC-RSP (multicarrier Response) Message

Based on information provided by the AMS on the AAI_MC-REQ message, the ABS shall respond to the AMS through the AAI_MC-RSP message to provide the AMS with information about its assigned carriers. The following are the parameters shall be included in the AAI_MC-RSP message:

Table 734 - AAI_MC-RSP message format

| Field | Size (bits) | Notes |
| --- | --- | --- |
| MAC Control Message Type |  |  |
| Global_Assign | 1 | All available carriers are assigned to AMS, and logical carrier index will be equal to physical carrier index. |
| Number of Assigned Carriers (N) | 3 | Number of Assigned Secondary Carriers. |
| For (i=0; i<N; i++) { |  |  |
| Physical Carrier Index | 6 | The index refers to a Physical carrier index in AAI_MC-ADV message |
| Logical Carrier Index | 3 | The index associated to the assigned physical carrier index used in AAI_CM-CMD/IND |
| Support of data transmission over guard sub-carrier | 1 | 0b0= not support 0b1= support |
| } |  |  |

The AAI_MC-RSP (Multi-Carrier Response) Message is typically sent to the AMS in response to the AAI_MC-REQ message, but it may also be sent by the ABS to an AMS to update the list of assigned carriers in unsolicited manner.

----------------------------------------------------------End of the Text-----------------------------------------------------------