| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | ASN.1 coding for Sleep Mode Configuration MAC Management Messages (16.2.3.23) |  |
| Date Submitted | 2009-03-05 |  |
| Source(s) | Yih-Shen Chen, Kelvin Chou, I-Kang Fu MediaTek | E-mail: yihshen.chen@mediatek.com |
| Re: | Letter Ballot #31a on the Draft Amendment (IEEE P802.16m/D4) |  |
| Abstract | This contribution proposes the recommendations of ASN.1 coding for sleep mode configuration MAC management messages. |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- ASN.1 coding for Sleep Mode Configuration MAC Management Messages

- Yih-Shen Chen, Kelvin Chou, I-Kang Fu

- MediaTek

- Introduction

In this contribution, the ASN.1 encodings for sleep mode configuration are provided, which include AAI_SLP-REQ and AAI_SLP-RSP.

- Proposed text

--------------------------------------------- Start of proposed text #1 ------------------------------------------------

16.2.3.23 AAI_SLP-REQ

An AMS in Active Mode may use the AAI_SLP-REQ message with Request_Code = 0b01(i.e., Enter Sleep Mode) to request a permission to enter Sleep Mode. The AMS in Sleep Mode can change the sleep mode settings by transmitting AAI_SLP-REQ with Operation = 0b10 (i.e., Change Sleep Mode settings). The AMS in Sleep Mode can exit from Sleep Mode by transmitting AAI_SLP-REQ with Operation = 0b00 (i.e., Exit from Sleep Mode).

| Name | Value | Usage |
| --- | --- | --- |
| Operation | 0b00 : Exit from Sleep Mode 0b01 : Enter Sleep Mode 0b10 : Change Sleep Mode 0b11 : Switch Sleep Cycle setting | This indicates operation request type of AAI_SLP-REQ message |
| SCID | 0~15 | Sleep Cycle ID. This field appears when Operationis not 0b00. |
| Start Frame Number | 0~63 | Least significant 6 bits of Frame Number. This field appears when Operation is not 0b00 |
| TIMF | 0~1 | Traffic Indication Message Flag 0 : AAI_TRF-IND message is not sent for the AMS 1 : AAI_TRF-IND message is sent to the AMS during every Listening Window This field appears when Operationis 0b01 or 0b10. |
| LWEF | 0~1 | Listening window Extension Flag. This field appears when Operationis 0b01 or 0b10. |
| NSCF | 0b00~0b11 | Next sleep cycle indicator. 0b00 = Reset to Initial Sleep Cycle 0b01 = min (2 x previous sleep cycle, Final Sleep Cycle) 0b10 = Reset to another Initial Sleep Cycle value 0b11 = Reserved This field appears when Operationis 0b01 or 0b10. |
| Initial Sleep Cycle | 0~15 | This indicates an assigned duration for the Initial Sleep Cycle during which an AMS keeps sleep state in Sleep Mode (measured in frames). This field appears when Operationis 0b01 or 0b10. |
| Final Sleep Cycle | 0~1023 | This indicates assigned duration for the Final Sleep Cycle (measured in frames). This field appears when Operationis 0b01 or 0b10. |
| Listening Window | 0~63 | Assigned duration of AMS’s default Listening Window (measured in frames). This Listening_Window may be extended as long as there is UL/DL data traffic between AMS and ABS when Listening Window Extension is enabled. This field appears when Operationis 0b01 or 0b10. |
| Listening sub-frame bitmap | Each bit in the bitmap indicates: 1: AMS wakes up at the specific AAI subframe 0: AMS does not wake up at the specific AAI subframe | The bitmap indicates the sub-frames in each frame where the AMS needs to remain awake The size of the bitmap equals to the number of AAI subframes of a frame. Bit #0 is mapped to the first AAI subframe. This field appears when Operationis 0b01 or 0b10. |
| New Initial Sleep Cycle | 0~31 | When the current Sleep Cycle is reset, if this value is included, the current Sleep Cycle shall be reset to this value. Otherwise, the current Sleep Cycle may be reset to Initial Sleep Cycle or may be updated to min (2 x Previous Sleep Cycle, Final Sleep Cycle). This field appears when Operationis 0b01 or 0b10 and NSCF equals to 0b10. |
| T_AMS | 0~31 | This timer is for Listening Window Extension of AMS. This field appears when Operationis 0b01 or 0b10 and LWEFequals to 1. |

-- Sleep Mode Request MAC Control Message

AAI-SLP-REQ ::= SEQUENCE {

messageType               OCTET STRING (SIZE(1)),

-- identify the purpose of configuration request

opCode	                 ENUMERATED  {

exitFromSleepMode(0),

enterSleepMode(1),

changeSleepMode(2),

switchSleepCycleSetting (3)},

-- sleep cycle ID

scid				INTEGER (0..15) OPTIONAL,

-- the least significant 6 bits of Frame Number

startFrameNumber		INTEGER (0..63) OPTIONAL,

-- parameter set for sleep mode configuration request

sleepModeRequestInfo       SleepModeRequestInfo OPTIONAL

}

SleepModeRequestInfo ::= SEQUENCE {

-- traffic indication message flag

timf				BOOLEAN,

-- listening window extension flag

lwef				BOOLEAN,

-- new sleep cycle indicator

nscf				INTEGER (0..3), --0b00: reset to Initial

sleep cycle;

-- 0b01: min (2*previous

sleep cycle, final

sleep cycle)

-- 0b10: reset to another

initial sleep cycle

value

-- 0b11: reserved

-- initial sleep cycle (measured in frames)

initialSleepCycle		INTEGER (0..15),

-- final sleep cycle (measured in frames)

finalSleepCycle		INTEGER (0..1023),

-- default listen window (measured in frames)

listenWindow			INTEGER (0..63),

-- listening subframe bitmap

listeningSubframeBitmap	BIT STRING {

b0 (0),   -- Bitmap to signal which subframe

b1 (1),   -- the AMS is required to keep awake.

b2 (2),   -- If nth bit in the bitmap is set to 1,

b3 (3),   -- it signals that the AMS should

b4 (4),   -- remain awake in the nth subframe.

b5 (5),

b6 (6),

b7 (7),

b8 (8)} (SIZE(8)),

-- new initial sleep cycle (measured in frames)

newInitialSleepCycle	INTEGER (0..31) OPTIONAL,

-- timer for listening window extension (measured in frames)

tAms				INTEGER (0..31) OPTIONAL

}

Table 698—AAI-SLP-REQ conditions

| Attributes | Conditions |
| --- | --- |
| scid | Present when opCode is not “exitFromSleepMode”. |
| startFrameNumber | Present when opCode is not “exitFromSleepMode”. |
| sleepModeRequestInfo | Present when opCode is “enterSleepMode” or “changeSleepMode” |
| newInitialSleepCycle | Present when nscf equals to 0b10 |
| tAms | Present when lwef equals to 1 |

------------------------------------------------- End of proposed text #1--------------------------------------------------

--------------------------------------------- Start of proposed text #2 ------------------------------------------------

16.2.3.24 AAI_SLP-RSP

The AAI_SLP-RSP message shall be sent from ABS to an AMS in response to an AAI_SLP-REQ message. The ABS may send the AAI_SLP-RSP message in unsolicited manner with Response_Code = 0b00 (i.e., Request by ABS in Unsolicited manner).If the request sent by an AMS is rejected by an ABS, the AMS shall not retransmit the AAI_SLP-REQ message

before the duration, indicated by the REQ_duration in AAI_SLP-RSP with Response_Code = 0b10 (i.e., Rejection of AAI_SLP-REQ), expires.

| Name | Value | Usage |
| --- | --- | --- |
| Response_Code | 0b00 : Request by ABS in Unsolicited manner 0b01 : Approval of AAI_SLP-REQ 0b10 : Rejection of AAI_SLP-REQ 0b11 : Reserved | This indicates response type of AAI_SLP-RSP message. |
| Operation | 0b00 : Exit Sleep Mode 0b01 : Enter Sleep Mode 0b10 : Change Sleep Mode 0b11 : Switch Sleep Cycle setting | This indicates operation type of AAI_SLP-RSP message. This field appears when Response_Code is 0b00 or 0b01. |
| SCID | 0~15 | Sleep Cycle ID This field appears when Response_Code is 0b00 or 0b01 and Operation is not 0b00. |
| CQI_Operation | 0~2 | 0: the CQICH assigned to the AMS is kept 1: the CQICH is de-allocated at the frame specified by Start_Frame_Number 2: the CQICH is automatically de-allocated at the beginning of the sleep window whenever the CQICH is newly assigned to the AMS during the Listening Window. This field appears when Response_Code is 0b00 or 0b01 and Operation is not 0b00 |
| Start Frame Number | 0~63 | Least Significant 6 bits of Frame Number. This field appears when Response_Code is 0b00 or 0b01 and Operation is not 0b00 |
| TIMF | 0~1 | Traffic Indication Message Flag 0 : AAI_TRF-IND message is not sent for the AMS 1 : AAI_TRF-IND message is sent to the AMS during every Listening Window This field appears when Response_Code is 0b00 or 0b01 and Operation is 0b01 or 0b10. |
| LWEF | 0~1 | Listening window Extension Flag.. This field appears when Response_Code is 0b00 or 0b01 and Operation is 0b01 or 0b10. |
| NSCF | 0b00~0b11 | Next sleep cycle indicator. 0b00 = Reset to Initial Sleep Cycle 0b01 = min (2 x previous sleep cycle, Final Sleep Cycle) 0b10 = Reset to another Initial Sleep Cycle value 0b11 = Reserved This field appears when Response_Code is 0b00 or 0b01 and Operation is 0b01 or 0b10. |
| Initial Sleep Cycle | 0~15 | This indicates an assigned duration for the Initial Sleep Cycle during which an AMS keeps sleep state in Sleep Mode (measured in frames). This field appears when Response_Code is 0b00 or 0b01 and Operation is 0b01 or 0b10. |
| Final Sleep Cycle | 0~1023 | This indicates assigned duration for the Final Sleep Cycle (measured in frames). This field appears when Response_Code is 0b00 or 0b01 and Operation is 0b01 or 0b10. |
| Listening Window | 0~63 | Assigned duration of AMS’s default Listening Window (measured in frames). This Listening_Window may be extended as long as there is UL/DL data traffic between AMS and ABS when Listening Window Extension is enabled. This field appears when Response_Code is 0b00 or 0b01 and Operation is 0b01 or 0b10. |
| Listening sub-frame bitmap | Each bit in the bitmap indicates: 1: AMS wakes up at the specific AAI subframe 0: AMS does not wake up at the specific AAI subframe | The bitmap indicates the sub-frames in each frame where the AMS needs to remain awake The size of the bitmap equals to the number of AAI subframes of a frame. This field appears when Response_Code is 0b00 or 0b01 and Operation is 0b01 or 0b10. |
| SLPID | 0~1023 | This ID shall be unique within an ABS. The other AMS shall not be assigned the same ID and SLP Group ID while the AMS is still in sleep mode. This field appears when Response_Code is 0b00 or 0b01, Operation is 0b01 or 0b10 and TIMF equals to 1. |
| New Initial Sleep Cycle | 0~63 | When the current Sleep Cycle is reset, if this value is included, the current Sleep Cycle shall be reset to this value. Otherwise, the current Sleep Cycle may be reset to Initial Sleep Cycle or may be updated to min (2 x Previous Sleep Cycle, Final Sleep Cycle). This field appears when Response_Code is 0b00 or 0b01, Operation is 0b01 or 0b10 and NSCF equals to 0b10. |
| T_AMS | 0~31 | This timer is for Listening Window Extension of AMS. This field appears when Operation is 0b01 or 0b10 and LWEF equals to 1. |
| REQ_duration | 0~255 | Waiting value for the AAI_SLP-REQ message retransmission , which is the Least Significant 8 bits of Frame Number. This field appears when Response_Code is 0b10. |

-- Sleep Mode Response MAC Control Message

AAI-SLP-RSP ::= SEQUENCE {

messageType               OCTET STRING (SIZE(1)),

-- identify the purpose of configuration response

responseCode	                 ENUMERATED  {

unsolictedResponseByAbs(0),

requestApproval(1),

requestRejection(2),

reserved (3)},

-- operation code

opCode					ENUMERATED  {

exitSleepMode(0),

enterSleepMode(1),

changeSleepMode(2),

switchSleepCycleSetting (3)} OPTIONAL,

-- sleep cycle ID

scid				INTEGER (0..15) OPTIONAL,

-- CQI operation indication

cqiOperation			INTEGER (0..2) OPTIONAL, -- 0: the CQICH assigned

-- to the AMS is kept.

-- 1: the CQICH is de

-- allocated at the frame

-- specified by

-- startFrameNumber.

-- 2: the CQICH is

-- automatically

-- deallocated at the

-- beginning of the sleep

-- window whenever the

-- CQICH is newly

-- assigned to the AMS

-- during the Listening

-- Window.

-- the least significant 6 bits of Frame Number

startFrameNumber		INTEGER (0..63) OPTIONAL,

-- parameter set for sleep mode configuration response

sleepModeResponseInfo       SleepModeResponseInfo OPTIONAL,

-- waiting value for AAI_SLP-REQ message retransmission

reqDuration			INTEGER (0..255) OPTIONAL –- This value is the

-- least significant 8

-- bits of Frame Number.

}

SleepModeResponseInfo ::= SEQUENCE {

-- traffic indication message flag

timf				BOOLEAN,

-- listening window extension flag

lwef				BOOLEAN,

-- new sleep cycle indicator

nscf				INTEGER (0..3), --0b00: reset to Initial

sleep cycle;

-- 0b01: min (2*previous

sleep cycle, final

sleep cycle)

-- 0b10: reset to another

initial sleep cycle

value

-- 0b11: reserved

-- initial sleep cycle (measured in frames)

initialSleepCycle		INTEGER (0..15),

-- final sleep cycle (measured in frames)

finalSleepCycle		INTEGER (0..1023),

-- default listen window (measured in frames)

listenWindow			INTEGER (0..63),

-- listening subframe bitmap

listeningSubframeBitmap	BIT STRING {

b0 (0),   -- Bitmap to signal which subframe

b1 (1),   -- the AMS is required to keep awake.

b2 (2),   -- If nth bit in the bitmap is set to 1,

b3 (3),   -- it signals that the AMS should

b4 (4),   -- remain awake in the nth subframe.

b5 (5),

b6 (6),

b7 (7),

b8 (8)} (SIZE(8)),

-- SLPID

slpid				INTEGER (0..1023),

-- new initial sleep cycle (measured in frames)

newInitialSleepCycle	INTEGER (0..31) OPTIONAL,

-- timer for listening window extension (measured in frames)

tAms				INTEGER (0..31) OPTIONAL

}

Table 699—AAI-SLP-RSP conditions

| Attributes | Conditions |
| --- | --- |
| opCode | Present if responseCode is “unsolictedResponseByAbs” or “requestApproval”. |
| scid | Present when responseCode is “unsolictedResponseByAbs” or “requestApproval” and opCode is not “exitSleepMode”. |
| cqiOperation | Present when responseCode is “unsolictedResponseByAbs” or “requestApproval” and opCode is not “exitSleepMode”. |
| startFrameNumber | Present when responseCode is “unsolictedResponseByAbs” or “requestApproval” and opCode is not “exitSleepMode”. |
| sleepModeResponseInfo | Present when responseCode is “unsolictedResponseByAbs” or “requestApproval” and opCode is “enterSleepMode” or “changeSleepMode” |
| newInitialSleepCycle | Present when nscf equals to 0b01. |
| tAms | Present when lwef equals to 1. |
| reqDuration | Present when responseCode is “requestRejection”. |

------------------------------------------------- End of proposed text #2--------------------------------------------------