| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | ASN.1 coding for Traffic Indication MAC Management Messages (16.2.3.25) |  |
| Date Submitted | 2009-03-05 |  |
| Source(s) | Yih-Shen Chen, Kelvin Chou, I-Kang Fu MediaTek | E-mail: yihshen.chen@mediatek.com |
| Re: | Letter Ballot #31a on the Draft Amendment (IEEE P802.16m/D4) |  |
| Abstract | This contribution proposes the recommendations of ASN.1 coding for sleep mode configuration MAC management messages. |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- ASN.1 coding for Traffic Indication MAC Management Messages

- Yih-Shen Chen, Kelvin Chou, I-Kang Fu

- MediaTek

- Introduction

In this contribution, the ASN.1 encodings for traffic indication MAC management messages are provided, which include AAI_TRF-IND, AAI_TRF-IND-REQ and AAI_TRF-IND-RSP.

- Proposed text

--------------------------------------------- Start of proposed text #1 ------------------------------------------------

16.2.3.25 AAI_TRF-IND

This message, when present, is sent from the ABS to the AMSs. The message is sent in the first frame of AMS' Listening Window. An AMS, that has not been assigned a SLPID, shall ignore this message. The message indicates whether there is DL traffic for AMSs addressed by the AAI_TRF-IND message. There are two formats for the AAI_TRF-IND message, indicated by the FRMT field. When FRMT = 0, and if the AMS does not find its own SLPID-Group Indication bit-map or Traffic Indication bit-map, the AMS shall consider it as a negative indication and may go back to sleep. When FRMT = 1, and if the AMS does not find its own SLPID in the AAI_TRF-IND message, the AMS shall consider it as a negative indication and may go back to sleep.

| Name | Value | Usage |
| --- | --- | --- |
| FRMT | 0 : It indicates the SLPID bitmap-based traffic indication 1: It indicates the SLPID-based traffic indication | This indicates type of Traffic Indication in AAI_TRF-IND message. |
| SLPID Group Indication Bitmap | 32bits N-th bit of SLPID-Group Indication Bitmap [MSB corresponds to N = 0] is allocated to SLPID Group that includes AMS with SLPID values from N*32 to N*32+31 0: There is no traffic for any of the 32 AMSs that belong to the SLPID-Group 1: There is traffic for at least one AMS in SLPID-Group. | This field appears when FRMT is set to 0. It indicates the existence of each SLPID group |
| Traffic Indication Bitmap | Multiple of 32bits (i.e., 0 ~ 32*32bits) Each Traffic Indication bitmap comprises multiples of 32-bit long Traffic Indication units. A Traffic Indication unit for 32 SLPIDs is added to AAI_TRF-IND message whenever its SLPID Group is set to 1 32 bits of Traffic Indication Unit (starting from MSB) are allocated to AMS in the ascending order of their SLPID values: 0: Negative indication 1: Positive indication | This field appears when FRMT is set to 0. It indicates the traffic indication for 32 AMSs in each SLPID group |
| Num_of_SLPI Ds | 0~63 | This field appears when FRMT is set to 1. It indicates the number of SLPID included in AAI_TRF-IND message |
| SLPID | 0~1023 | This field appears when FRMT is set to 1. Each SLPID is used to indicate the positive traffic indication for an AMS |

-- Traffic Indication MAC Control Message

AAI-TRF-IND ::= SEQUENCE {

messageType               OCTET STRING (SIZE(1)),

-- identify the indication type

frmt	           ENUMERATED  {

mod0(0),  -- mod0: SLPID bitmap-based traffic

-- indication

mod1(1)}, -- mod1: SLPID-based traffic

-- indication

-- traffic indication for SLPID bitmap-based traffic indication

trfIndInfomod0	SEQUENCE (SIZE(1..32)) OF TrfIndInfoMod0	OPTIONAL,

-- traffic indication for SLPID-based traffic indication

trfIndInfomod1	TrfIndInfoMod1	OPTIONAL

}

TrfIndInfoMod0 ::= SEQUENCE {

slpidGroupIndicationBitmap	BOOLEAN,

trafficIndicationBitmap		BIT STRING {

b0 (0),   -- 0: Negative indication

b1 (1),   -- 1: Positive indication

b2 (2),

b3 (3),

b4 (4),

b5 (5),

b6 (6),

b7 (7),

b8 (8),

b9 (9),

b10 (10),

b11 (11),

b12 (12),

b13 (13),

b14 (14),

b15 (15),

b16 (16),

b17 (17),

b18 (18),

b19 (19),

b20 (20),

b21 (21),

b22 (22),

b23 (23),

b24 (24),

b25 (25),

b26 (26),

b27 (27),

b28 (28),

b29 (29),

b30 (30),

b31 (31)

} (SIZE(32))	OPTIONAL

}

TrfIndInfoMod1 ::= SEQUENCE {

numOfSlpids		INTEGER(0..63)

slpid			SEQUENCE (SIZE(1..numOfSlpids)) OF INTEGER(0..1023)

}

Table 700—AAI-TRF_IND conditions

| Attributes | Conditions |
| --- | --- |
| trfIndInfomod0 | Present when frmt equals to 0 |
| trfIndInfomod0 | Present when frmt equals to 1 |
| trafficIndicationBitmap | Present when slpidGroupIndicationBitmap equals to 1 |
| slpid | Present when numOfSlpids is larger than 0 |

--------------------------------------------- End of proposed text #1--------------------------------------------------

--------------------------------------------- Start of proposed text #2 ------------------------------------------------

16.2.3.26 AAI_TRF-IND-REQ

If the TIMF is set to 1, the AMS shall receive AAI_TRF-IND message in the first frame during its own Listening

Window. However, if the traffic indication message is lost or otherwise not detected by the AMS, the AMS shall stay awake for the rest of the Listening Window. If the AMS receives any unicast data during the listening window, then it shall assume that the traffic indication was positive. If the AMS receives neither the traffic indication message nor any unicast data in the Listening Window, the AMS shall send an AAI_TRF_IND-REQ message to the ABS in order to ask which kind of traffic indication (i.e., positive or negative traffic indication) the ABS sent to it. The ABS shall respond to the AMS by sending an AAI_TRF_IND-RSP with traffic indication for the AMS.

| Name | Value | Usage |
| --- | --- | --- |
| Frame_Number | 0-15 | This indicates the least significant 4 bits of frame number in which the AMS expected to receive the AAI_TRF-IND. |

-- Traffic Indication Request MAC Control Message

AAI-TRF-IND-REQ ::= SEQUENCE {

messageType               OCTET STRING (SIZE(1)),

-- identify the least 4 bits of the frame number in which the AMS expected to

-- receive AAI_TRF-IND

frameNumber	INTEGER (0..15)

}

------------------------------------------------- End of proposed text #2----------------------------------------------

--------------------------------------------- Start of proposed text #3 ------------------------------------------------

16.2.3.27 AAI_TRF-IND-RSP

When the ABS receives AAI_TRF_IND-REQ message from an AMS, the ABS shall respond to the AMS by sending AAI_TRF_IND-RSP message with the traffic indication for the AMS. When the AMS receives AAI_TRF_IND-RSP from the ABS, the AMS shall update the current Sleep Cycle based on the Traffic_Indication in the AAI_TRF_IND-RSP.

| Name | Value | Usage |
| --- | --- | --- |
| Frame_Number | 0-15 | Least Significant 4 bits of Frame number |
| Traffic_Indication | 0: Negative Traffic Indication 1: Positive Traffic Indication | It indicates the traffic indication which the ABS has transmitted to the AMS |

-- Traffic Indication Response MAC Control Message

AAI-TRF-IND-REQ ::= SEQUENCE {

messageType               OCTET STRING (SIZE(1)),

-- identify the least 4 bits of the frame number

frameNumber	INTEGER (0..15)

trafficIndication	BOOLEAN –- 0: Negative traffic indication

-- 1: Positive traffic indication

}

------------------------------------------------- End of proposed text #3--------------------------------------------------