| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | New AAI-UL-POWER-ADJ Message in 802.16m Systems in IEEE802.16 D4 (16.2.3) |  |
| Date Submitted | 2010-03-15 |  |
| Source(s) | Yu-Hao Chang, Yih-Shen Chen MediaTek Inc. Jeongho Park Samsung Wookbong Lee, Dongcheol Kim LG Electronics Zheng Yan-Xiu, Chang-Lan Tsai, Chung-Lien Ho, Yu-Chuan Fang ITRI Rongzhen Yang, Yi Hsuan Intel Corp. | E-mail : yuhao.chang@mediatek.com E-mail : jeongho.jh.park@samsung.com E-mail : wookbong.lee@lge.com, dongcheol.kim@lge.com |
| Re: | Comments on P802.16m/D4 |  |
| Abstract | This contribution provides the cleanup version for message AAI_UL_POWER_ADJUST. |  |
| Purpose | To be discussed and adopted into P802.16m/D4 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

New AAI-UL-POWER-ADJ Message in 802.16m Systems in IEEE802.16 D4 (16.2.3)

Yu-Hao Chang, Yih-Shen Chen

MediaTek Inc.

Jeongho Park

Samsung

Wookbong Lee, Dongcheol Kim

LG Electronics

Zheng Yan-Xiu, Chang-Lan Tsai, Chung-Lien Ho, Yu-Chuan Fang

ITRI

Rongzhen Yang, Yi Hsuan

Intel Corp.

- Text proposal

Instruction to editor;

Black text: existing D4 text

Red and strike out text: for deletion

Blue and underline text: for addition

[Remedy 1:]

[Change the text from line 55 on the page 118, section 16.2.3.31 as follows:]

============================== Start of Proposed Text I =================================

16.2.3.31 AAI_UL_POWER_ADJUST message

An AAI_UL_POWER_ADJUST message shall be transmitted by the ABS to control the transmit power level of AMS.

---ASN.1START

AAI_UL_POWER_ADJUST::= SEQUENCE {

OFFSET_DATA 		INTEGER (0..63),

OFFSET_CONTROL	INTEGER (0..63)

}

---ASN.1STOP

Table 70910—AAI_UL_POWER_ADJUST message Field Descriptions

| Parameters | Description |
| --- | --- |
| OFFSET_DATA | OFFSET_DATA is the transmit power adjustment value transmitted by the ABS. It has 6 bits to represent the value among -15.5 to 16 dB with 0.5 dB step |
| OFFSET_CONTROL | OFFSET_CONTROL is the transmit power adjustment value transmitted by the ABS. It has 6 bits to represent the value among -15.5 to 16 dB with 0.5 dB step; |

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | offsetData | 6 | offsetData is the transmit power adjustment value transmitted by the ABS. It represents the value among -15.5 to 16 dB with 0.5 dB step | NA |
| M | offsetControl | 6 | offsetControl is the transmit power adjustment value transmitted by the ABS. It represents the value among -15.5 to 16 dB with 0.5 dB step | NA |
| O | channelIndex | 8 | Uniquely identifies a fast feedback channel on which an AMS can transmit fast feedback information. With this allocation, a one-to-one relationship is established between channel index and the AMS | Present when ABS want to change FF Channel Period by using AAI-POWER-ADJ message. (More details can be found in 16.3.6.5.2.4.5.) |
|  | shortTermFeedbackPeriod | 3 | Short-term feedback is transmitted on the FBCH every 2p frames, where p is the value specified by shortTermFeedbackPeriod |  |
|  | longTermFeedbackPeriod | 2 | Long-term feedback is transmitted on the FBCH every 2q short-term feedback reports are transmitted, where q is the value specified by longTermFeedbackPeriod. If q = 0b00, long-term feedback is not used |  |
|  | frameOffsetNumber | 2 | The AMS starts reporting at the frame of which the number has the same 3 LBS as the specified frame number. If the current frame is specified, the AMS starts reporting in eight frames |  |
|  | subframeIndex | 3 | Indicates the UL AAI subframe index in the UL portion of the frame |  |
|  | allocationDuration | 3 | An FBCH is transmitted on the FBCH channels indexed by Channel Index for 8×2d frames, where d is the value given by allocationDuration. If d = 0b000, the FBCH is deallocated. If d = 0b111, the AM reports until the ABS command for the AMS to stop |  |

============================== End of Proposed Text I=================================

[Remedy 2:]

Annex P.

(Normative)

Definition of AAI MAC control messages

Annex P.2 MAC Control Message Definitions

------------------------------------------------- Start of proposed text II --------------------------------------------------

[Add the following ASN.1 code to Annex P.2]

-- Uplink Power Adjust Command Message

AAI-UL-POWER-ADJ ::= SEQUENCE {

messageType                    OCTET STRING (SIZE(1)),

offsetData                     INTEGER (0..63),

offsetControl                  INTEGER (0..63),

feedbackControlInfoIE	  FeedbackControlInfo Optional

}

FeedbackControlInfo ::= SEQUENCE{

channelIndex               		INTEGER (0..63),

shortTermFeedbackPeriod              	INTEGER (0..7),

longTermFeedbackPeriod               	INTEGER (0..3),

frameOffsetNumber                    INTEGER (0..3),

subframeIndex                        	INTEGER (0..7),

allocationDuration                         	INTEGER (0..7)

}

------------------------------------------------- End of proposed text II --------------------------------------------------