| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | ASN.1 Encoding for Ranging Messages (16.2.3) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Kelvin Chou, Yih-Shen Chen, I-Kang Fu and Paul Cheng MediaTek Inc. | Kelvin.Chou@mediatek.com |
| Re: | Letter Ballot #31a on the Draft Amendment (IEEE P802.16m/D4) |  |
| Abstract | Text proposal on the format of ranging messages in ASN.1 |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

ASN.1 Encoding for Ranging Messages (16.2.3)

Kelvin Chou, Yih-Shen Chen, I-Kang Fu and Paul Cheng

MediaTek Inc.

Introduction

This contribution proposes the ASN.1 encoding formats for the following messages:

- AAI_RNG-REQ

- AAI_RNG-RSP

- AAI_RNG-ACK

Proposed Text

Remedy 1:

In the P802.16m/D4, page 63, line 45, replace Table 676 to the following table and ASN.1 encoding format:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 676 — AAI_RNG-REQ parameters

| M/O | Attributes / Array of attributes |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- |
| O | Paging Controller ID |  | 48 | The Paging Controller ID which the AMS currently maintains in idle mode. | It shall be included when the AMS in Idle Mode is attempting to perform reentry or location update |
| O | Deregistration Identifier (DID) |  | 10 | The ID which the AMS is assigned for idle mode and currently maintains. |  |
| O | PGID |  | 16 | The identification of the paging group that the AMS is previously belonging to. |  |
| O | Paging Cycle |  | 4 | PAGING_CYCLE applied to the AMS |  |
| O | Paging Offset |  | 4 | PAGING_OFFSET applied to the AMS |  |
| O | Paging Cycle Change |  | 4 | PAGING_CYCLE requested by the AMS | It shall be included when the AMS in Idle Mode is attempting to change Paging Cycle during Location Update |
| O | Power Down Indicator |  | 1 | Indicates the AMS is currently attempting to perform location update due to power down. | It shall be included when the AMS in Idle Mode is attempting to perform location update due to power down |
| O | AMS Mobility Information |  | 2 | 0b00 = Slow 0b01 = Medium 0b10 = Fast 0b11 = Reserved | It may be included when the AMS in Idle Mode is attempting to perform the location update |
| O | CRID |  | TBD | AMS Identifier which the AMS has been assigned for coverage loss or DCR mode and are currently maintained | It shall be included when the AMS is attempting to perform network reentry from coverage loss or DCR mode |
| O | CSGID Info [0..15] | Number of CSGID | 4 | Indicates the number of CSGID included in CSG white list 0..15 | Present when AMS with a CSG white list performs network entry to aid CSG membership detection and ABS reselection |
|  |  | CSGID | 10 | One or more CSG(s) |  |
| O | Previous CID |  | 12 | The CID which the AMS used in the previous serving BS | It shall be included when the AMS is attempting to perform Direct HO reentry |
| O | STID |  | 12 | The STID which the AMS uses in the previous serving ABS. | It shall be included when the AMS is attempting to perform HO reentry |
| O | NONCE_AMS |  | 64 | A freshly generated 64-bit random number used for PMK derivation | It shall be included when zone switching from LZone to MZone occurs. |
| O | NONCE_ABS |  | 64 | A 64-bit number transferred from ABS and used for PMK derivation | It shall be included when zone switching from LZone to MZone occurs. |
| O | AK_COUNT |  | 8 | The AMS’s current value of the AK_COUNT, which is used to generate the security keys in the target ABS. | It shall be included during reentry, secure Location Update or HO |
| O | CMAC Tuple |  | 92 | If included, the CMAC Tuple shall be the last attribute in the message. | It shall be included when the AMS is attempting to perform Network Reentry from idle mode, Secure Location Update, or HO, or a reentry after incurring a coverage loss if the AMS has a CMAC tuple necessary to expedite security authentication. |
| O | LEGACY_CMAC_KEY_COUNT |  |  | The AMS's current value of the CMAC_KEY_COUNT, which was used at the previous serving Legacy BS. | It shall be included during Direct HO from a 16e only Legacy BS to the target ABS. |
| O | Legacy CMAC Tuple |  |  | If included, the CMAC Tuple shall be the last attribute in the message. | It shall be included when the AMS is attempting to perform Network Re-Entry from Direct HO, if the AMS has a CMAC tuple necessary to expedite security authentication. This CMAC is the CMAC tuple used in the previous serving Legacy BS. |
| O | AMSID* |  | 48 | AMSID* is the hash value of AMSID in order to protect AMS privacy, which is used for ABS to distinguish AMSs when more than one AMS send AAI_RNG-REQ message at the same time | It shall be included when the AMS is attempting network entry without its STID/DID which the ABS/Paging Controller assigns |
| O | MAC version |  | 8 | MAC version is version number of IEEE 802.16 supported by the AMS. |  |

-- ASN1START -- Ranging request MAC control message AAI-RNG-REQ ::= SEQUENCE { messageType OCTET STRING (SIZE(1)), } -- ASN1STOP

------------------------------------------------------  Text End  ---------------------------------------------------------------

Remedy 2:

In the P802.16m/D4, page 66, line 42, replace Table 677 to the follow table and ASN.1 encoding format:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 677 — AAI_RNG-RSP parameters

| M/O | Attributes / Array of attributes |  |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- | --- |
| O | Location Update Response |  |  | 8 | 0x00= Success of Location Update 0x01= Failure of Location Update 0x02 = Reserved 0x03=Success of location update and DL traffic pending 0x04 = Allow AMS’s DCR mode initiation request 0x06~0xFF: Reserved | It shall be included when an ABS sends an AAI_RNG-RSP message in response to an AAI_ RNG-REQ message used to perform location update or DCR mode initiation from Idle Mode or DCR mode extension. |
| O | Paging Cycle |  |  | 4 | New Paging Cycle of the AMS 0b0000 – 0b1111 | It shall be included only if the Location Update Response is set to 0x00 (Success of Idle Mode Location Update) and the Paging Information has changed |
| O | Paging Offset |  |  | 4 | New PAGING_OFFSET of AMS |  |
| O | Paging Group ID |  |  | 16 | New PGID of the AMS |  |
| O | Paging Controller ID |  |  | 48 | The new Paging Controller ID which the AMS shall maintain in idle mode. |  |
| O | Deregistration Identifier (DID) |  |  | 10 | The new DID which the AMS shall maintain in idle mode. |  |
| O | Unsolicited bandwidth indicator |  |  | 1 | Indicates that ABS should allocate UL Bandwidth for transmission or retransmission of MAC message without request from AMS during network entry | It shall be included when AMS is attempting network entry |
| O | CLC-INFO | Information Type |  | 2 | 0x00 : CLC Limit 0x01 : CLC Request 0x10 : CLC Response 0x11 : CLC Report | It shall be included when AMS is attempting network reentry after HO if the AMS has any Type I or II CLC class active before HO. The information Type field of CLC INFO shall be set to 2 to indicate it provides CLC response information |
|  |  | CLC Response | Confirmed Action | 8 | Table 696 |  |
|  |  |  | Start Time parameters | 8n [Note : n is the number of CLC class] |  |  |
| O | Redirection Info [0..56] | BSID |  | 48 | BS Identifier | It should be included when serving ABS aids cell reselection in the event AMS is not allowed to perform network entry with the current serving ABS |
|  |  | Preamble Index |  | 10 |  |  |
|  |  | Center Frequency |  |  |  |  |
| O | CRID |  |  | TBD | AMS Identifier which the AMS has been assigned for coverage loss or DCR mode and are currently maintained | It shall be included when the AMS is attempting to perform network reentry from coverage loss or DCR mode |
| O | Ranging Request bit |  |  | 1 | Indicates that AMS should perform periodic ranging and send AAI_RNG-CFM to serving ABS | It shall be included when serving ABS performs coverage loss detection procedure (16.2.26) |
| M | Ranging Status |  |  | 2 | Ranging status is used to indicate whether UL messages are received within acceptable limits by ABS. 0b01 = continue, 0b10 = abort, 0b11 = success | NA |
| O | Temporary STID |  |  | 12 | Temporary STID is used for AMS identification until STID is assigned to the AMS during registration procedure. | It shall be included in the AAI_RNG-RSP message in response to the AAI_RNG-REQ message, which is not CMAC protected, when the AMS is not assigned its STID/DID yet. |
| O | AMSID* |  |  | 48 | AMSID* is used when the AMS confirms if the AAI_RNG-RSP is a response to the AAI_RNG-REQ message which the AMS sent. |  |
| O | HO Process Optimization |  |  | TBD | Bitmap definition is the same as in 16.2.3.11. Identifies reentry process MAC control messages that may be omitted during the current HO attempt due to the availability of MS service and operational context information obtained by means that are beyond the scope of this standard, and the MS service and operational status post-HO completion. The AMS shall not enter normal operation with target ABS until completing receiving all network reentry, MAC control message responses as indicated in HO process optimization. | It shall be included when the AMS is attempting to perform network reentry or HO and the target ABS wishes to identify reentry process MAC control messages that may be omitted during the current HO attempt |
| O | NONCE_AMS |  |  | 64 | A freshly generated 64-bit random number used for PMK derivation | It shall be included when zone switching from LZone to MZone occurs. |
| O | NONCE_ABS |  |  | 64 | A 64-bit number transferred from ABS and used for PMK derivation | It shall be included when zone switching from LZone to MZone occurs. |
| O | Number of carriers (Redirection Info) |  |  | 4 | It indicates the number of carriers of neighbor ABSs which is included in the AAI_RNG-RSP message. | It shall be included when ranging status is abort. It is sent by serving ABS to aid cell reselection in the event the serving ABS is not able to allow the AMS to perform entry (due to various reasons such as high serving ABS load, non existence of CSG membership etc) |
| O | ABSID (Redirection Info) |  |  | 48 | ID of ABS | It shall be included when ranging status is abort and included for each neighbor ABS. |
| O | SA-Preamble index (Redirection Info) |  |  | 10 | Index of SA-Preamble |  |
| O | Physical carrier index (Redirection Info) |  |  | 6 | Index of the physical carrier |  |

-- ASN1START -- Ranging response MAC control message AAI-RNG-RSP ::= SEQUENCE { messageType OCTET STRING (SIZE(1)), } -- ASN1STOP

------------------------------------------------------  Text End  ---------------------------------------------------------------

Remedy 3:

In the P802.16m/D4, page 68, line 59, replace Table 678 to the follow table and ASN.1 encoding format:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

16.2.3.3 AAI_RNG-ACK

Table 676 — AAI_RNG-ACK parameters

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Management Message Type | 8 | AAI_RNG-ACK | NA |
| M | Unicast indication | 1 | 0b0: AAI_RNG-ACK message is sent in broadcast manner. 0b1: AAI_RNG-ACK message is sent in unicast manner. | NA |
| M | Frame identifier | 4 | Frame identifier identifies the frame which contains the ranging opportunities to which this message refers. The frame identifier is produced by concatenating the following two values: 1.The 2 least significant bits of the superframe number 2.The frame index within the superframe (ranging from 0b00 to 0b11). | NA |
| O | RNG-ACK Bitmap | 4 | The size of this bitmap is decided by the number of ranging slots in the referenced frame. Each bit indicates the decoding status of the corresponding ranging opportunity. 0b0: No ranging code is detected; 0b1: At least one code is detected. | It shall be included when unicast indication is 0b1. |
| O | Number of received codes | 5 | The number of ranging code indices detected in this corresponding ranging opportunity. | It shall be included for each bit=0b1 in the RNG-ACK Bitmap. |
| O | Code Index | 7 | Code index received in this ranging opportunity. | Shall be included for each received ranging codes if Number of received codes > 0 |
| O | Ranging status | 2 | Ranging status is used to indicate whether ranging codes are received within acceptable limits by ABS. 0b01 = continue, 0b10 = abort, 0b11 = success | If unicast indication is 0b1, it shall be included for each received ranging codes. Otherwise, it shall be included for the specific AMS. |
| O | Adjustment parameters indication (API) | 3 | Bit#0: Time offset adjustment indication. Bit#1: Power level adjustment indication Bit#2: Frequency offset adjustment indication | If unicast indication is 0b1, it shall be included for each received ranging codes when ranging status is continue or success. Otherwise, it shall be included for the specific AMS when ranging status is continue or success. |
| O | Timing offset adjustment | 10 | Amount of advance time required to adjust AMS transmission. | It shall be included when Time offset adjustment indication bit is 1. |
| O | Power level adjustment | 3 | Relative change in transmission power level. | It shall be included when Power level adjustment indication bit is 1. |
| O | Frequency offset adjustment | 8 | Relative change in transmission frequency. | It shall be included when Frequency offset adjustment indication bit is 1. |

-- ASN1START -- Ranging acknowledgement MAC control message AAI-RNG-ACK ::= SEQUENCE { messageType OCTET STRING (SIZE(1)), -- False: for broadcast, True: for unicast unicastIndication BOOLEAN, -- Identifies the frame which contains the ranging opportunities to which this message refers frameIdentifier FrameIdentifier, rangingAck CHOICE { unicastRngAck UnicastRngAck, -- when unicastIndication is True broadcastRngAck BroadcastRngAck -- when unicastIndication is False } } FrameIdentifier ::= SEQUENCE { -- The 2 least significant bits of the superframe number superFrameNumber INTEGER (0..3), -- The frame index within the superframe frameIndex INTEGER (0..3) } UnicastRngAck ::= SEQUENCE { rangingStatus RangingStatus, adjustmentParameters AdjustmentParameters OPTIONAL } RangingStatus ::= ENUMERATED { continue (0), abort (1), success (2) } AdjustmentParameters ::= SEQUENCE { adjustmentParamIndication BIT STRING { b0 (0), -- Time offset adjustment indication b1 (1), -- Power level adjustment indication b2 (2) -- Frequency offset adjustment indication }, timeingOffsetAdjustment INTEGER (0..1023) OPTIONAL, powerLevelAdjustment INTEGER (0..7) OPTIONAL, frequencyOffsetAdjustment INTEGER (0..255) OPTIONAL } BroadcastRngAck ::= SEQUENCE { -- Size of the rngAckBitmap rngAckBitmapSize INTEGER (0..15), -- Each bit indicates the decoding status of the corresponding ranging opportunity -- 0b0: No ranging code is detected -- 0b1: At least one code is detected rngAckBitmap BIT STRING (SIZE(rngAckBitmapSize)), -- Number of 0b1's in the rngAckBitmap numberOfDetectedSlots INTEGER (0..15), detectedSlotsList SEQUENCE (SIZE (1..numberOfDetectedSlots)) OF DetectedSlot } DetectedSlot ::= SEQUENCE { numberOfReceivedCodes INTEGER (0..31), receivedCodesList SEQUENCE (SIZE (1..numberOfReceivedCodes)) OF ReceivedCode } ReceivedCode ::= SEQUENCE { codeIndex INTEGER (0..127), unicastRngAck UnicastRngAck } -- ASN1STOP

------------------------------------------------------  Text End  ---------------------------------------------------------------