| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | ASN.1 Encoding for Ranging Messages (16.2.3) |  |
| Date Submitted | 2010-03-14 |  |
| Source(s) | Kelvin Chou, Yih-Shen Chen, I-Kang Fu and Paul Cheng MediaTek Inc. | Kelvin.Chou@mediatek.com |
| Re: | Letter Ballot #31a on the Draft Amendment (IEEE P802.16m/D4) |  |
| Abstract | Text proposal on the format of ranging messages in ASN.1 |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

ASN.1 Encoding for Ranging Messages (16.2.3)

Kelvin Chou, Yih-Shen Chen, I-Kang Fu and Paul Cheng

MediaTek Inc.

Introduction

This contribution proposes the ASN.1 encoding formats for the following messages:

- AAI_RNG-REQ

- AAI_RNG-RSP

- AAI_RNG-ACK

The original parameters for these messages have been restructured to fit into the ASN.1 coding structure. The presented ASN.1 codes have been syntax checked using the ANS.1 compiler (asn1c).

Proposed Text

Remedy 1:

In the P802.16m/D4, page 63, line 45, replace Table 676 to the following table and ASN.1 encoding format:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 676 — AAI_RNG-REQ parameters

| M/O | Attributes / Array of attributes | Value / Note | Conditions |
| --- | --- | --- | --- |
| AAI-RNG-REQ |  |  |  |
| O | rngPurposeIndication | The presence of this item in the message indicates the following AMS action: If Bit#0 is set to 1, it indicates that the AMS is currently attempting HO reentry, or, in combination with a Paging Controller ID, indicates that the MS is attempting network reentry from idle mode to the BS. In this case, Bit#1 shall be 0. If Bit#1 is set to 1, it indicates that the AMS is initiating the idle mode location update process, or, in combination with CRID, it indicates that the AMS is initiating DCR mode extension. In this case, Bit#0 shall be 0. If Bit#2 is set to 1, ranging request for emergency call setup. When this bit is set to 1, it indicates AMS action of Emergency Call process. If Bit#3 is set to 1, it indicates that the AMS is attempting to perform location update due to a need to update service flow management encodings for E-MBS flows. If Bit #4 is set to 1, it indicates that the AMS is initiating location update for transmission to DCR mode from idle mode. If Bit #5 is set to 1 in combination with ID of the network entity that assigns/retains the context, it indicates that the AMS is currently attempting reentry from DCR mode. If Bit#6 is set to 1, it indicates that the AMS is currently attempting network reentry after experiencing a coverage loss. If Bit#7 is set to 1, it indicates that the AMS is currently attempting network reentry from a IEEE802.16e only Legacy BS Bit#8: Reserved | It shall be included when the AMS is attempting to perform reentry, HO, location update or DCR mode extension |
| O | csgIDList | A list of CSGIDs. | Present when AMS with a CSG white list performs network entry to aid CSG membership detection and ABS reselection |
| M | rngInfo | In case of HO reentry, idle mode reentry, location update or DCR extension, nwReentryInfo is presented. See NWReentryInfo. In case of Zone Switch from LZone to MZone, zoneSwitchInfo in presented. See ZoneSwitchInfo. In case of Direct HO, directHOInfo is presented. See DirectHOInfo. | N/A |
| NWReentryInfo |  |  |  |
| O | servingBSID | The BSID of the AMS’s previous serving ABS before incurring a coverage loss, or the BSID of the serving ABS to which the AMS is currently connected (has completed the registration cycle and is in normal operation). Inclusion of serving BSID in the AAI_RNG-REQ message signals to the target ABS that the AMS is currently connected to the network through the serving ABS and is in the process of HO network reentry. | Shall be included when the AMS is attempting to perform HO reentry. |
| O | crID | AMS Identifier which the AMS has been assigned for coverage loss or DCR mode and are currently maintained | Shall be included when the AMS is attempting to perform network reentry from coverage loss or DCR mode |
| O | stID | The STID which the AMS uses in the previous serving ABS. | Shall be included when the AMS is attempting to perform HO reentry |
| O | akCount | The AMS’s current value of the AK_COUNT, which is used to generate the security keys in the target ABS. | Shall be included during reentry, secure Location Update or HO |
| O | amsIDStar | AMSID* is the hash value of AMSID in order to protect AMS privacy, which is used for ABS to distinguish AMSs when more than one AMS send AAI_RNG-REQ message at the same time | Shall be included when the AMS is attempting network entry without its STID/DID which the ABS/Paging Controller assigns |
| O | macVersion | MAC version is version number of IEEE 802.16 supported by the AMS. | Shall be included when the AMS is attempting network entry without its STID/DID which the ABS/Paging Controller assigns |
| O | pagingInfo | See PagingInfo | Shall be included when the AMS in Idle Mode is attempting to perform reentry or location update |
| O | pagingCycleChange | Paging cycle requested by the AMS | Shall be included when the AMS in Idle Mode is attempting to change Paging Cycle during Location Update |
| O | powerDownIndicator | Indicates the AMS is currently attempting to perform location update due to power down. | Shall be included when the AMS in Idle Mode is attempting to perform location update due to power down |
| O | amsMobilityInfo | 0b00 = Slow 0b01 = Medium 0b10 = Fast 0b11 = Reserved | May be included when the AMS in Idle Mode is attempting to perform the location update |
| O | cmacTuple | If included, the CMAC Tuple shall be the last attribute in the message. | Shall be included when the AMS is attempting to perform Network Reentry from idle mode, Secure Location Update, or HO, or a reentry after incurring a coverage loss if the AMS has a CMAC tuple necessary to expedite security authentication. |
| PagingInfo |  |  |  |
| M | pagingControllerID | The Paging Controller ID which the AMS currently maintains in idle mode. | N/A |
| M | deregistrationID | The ID which the AMS is assigned for idle mode and currently maintains | N/A |
| M | pagingGroupID | The identification of the paging group that the AMS is previously belonging to. | N/A |
| M | pagingCycle | Paging cycle applied to the AMS | N/A |
| M | pagingOffset | Paging offset applied to the AMS | N/A |
| ZoneSwitchInfo |  |  |  |
| M | nonceAMS | A freshly generated 64-bit random number used for PMK derivation | N/A |
| M | nonceABS | A 64-bit number transferred from ABS and used for PMK derivation | N/A |
| DirectHOInfo |  |  |  |
| M | servingBSID | The BSID of the previous serving Legacy BS. | N/A |
| M | legacyCID | The CID which the AMS used in the previous serving BS | N/A |
| M | legacyCMACKeyCount | The AMS's current value of the CMAC_KEY_COUNT, which was used at the previous serving Legacy BS. | N/A |
| M | legacyCMACTuple | If included, the CMAC Tuple shall be the last attribute in the message. | N/A |

--ASN1START

-- Ranging request MAC control message

AAI-RNG-REQ ::= SEQUENCE {

rngPurposeIndication		BIT STRING {

reentry-ho-idle(0),

location-update-dcr-ext(1),

emergency-call-setup(2),

location-update-embs(3),

location-update-dcr(4),

reentry-dcr(5),

reentry-coverage-loss(6),

reentry-legacy(7) }				OPTIONAL,

csgIDList					SEQUENCE (SIZE (1..maxCSGIDs)) OF CSGID					OPTIONAL,

rngInfo						CHOICE {

nwReentryInfo	NWReentryInfo,

zoneSwitchInfo	ZoneSwitchInfo,

directHOInfo	DirectHOInfo

}

}

CSGID ::= 						BIT STRING (SIZE (10))

NWReentryInfo ::= SEQUENCE {

servingBSID					BIT STRING (SIZE (48))				OPTIONAL, -- HO reentry

crID						BIT STRING (SIZE (16))				OPTIONAL, -- reentry from CR or DCR

stationID					StationID							OPTIONAL, -- HO reentry

akCount						AKCount								OPTIONAL, -- reentry, locateion update, HO

amsIDStar					AMSIDStar							OPTIONAL,

macVersion					MACVersion							OPTIONAL,

pagingInfo					PagingInfo							OPTIONAL, -- reentry from idle

pagingCycleChange			INTEGER (0..15)						OPTIONAL,

powerDownIndicator			BOOLEAN								OPTIONAL,

amsMobilityInfo				ENUMERATED {slow, medium, fast}		OPTIONAL,

cmacTuple					CMACTuple							OPTIONAL

}

PagingInfo ::= SEQUENCE {

pagingControllerID			BIT STRING (SIZE (48)),

deregistrationID			BIT STRING (SIZE (10)),

pagingGroupID				BIT STRING (SIZE (16)),

pagingCycle					INTEGER (0..15),

pagingOffset				INTEGER (0..15)

}

ZoneSwitchInfo ::= SEQUENCE {

nonceAMS					BIT STRING (SIZE (64)),

nonceABS					BIT STRING (SIZE (64))

}

DirectHOInfo ::= SEQUENCE {

servingBSID					BIT STRING (SIZE (48)),

legacyCID					INTEGER(0..4095),

legacyCMACKeyCount			INTEGER(0..65535),

legacyCMACTuple				BIT STRING (SIZE (104))

}

--ASN1STOP

------------------------------------------------------  Text End  ---------------------------------------------------------------

Remedy 2:

In the P802.16m/D4, page 66, line 42, replace Table 677 to the follow table and ASN.1 encoding format:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 677 — AAI_RNG-RSP parameters

| M/O | Attributes / Array of attributes | Value / Note | Conditions |
| --- | --- | --- | --- |
| AAI-RNG-RSP |  |  |  |
| M | rangingStatus | Ranging status is used to indicate whether UL messages are received within acceptable limits by ABS. 0b01 = continue, 0b10 = abort, 0b11 = success | N/A |
| O | stID | stID is the temporary STID for AMS identification when the AMS is not assigned a STID/DID yet; otherwise stID is the AMS’s STID | Temporary STID is presented in the AAI_RNG-RSP message in response to the AAI_RNG-REQ message, which is not CMAC protected, when the AMS is not assigned its STID/DID yet. STID is presented in the AAI_RNG-RSP message during uncontrolled HO, NW reentry or Zone switching in case that the AAI_RNG-RSP is encrypted |
| O | amsIDStar | AMSID* is used when the AMS confirms if the AAI_RNG-RSP is a response to the AAI_RNG-REQ message which the AMS sent. | Presented in the AAI_RNG-RSP message in response to the AAI_RNG-REQ message, which is not CMAC protected, when the AMS is not assigned its STID/DID yet. |
| O | locationUpdateResponse | Enumeration of the following values: Success of Location Update(0), Failure of Location Update(1), Success of location update and DL traffic pending(2), Allow AMS’s DCR mode initiation request(3) | Presented when an ABS sends an AAI_RNG-RSP message in response to an AAI_ RNG-REQ message used to perform location update or DCR mode initiation from Idle Mode or DCR mode extension. |
| O | pagingInfo | Paging information. See PagingInfo. | Presented only if locationUpdateResponse is 0x00 (Success of Idle Mode Location Update) and the Paging Information has changed |
| O | hoProcessOptimization | For each bit location, a value of 0 indicates the associated reentry MAC control messages shall be required, a value of 1 indicates the reentry MAC control message may be omitted. Bit #0: Omit AAI_SBC-REQ/RSP MAC control messages during reentry processing Bit #1: Omit PKM Authentication phase Bit #2: Omit AAI_REG-REQ/RSP message and higher layer protocol triggering (for IP address refresh) during reentry processing. Bit #3: If Bit #3 = 1, Full service and operational state transfer or sharing between Serving BS and Target BS (All static and dynamic context, e.g., ARQ windows, state machines | Presented when the AMS is attempting to perform network reentry or HO and the target ABS wishes to identify reentry process MAC control messages that may be omitted during the current HO attempt |
| O | nbrMeasurementReport | Identifies Neighbor station measurement report is required during current network entry for ARS | Presented when network entry to an ARS |
| O | zoneSwitchInfo | See ZoneSwitchInfo | Presented when zone switching from LZone to MZone occurs. |
| O | redirectionInfoList | List of neighbor ABS information. See ABSInfo | Presented when serving ABS aids cell reselection in the event AMS is not allowed to perform network entry with the current serving ABS |
| O | unsolicitedBWIndicator | Indicates that ABS should allocate UL Bandwidth for transmission or retransmission of MAC message without request from AMS during network entry | Presented when AMS is attempting network entry |
| O | clcInfo | See CLCInfo | Presented when AMS is attempting network reentry after HO if the AMS has any Type I or II CLC class active before HO. The information Type field of CLC INFO shall be set to 2 to indicate it provides CLC response information |
| O | CRID | AMS Identifier which the AMS has been assigned for coverage loss or DCR mode and are currently maintained | Presented when the AMS is attempting to perform network reentry from coverage loss or DCR mode |
| O | rangingRequestBit | Indicates that AMS should perform periodic ranging and send AAI_RNG-CFM to serving ABS | Presented when serving ABS performs coverage loss detection procedure (16.2.26) |
| ABSInfo |  |  |  |
| M | absID | ABS ID | N/A |
| M | carrierInfoList | List of carrier information. See CarrierInfo | N/A |
| CarrierInfo |  |  |  |
| M | saPreambleIndex |  | N/A |
| M | aPreambleTxPower |  | N/A |
| M | physicalCarrierIndex | Refer to the physical carrier index in AAI_Global-Config message | N/A |
| M | macProtocolVersion | MAC protocol version of the BS Consistent with REV.2 definition, with new MAC protocol version 9 defined for 16m. | N/A |
| CLCInfo |  |  |  |
| M | infoType | CLC limit, CLC request, CLC | M |

-- ASN1START

-- Ranging response MAC control message

AAI-RNG-RSP ::= SEQUENCE {

rangingStatus				RangingStatus,

stID						CHOICE {

temporarySTID	TemporarySTID,

stationID		StationID

}									OPTIONAL,

amsIDStar					AMSIDStar							OPTIONAL,

locationUpdateResponse		LocationUpdateResponse				OPTIONAL,

pagingInfo					PagingInfo							OPTIONAL,

hoProcessOptimization		BIT STRING (SIZE (4))				OPTIONAL,

nbrMeasurementReport		BOOLEAN								OPTIONAL,

zoneSwitchInfo				ZoneSwitchInfo						OPTIONAL,

redirectionInfoList			SEQUENCE (SIZE(1..maxRedirectInfos)) OF ABSInfo	OPTIONAL,

unsolicitedBWIndicator		BOOLEAN								OPTIONAL,

clcInfo						CLCInfo								OPTIONAL,

crID						BIT STRING (SIZE (16))				OPTIONAL,

rangingRequestBit			BOOLEAN								OPTIONAL

}

LocationUpdateResponse ::= ENUMERATED {

success,

failed,

success-DL-traffic-pending,

dcr-mode-initiation-allowed }

ABSInfo ::= SEQUENCE {

absID						ABSID,

carrierInfoList				SEQUENCE (SIZE(1..maxCarriers)) OF CarrierInfo

}

CarrierInfo ::= SEQUENCE {

saPreambleIndex				INTEGER(0..767),

aPreambleTxPower			INTEGER(0..255),

physicalCarrierIndex		PhysicalCarrierIndex,

macProtocolVersion			INTEGER(0..255)

}

CLCInfo ::= SEQUENCE {

infoType					ENUMERATED {

clc-limit,

clc-request,

clc-response,

clc-report }

--	response					CHOICE {

----- content TBD ----------

--								}

}

-- ASN1STOP

------------------------------------------------------  Text End  ---------------------------------------------------------------

Remedy 3:

In the P802.16m/D4, page 68, line 59, replace Table 678 to the follow table and ASN.1 encoding format:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

16.2.3.3 AAI_RNG-ACK

Table 676 — AAI_RNG-ACK parameters

| M/O | Attributes / Array of attributes | Value / Note | Conditions |
| --- | --- | --- | --- |
| AAI-RNG-ACK |  |  |  |
| M | frameIdentifier | Frame identifier identifies the frame which contains the ranging opportunities to which this message refers. See FrameIdentifier | N/A |
| M | rangingAck | Choice of the following structures: - UnicastRngAck when this message is sent in unicast manner. See UnicastRngAck; - BroadcastRngAck when this message is sent in broadcast manner. See BroadcastRngAck | N/A |
| FrameIdentifier |  |  |  |
| M | superFrameNumber | The 2 least significant bits of the superframe number | N/A |
| M | frameIndex | The frame index within the superframe | N/A |
| UnicastRngAck |  |  |  |
| M | rangingStatus | Ranging status is used to indicate whether ranging codes are received within acceptable limits by ABS. Value: continue(0)¦abort (1)¦success(2) | N/A |
| O | adjustmentParameters | See AdjustmentParameters | Presented when rangingStatus is continue or success |
| AdjustmentParameters |  |  |  |
| O | timeingOffsetAdjustment | Amount of advance time required to adjust AMS transmission. | Presented when timing adjustment is required for the AMS |
| O | powerLevelAdjustment | Relative change in transmission power level. | Presented when power level adjustment is required for the AMS |
| O | frequencyOffsetAdjustment | Relative change in transmission frequency. | Presented when frequency adjustment is required for the AMS. |
| BroadcastRngAck |  |  |  |
| M | rngAckBitmap | Each bit indicates the decoding status of the corresponding ranging opportunity 0b0: No ranging code is detected 0b1: At least one code is detected | N/A |
| O | detectedSlotsList | A list of DetectedSlot. See DetectedSlot | Presented when at least one bit in the rngAckBitmap is 1 |
| DetectedSlot |  |  |  |
| M | receivedCodesList | A list of ReceivedCode of size numberOfReceivedCodes | N/A |
| ReceivedCode |  |  |  |
| M | codeIndex | Code index received in this ranging opportunity. | N/A |
| M | unicastRngAck | See UnicastRngAck | N/A |

-- ASN1START

-- Ranging acknowledgement MAC control message

AAI-RNG-ACK ::= SEQUENCE {

frameIdentifier				FrameIdentifier,

rangingAck					CHOICE {

unicastRngAck	UnicastRngAck,

broadcastRngAck	BroadcastRngAck

}

}

FrameIdentifier ::= SEQUENCE {

superFrameNumber			INTEGER (0..3),

frameIndex					INTEGER (0..3)

}

UnicastRngAck ::= SEQUENCE {

rangingStatus				RangingStatus,

adjustmentParameters		AdjustmentParameters		OPTIONAL

}

RangingStatus ::= ENUMERATED {	continue(0), abort(1), success(2) }

AdjustmentParameters ::= SEQUENCE {

timeingOffsetAdjustment		INTEGER (0..1023)			OPTIONAL,

powerLevelAdjustment		INTEGER (0..7)				OPTIONAL,

frequencyOffsetAdjustment	INTEGER (0..255)			OPTIONAL

}

BroadcastRngAck ::=	SEQUENCE {

rngAckBitmap				BIT STRING,

detectedSlotsList			SEQUENCE OF DetectedSlot	OPTIONAL

}

DetectedSlot ::= SEQUENCE {

receivedCodesList			SEQUENCE (SIZE(1..maxReceivedCodes)) OF ReceivedCode

}

ReceivedCode ::= SEQUENCE {

codeIndex					INTEGER (0..127),

unicastRngAck				UnicastRngAck

}

-- ASN1STOP

------------------------------------------------------  Text End  --------------------------------------------------------------

Appendix

A complete version of ASN.1 code for the proposed messages in this contribution is provided. The code has been successfully compiled using open ASN.1 compiler (asn1c).

MacControlMsg DEFINITIONS AUTOMATIC TAGS ::= BEGIN

-- MAC Control Messages

MAC-Control-Message ::= SEQUENCE {

message MAC-Control-Msg-Type

}

MAC-Control-Msg-Type ::= CHOICE {

aaiRngReq			AAI-RNG-REQ,

aaiRngRsp			AAI-RNG-RSP,

aaiRngAck			AAI-RNG-ACK,

aaiNbrAdv			AAI-NBR-ADV,

aaiHoInd			AAI-HO-IND,

aaiHoReq			AAI-HO-REQ,

aaiHoCmd			AAI-HO-CMD,

aaiMcReq			AAI-MC-REQ,

aaiMcRsp			AAI-MC-RSP,

aaiGlobalConfig		AAI-Global-Config

}

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

AKCount	::= 		INTEGER (0..7)

CMACTuple ::= 		BIT STRING (SIZE (92))

ABSID ::=			BIT STRING (SIZE (48))

AMSIDStar ::=		BIT STRING (SIZE (48))

MACVersion ::=		BIT STRING (SIZE (8))

StationID ::=   	BIT STRING (SIZE (12))

TemporarySTID ::=	BIT STRING (SIZE (12))

PhysicalCarrierIndex ::= INTEGER (0..63)

-------------------------------------------------------------------------------

------------------------ AAI-HO-IND ------------------------------------------

-------------------------------------------------------------------------------

-- HO indication MAC control message

AAI-HO-IND ::= SEQUENCE {

hoEvent			CHOICE {

targetABSSelection		TargetABSSelection,

targetABSUnreachable	TargetABSUnreachable,

servingABSDisconnected	NULL,

hoCancel				HandoverCancel

}

}

TargetABSSelection ::= SEQUENCE {

targetABSID				INTEGER (0..7),

physicalCarrierIndex	PhysicalCarrierIndex		OPTIONAL

}

TargetABSUnreachable ::= SEQUENCE {

targetABSID				INTEGER (0..7)				OPTIONAL,

physicalCarrierIndex	PhysicalCarrierIndex		OPTIONAL

}

HandoverCancel ::= SEQUENCE {

akCount					AKCount

}

-------------------------------------------------------------------------------

------------------------ AAI-HO-REQ ------------------------------------------

-------------------------------------------------------------------------------

-- HO request MAC control message

AAI-HO-REQ ::= SEQUENCE {

reportMetrix					BIT STRING {

cinr-Mean(0),

rssi-Mean(1),

relative-Delay(2)

},

neighborReportList				SEQUENCE (SIZE(1..maxNewABSIndices)) OF NeighborReport,

carrierPreassignmentIndicator	BOOLEAN							OPTIONAL

}

NeighborReport ::= SEQUENCE {

absIndex						INTEGER(0..255),

cinrMean						INTEGER(0..255)					OPTIONAL,

rssiMean						INTEGER(0..255)					OPTIONAL,

relativeDelay					INTEGER(0..255)					OPTIONAL

}

-------------------------------------------------------------------------------

------------------------ AAI-HO-CMD ------------------------------------------

-------------------------------------------------------------------------------

-- HO command MAC control message

AAI-HO-CMD ::= SEQUENCE {

mode		CHOICE {

hoCmd 		HandoverCommand,

zsCmd		ZoneSwitchCommand,

hoReject	NULL

}

}

HandoverCommand ::= SEQUENCE {

hoReentryMode					BOOLEAN,

hoReentryInterleavingInterval	INTEGER(0..255)					OPTIONAL,

hoReentryIteration				INTEGER(0..255)					OPTIONAL,

disconnectTimeOffset			INTEGER(0..255),

resourceRetainTime				INTEGER(0..255),

targetBSList					SEQUENCE (SIZE(1..maxNewABSIndices)) OF TargetBSInfo,

preallocatedBasicCID			INTEGER(0..65535)				OPTIONAL

}

ZoneSwitchCommand ::= SEQUENCE {

hoReentryMode					BOOLEAN,

resourceRetainTime				INTEGER(0..255),

actionTime						INTEGER(0..3),

preambleIndex-LZone				INTEGER(0..113),

cdmaRngFlag						BOOLEAN

}

TargetBSInfo ::= SEQUENCE {

targetBSID						ABSID,

preambleIndex					INTEGER(0..255),

faIndex							INTEGER(0..255),

s-sfhChangeCount				INTEGER(0..255),

actionTime						INTEGER(0..3),

cdmaRngFlag						BOOLEAN,

dedicatedCDMARngCode			INTEGER(0..31)					OPTIONAL,

dedicatedRngOpp					DedicatedRngOpp					OPTIONAL,

hoProcessOptimization			BIT STRING (SIZE (4)),

seamlessHO						BOOLEAN,

rngInitiationDeadline			INTEGER(0..31),

serviceLevelPrediction			ServiceLevelPrediction,

physicalCarrierIndex			INTEGER(0..63)					OPTIONAL,

preassignedCarrierInfo			PreassignedCarrierInfo			OPTIONAL

}

DedicatedRngOpp ::= SEQUENCE {

rngOppIndex						INTEGER(0..7),

subframeIndex					INTEGER(0..7)

}

ServiceLevelPrediction ::= ENUMERATED {

noServiceAvailable(0),

partiallyAvailable(1),

fullyAvailable(2),

noPrediction(3) }

PreassignedCarrierInfo ::= SEQUENCE {

carrierStatusBitmap				BIT STRING (SIZE(maxPreassignedCarriers)),

physicalCarrierIndexList		SEQUENCE (SIZE(1..maxPreassignedCarriers)) OF PhysicalCarrierIndex

}

-------------------------------------------------------------------------------

------------------------ AAI-RNG-REQ ------------------------------------------

-------------------------------------------------------------------------------

-- Ranging request MAC control message

AAI-RNG-REQ ::= SEQUENCE {

rngPurposeIndication		BIT STRING {

reentry-ho-idle(0),

location-update-dcr-ext(1),

emergency-call-setup(2),

location-update-embs(3),

location-update-dcr(4),

reentry-dcr(5),

reentry-coverage-loss(6),

reentry-legacy(7) }				OPTIONAL,

csgIDList					SEQUENCE (SIZE (1..maxCSGIDs)) OF CSGID					OPTIONAL,

rngInfo						CHOICE {

nwReentryInfo	NWReentryInfo,

zoneSwitchInfo	ZoneSwitchInfo,

directHOInfo	DirectHOInfo

}

}

CSGID ::= 						BIT STRING (SIZE (10))

NWReentryInfo ::= SEQUENCE {

servingBSID					BIT STRING (SIZE (48))				OPTIONAL, -- HO reentry

crID						BIT STRING (SIZE (16))				OPTIONAL, -- reentry from CR or DCR

stationID					StationID							OPTIONAL, -- HO reentry

akCount						AKCount								OPTIONAL, -- reentry, locateion update, HO

amsIDStar					AMSIDStar							OPTIONAL,

macVersion					MACVersion							OPTIONAL,

pagingInfo					PagingInfo							OPTIONAL, -- reentry from idle

pagingCycleChange			INTEGER (0..15)						OPTIONAL,

powerDownIndicator			BOOLEAN								OPTIONAL,

amsMobilityInfo				ENUMERATED {slow, medium, fast}		OPTIONAL,

cmacTuple					CMACTuple							OPTIONAL

}

PagingInfo ::= SEQUENCE {

pagingControllerID			BIT STRING (SIZE (48)),

deregistrationID			BIT STRING (SIZE (10)),

pagingGroupID				BIT STRING (SIZE (16)),

pagingCycle					INTEGER (0..15),

pagingOffset				INTEGER (0..15)

}

ZoneSwitchInfo ::= SEQUENCE {

nonceAMS					BIT STRING (SIZE (64)),

nonceABS					BIT STRING (SIZE (64))

}

DirectHOInfo ::= SEQUENCE {

servingBSID					BIT STRING (SIZE (48)),

legacyCID					INTEGER(0..4095),

legacyCMACKeyCount			INTEGER(0..65535),

legacyCMACTuple				BIT STRING (SIZE (104))

}

-------------------------------------------------------------------------------

------------------------ AAI-RNG-RSP ------------------------------------------

-------------------------------------------------------------------------------

-- Ranging response MAC control message

AAI-RNG-RSP ::= SEQUENCE {

rangingStatus				RangingStatus,

stID						CHOICE {

temporarySTID	TemporarySTID,

stationID		StationID

}									OPTIONAL,

amsIDStar					AMSIDStar							OPTIONAL,

locationUpdateResponse		LocationUpdateResponse				OPTIONAL,

pagingInfo					PagingInfo							OPTIONAL,

hoProcessOptimization		BIT STRING (SIZE (4))				OPTIONAL,

nbrMeasurementReport		BOOLEAN								OPTIONAL,

zoneSwitchInfo				ZoneSwitchInfo						OPTIONAL,

redirectionInfoList			SEQUENCE (SIZE(1..maxRedirectInfos)) OF ABSInfo	OPTIONAL,

unsolicitedBWIndicator		BOOLEAN								OPTIONAL,

clcInfo						CLCInfo								OPTIONAL,

crID						BIT STRING (SIZE (16))				OPTIONAL,

rangingRequestBit			BOOLEAN								OPTIONAL

}

LocationUpdateResponse ::= ENUMERATED {

success,

failed,

success-DL-traffic-pending,

dcr-mode-initiation-allowed }

ABSInfo ::= SEQUENCE {

absID						ABSID,

carrierInfoList				SEQUENCE (SIZE(1..maxCarriers)) OF CarrierInfo

}

CarrierInfo ::= SEQUENCE {

saPreambleIndex				INTEGER(0..767),

aPreambleTxPower			INTEGER(0..255),

physicalCarrierIndex		PhysicalCarrierIndex,

macProtocolVersion			INTEGER(0..255)

}

CLCInfo ::= SEQUENCE {

infoType					ENUMERATED {

clc-limit,

clc-request,

clc-response,

clc-report }

--	response					CHOICE {

----- content TBD ----------

--								}

}

-------------------------------------------------------------------------------

------------------------ AAI-RNG-ACK ------------------------------------------

-------------------------------------------------------------------------------

-- Ranging acknowledgement MAC control message

AAI-RNG-ACK ::= SEQUENCE {

frameIdentifier				FrameIdentifier,

rangingAck					CHOICE {

unicastRngAck	UnicastRngAck,

broadcastRngAck	BroadcastRngAck

}

}

FrameIdentifier ::= SEQUENCE {

superFrameNumber			INTEGER (0..3),

frameIndex					INTEGER (0..3)

}

UnicastRngAck ::= SEQUENCE {

rangingStatus				RangingStatus,

adjustmentParameters		AdjustmentParameters		OPTIONAL

}

RangingStatus ::= ENUMERATED {	continue(0), abort(1), success(2) }

AdjustmentParameters ::= SEQUENCE {

timeingOffsetAdjustment		INTEGER (0..1023)			OPTIONAL,

powerLevelAdjustment		INTEGER (0..7)				OPTIONAL,

frequencyOffsetAdjustment	INTEGER (0..255)			OPTIONAL

}

BroadcastRngAck ::=	SEQUENCE {

rngAckBitmap				BIT STRING,

detectedSlotsList			SEQUENCE OF DetectedSlot	OPTIONAL

}

DetectedSlot ::= SEQUENCE {

receivedCodesList			SEQUENCE (SIZE(1..maxReceivedCodes)) OF ReceivedCode

}

ReceivedCode ::= SEQUENCE {

codeIndex					INTEGER (0..127),

unicastRngAck				UnicastRngAck

}

-------------------------------------------------------------------------------

------------------------ AAI-NBR-ADV ------------------------------------------

-------------------------------------------------------------------------------

-- Neighbor advertisement MAC control message

AAI-NBR-ADV ::= SEQUENCE {

changeCount					INTEGER (0..7),

cellType					ENUMERATED {macro, micro, macro-hotzone, femto, relay},

totalNumberOfSegments		INTEGER (0..15),

segmentIndex				INTEGER (0..15),

startingABSIndex			INTEGER (0..255),

neighborInfoList			SEQUENCE (SIZE(1..maxNeighborBSs)) OF NeighborInfo

}

NeighborInfo ::= SEQUENCE {

absInfo						ABSInfo,

sfhEncodingFormat			CHOICE {

fullSubpktInfo	FullSubpktInfo,

deltaEncoding	DeltaEncoding,

noSFHIncluded	NULL }

}

FullSubpktInfo ::= SEQUENCE {

sfh-subpkt1					BIT STRING (SIZE (88))				OPTIONAL,

sfh-subpkt2					BIT STRING (SIZE (88))				OPTIONAL,

sfh-subpkt3					BIT STRING (SIZE (88))				OPTIONAL

}

DeltaEncoding ::= SEQUENCE {

deltaInfo					BIT STRING

}

-------------------------------------------------------------------------------

------------------------ AAI-MC-REQ -------------------------------------------

-------------------------------------------------------------------------------

-- Multicarrier request MAC control message

AAI-MC-REQ ::= SEQUENCE {

multiCarrierCapability				MultiCarrierCapability

}

MultiCarrierCapability ::= SEQUENCE {

candidateCombinaitonList			SEQUENCE (SIZE(1..maxCandidateCombinations)) OF CandidateCombinaiton,

supportOfDataTxOnGuardSubcarrier	BOOLEAN

}

CandidateCombinaiton ::= SEQUENCE {

physicalCarrierIndexList			SEQUENCE (SIZE(1..maxCandidateAssignedCarriers)) OF PhysicalCarrierIndex

}

-------------------------------------------------------------------------------

------------------------ AAI-MC-RSP -------------------------------------------

-------------------------------------------------------------------------------

-- Multicarrier response MAC control message

AAI-MC-RSP ::= SEQUENCE {

multiCarrierCapability				MultiCarrierCapability

}

-------------------------------------------------------------------------------

------------------------ AAI-Global-Config-------------------------------------

-------------------------------------------------------------------------------

-- Multicarrier global configuration MAC control message

AAI-Global-Config ::= SEQUENCE {

carrierGroupList					SEQUENCE (SIZE(1..maxCarrierGroups)) OF CarrierGroup,

currentPhysicalCarrierIndex			PhysicalCarrierIndex

}

CarrierGroup ::= SEQUENCE {

networkWiseMCConfigIndex			INTEGER(0..31),

startFreqAssignmentIndex			INTEGER(0..31),

carrierList							SEQUENCE (SIZE(1..maxCarriers)) OF Carrier

}

Carrier ::= SEQUENCE {

physicalCarrierIndex				PhysicalCarrierIndex,

duplexingMode						ENUMERATED {tdd(0), fdd(1)}

}

-------------------------------------------------------------------------------

--------------------  Multiplicity and type constraint definitions ------------

-------------------------------------------------------------------------------

maxNewABSIndices				INTEGER ::= 64

maxPreassignedCarriers			INTEGER ::= 64

maxCSGIDs						INTEGER ::= 64

maxRedirectInfos				INTEGER ::= 64

maxReceivedCodes				INTEGER ::= 32

maxNeighborBSs					INTEGER ::= 256

maxCandidateCombinations		INTEGER ::= 16

maxCandidateAssignedCarriers	INTEGER ::= 8

maxCarrierGroups				INTEGER ::= 16

maxCarriers						INTEGER ::= 32

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

END