| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | ASN.1 Encoding for Handover Messages (16.2.3) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Kelvin Chou, Yih-Shen Chen, I-Kang Fu and Paul Cheng MediaTek Inc. | Kelvin.Chou@mediatek.com |
| Re: | Letter Ballot #31a on the Draft Amendment (IEEE P802.16m/D4) |  |
| Abstract | Text proposal on the format of handover messages in ASN.1 |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

ASN.1 Encoding for Handover Messages (16.2.3)

Kelvin Chou, Yih-Shen Chen, I-Kang Fu and Paul Cheng

MediaTek Inc.

Introduction

This contribution proposes the ASN.1 encoding formats for the following messages:

- AAI_HO-CMD

- AAI_HO-REQ

- AAI_HO-IND

Proposed Text

Remedy 1:

In the P802.16m/D4, page 79, line 51, replace Table 684 to the follow table and ASN.1 encoding format:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Control Message Type | 8 | AAI_HO-CMD | NA |
| M | Mode | 2 | 0b00: HO command; 0b01: Zone switch command from MZone to LZone; 0b10: AMS HO request rejected (ABS in list unavailable). In this case, AAI_HO-CMD message shall not include any target ABS. |  |
| O | HO Reentry Mode | 1 | 1: the AMS maintains communication with Serving ABS while performing network reentry with the Target ABS (Mode=0b00); or the AMS maintains communication with MZone while performing network reentry with LZone in the same frame or on another carrier (Mode=0b01). 0: the AMS disconnects from the Serving ABS before performing network reentry with the Target ABS (Mode=0b00); or the AMS disconnects from the MZone before performing network reentry with the LZone (Mode=0b01). | Should be included when Mode = 0b00. Shall be included when Mode = 0b01. |
| O | HO Reentry Interleaving Interval | 8 | If HO Reentry Interleaving Interval > 0, the AMS performs network reentry to the target ABS within the HO Reentry Interleaving Interval and continues data transmission with the Serving ABS in the remaining time. If HO Reentry Interleaving Interval =0, the AMS performs multi-carrier EBB (Established Before Break) HO procedure per 16.2.8.2.9.2.2 | Shall be included when HO Reentry Mode is set to 1 |
| O | HO Reentry Iteration | 8 | The requested number of iterating HO Reentry Intervals by an AMS. | Shall be included when HO Reentry Interleaving Interval > 0 |
| O | Disconnect time offset | 8 | Difference between Disconnect time and Action time in units of frames. The value of disconnect time shall be calculated for each T-ABS by adding/ subtracting this value with the value of Action time specified for this T-ABS. For HO_Reentry_Mode = 0, Disconnect time will be (Action time - Disconnect time offset); For HO_Reentry_Mode = 1, Disconnect time will be (Action time + Disconnect time offset). | Should be included when Mode = 0b00. |
| O | Resource_Retain_Time | 8 | Indicates the time when the Serving ABS will discard AMS’s context upon expiration of this timer. [*Note : Description was missing] | Should be included when Mode = 0b00. Shall be included when Mode = 0b01. |
| O | Target ABS/YBS ID | [8][48] | The BS ID of the target BS [*Note : Description was missing] | Shall be included for each target ABS when Mode=0b00 |
| O | Preamble Index | 8 | Preamble Index of Target ABS | Shall be included for each target ABS when Mode=0b00 |
| O | FA index | 8 | FA Index of Target ABS | Shall be included for each target ABS when Mode=0b00 |
| O | S-SFH change count | 8 | The change count of this BS's S-SFH IE | Shall be included for each target ABS when Mode=0b00 |
| O | Action Time | 2 | (Mode=0b00) Action Time included in this message is the absolute frame number at the serving ABS (Mode=0b01) This value is defined as the frame number that AMS starts zone switch. Action Time included in this message is indicated by frame number | Shall be included for each target ABS when Mode=0b00. Shall be included when Mode = 0b01. |
| O | CDMA_RNG_FLAG | 1 | Indicates whether CDMA ranging may be omitted or not. 0: The AMS may omit ranging 1: The AMS shall perform ranging [*Note : Description was missing] | If CDMA_RNG_FLAG=0, CDMA based ranging shall be skipped. Otherwise, if set to 1, it indicates that ranging shall be done. This parameter shall be included for each target ABS when Mode=0b00.; It shall be included when Mode = 0b01. |
| O | Dedicated Ranging Code Flag | 1 | Indicates whether a dedicated ranging code is assigned or not 0: No dedicated ranging code assigned 1: Dedicated ranging code is assigned [*Note : Description was missing] | If set to 0, it indicates that no dedicated ranging code is provided. If set to 1, it indicates that no dedicated ranging code is provided. May be included when CDMA_RNG_FLAG=1 |
| O | Dedicated Ranging Opportunity Flag | 1 | Indicates whether a dedicated ranging opportunity is assigned or not 0: No dedicated ranging opportunity assigned 1: Dedicated ranging opportunity is assigned [*Note : Description was missing] | If set to 0, it indicates that no dedicated ranging opprotunity is provided. If set to 1, it indicates that no dedicated ranging opportunity is provided. May be included when CDMA_RNG_FLAG=1 |
| O | Dedicated CDMA ranging code | 5 | The assigned dedicated ranging code | Indicates the dedicated ranging code. Shall be included when Dedicated Ranging Code Flag =1 |
| O | Ranging opportunity index | 3 | The assigned dedicated ranging opportunity index | Indicates the index of the allocated ranging opportunity. Shall be included when Dedicated Opportunity Code Flag=1 |
| O | Subframe Index | 3 | Indicates the subframe index where the assigned dedicated ranging opportunity is allocated | Shall be included when Dedicated Opportunity Code Flag=1 |
| O | HO process optimization | 4 | For each bit location, a value of 0 indicates the associated reentry MAC control messages shall be required, a value of 1 indicates the reentry MAC control message may be omitted. Bit #0: Omit AAI_SBC-REQ/RSP MAC control messages during reentry processing Bit #1: Omit PKM Authentication phase Bit #2: Omit AAI_REG-REQ/RSP message and higher layer protocol triggering (for IP address refresh) during reentry processing. Bit #3: If Bit #3 = 1, Full service and operational state transfer or sharing between Serving BS and Target BS (All static and dynamic context, e.g., ARQ windows, state machines | Shall be included for each target ABS when Mode=0b00 When Bit #2 is set to 0 and IP address refresh is needed, the AAI_REG-REQ/RSP includes Host configuration capabilities and its parameters to provide IP address information. |
| O | Seamless HO | 1 | Indicates whether seamless HO is supported 0: Seamless HO is no supported 1: Seamless HO is supported [*Note : Description was missing] |  |
| O | Ranging initiation deadline | 8 |  | Shall be included for each target ABS when Mode=0b00 |
| O | Service level prediction | 4 | Indicates the level of service the AMS can expect from this ABS. The following encodings apply: 0 = No service possible for this AMS 1 = Some service is available for one or several service flows authorized for the AMS 2 = For each authorized service flow, a MAC connection can be established with QoS specified by the AuthorizedQoSParamSet. 3 = No service level prediction available. | Shall be included for each target ABS when Mode=0b00 |
| O | Physical_Carrier_Index | 6 | Physical carrier index of the Target ABS where AMS will perform network reentry procedure. | Should be included for each target ABS which is multi-carrier ABS when Mode = 0b00 |
| O | Carrier Preassignment Indication | 1 | 1: information of pre-assigned secondary carriers by the target ABS is included in this message; 0: no pre-assigned secondary carrier | Shall be included for an AMS with multi-carrier capability when Mode = 0b00 |
| O | N_Preassigned_Carriers | 6 | Number of pre-assigned secondary carriers at the target ABS. A value of 0 means no secondary is pre-assigned. | Shall be included when Carrier Preassignment Indication = 1 |
| O | Pre-assigned secondary carrier information | TBD | Carrier information of each pre-assigned secondary carrier, which includes: Carrier Status Bitmap: indicating whether this pre-assigned carrier will be activated immediately after HO procedure is done; Physical carrier index of pre-assigned secondary carriers; Logical carrier index, which is assigned implicitly in the order of Carrier information of each pre-assigned secondary carriers. | Shall be included when N_Preassigned_Carriers > 0 |
| O | LZone Preamble Index | 8 | LZone Preamble Index for AMS to switch from MZone to LZone | Shall be included when Mode = 0b01. |
| O | Pre allocated Basic CID | 16 | Used by the AMS to derive its own primary CID and transport CID in target YBS. | May be included when HO to a LZone or a YBS |

Table 684 — AAI_HO-CMD Parameters

-- ASN1START -- HO command MAC control message AAI-HO-IND ::= SEQUENCE { messageType OCTET STRING (SIZE(1)), hoEventCode ENUMERATED { target-ABS-Selection, target-ABS-unreachable, serving-ABS-disconnected, ho-cancel }, targetABSID INTEGER (0..7) OPTIONAL, physicalCarrierIndex INTEGER (0..7) OPTIONAL, newTargetABSID INTEGER (0..7) OPTIONAL, AK_COUNT INTEGER (0..7) OPTIONAL } -- ASN1STOP

------------------------------------------------------  Text End  ---------------------------------------------------------------

Remedy 2:

In the P802.16m/D4, page 80, line 11, replace Table 685 to the following table and ASN.1 encoding format:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 685 — AAI_HO-REQ parameters

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Control Message Type | 8 | AAI_HO-REQ | NA |
| M | Report Metric | 8 | Bitmap indicator of trigger metrics that the AMS reports in this message. Bit #0: BS CINR mean Bit #1: BS RSSI mean Bit #2: Relative delay Bits #3-6: Reserved; shall be set to zero. | NA |
| M | N_New_BS_Index | 8 | Number of neighboring ABSs to be considered for HO, which are included in AAI_NBR-ADV message. | NA |
| M | Neighbor_ABS_ID | 8 | Index of ABS in AAI_NBR-ADV | Shall be included for each recommended neighbor BS |
| O | ABS CINR mean | 8 |  | Shall be included if Report Metric Bit #0 is set to 1 |
| O | ABS RSSI mean | 8 |  | Shall be included if Report Metric Bit #1 is set to 1 |
| O | Relative delay | 8 |  | Shall be included if Report Metric Bit #2 is set to 1 |
| O | Carrier Preassignment Indication | 1 | Indicates whether AMS needs pre-assignment of secondary carriers at the Target ABS. | May be included when Multi-Carrier Support = 1 |

-- ASN1START -- HO request MAC control message AAI-HO-REQ ::= SEQUENCE { messageType OCTET STRING (SIZE(1)), } -- ASN1STOP

------------------------------------------------------  Text End  ---------------------------------------------------------------

Remedy 3:

In the P802.16m/D4, page 80, line 49, replace Table 686 to the following table and ASN.1 encoding format:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 686 — AAI_HO-CMD parameters

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Control Message Type | 8 | AAI_HO-CMD | NA |
| M | Mode | 2 | 0b00: HO command; 0b01: Zone switch command from MZone to LZone; 0b10: AMS HO request rejected (ABS in list unavailable). In this case, AAI_HO-CMD message shall not include any target ABS. |  |
| O | HO Reentry Mode | 1 | 1: the AMS maintains communication with Serving ABS while performing network reentry with the Target ABS (Mode=0b00); or the AMS maintains communication with MZone while performing network reentry with LZone in the same frame or on another carrier (Mode=0b01). 0: the AMS disconnects from the Serving ABS before performing network reentry with the Target ABS (Mode=0b00); or the AMS disconnects from the MZone before performing network reentry with the LZone (Mode=0b01). | Should be included when Mode = 0b00. Shall be included when Mode = 0b01. |
| O | HO Reentry Interleaving Interval | 8 | If HO Reentry Interleaving Interval > 0, the AMS performs network reentry to the target ABS within the HO Reentry Interleaving Interval and continues data transmission with the Serving ABS in the remaining time. If HO Reentry Interleaving Interval =0, the AMS performs multi-carrier EBB (Established Before Break) HO procedure per 16.2.8.2.9.2.2 | Shall be included when HO Reentry Mode is set to 1 |
| O | HO Reentry Iteration | 8 | The requested number of iterating HO Reentry Intervals by an AMS. | Shall be included when HO Reentry Interleaving Interval > 0 |
| O | Disconnect time offset | 8 | Difference between Disconnect time and Action time in units of frames. The value of disconnect time shall be calculated for each T-ABS by adding/ subtracting this value with the value of Action time specified for this T-ABS. For HO_Reentry_Mode = 0, Disconnect time will be (Action time - Disconnect time offset); For HO_Reentry_Mode = 1, Disconnect time will be (Action time + Disconnect time offset). | Should be included when Mode = 0b00. |
| O | Resource_Retain_Time | 8 | Indicates the time when the Serving ABS will discard AMS’s context upon expiration of this timer. | Should be included when Mode = 0b00. Shall be included when Mode = 0b01. |
| O | Target ABS/YBS ID | [8][48] | The BS ID of the target BS | Shall be included for each target ABS when Mode=0b00 |
| O | Preamble Index | 8 | Preamble Index of Target ABS | Shall be included for each target ABS when Mode=0b00 |
| O | FA index | 8 | FA Index of Target ABS | Shall be included for each target ABS when Mode=0b00 |
| O | S-SFH change count | 8 | The change count of this BS's S-SFH IE | Shall be included for each target ABS when Mode=0b00 |
| O | Action Time | 2 | (Mode=0b00) Action Time included in this message is the absolute frame number at the serving ABS (Mode=0b01) This value is defined as the frame number that AMS starts zone switch. Action Time included in this message is indicated by frame number | Shall be included for each target ABS when Mode=0b00. Shall be included when Mode = 0b01. |
| O | CDMA_RNG_FLAG | 1 | Indicates whether CDMA ranging may be omitted or not. 0: The AMS may omit ranging 1: The AMS shall perform ranging | If CDMA_RNG_FLAG=0, CDMA based ranging shall be skipped. Otherwise, if set to 1, it indicates that ranging shall be done. This parameter shall be included for each target ABS when Mode=0b00.; It shall be included when Mode = 0b01. |
| O | Dedicated Ranging Code Flag | 1 | Indicates whether a dedicated ranging code is assigned or not 0: No dedicated ranging code assigned 1: Dedicated ranging code is assigned | If set to 0, it indicates that no dedicated ranging code is provided. If set to 1, it indicates that no dedicated ranging code is provided. May be included when CDMA_RNG_FLAG=1 |
| O | Dedicated Ranging Opportunity Flag | 1 | Indicates whether a dedicated ranging opportunity is assigned or not 0: No dedicated ranging opportunity assigned 1: Dedicated ranging opportunity is assigned | If set to 0, it indicates that no dedicated ranging opprotunity is provided. If set to 1, it indicates that no dedicated ranging opportunity is provided. May be included when CDMA_RNG_FLAG=1 |
| O | Dedicated CDMA ranging code | 5 | The assigned dedicated ranging code | Indicates the dedicated ranging code. Shall be included when Dedicated Ranging Code Flag =1 |
| O | Ranging opportunity index | 3 | The assigned dedicated ranging opportunity index | Indicates the index of the allocated ranging opportunity. Shall be included when Dedicated Opportunity Code Flag=1 |
| O | Subframe Index | 3 | Indicates the subframe index where the assigned dedicated ranging opportunity is allocated | Shall be included when Dedicated Opportunity Code Flag=1 |
| O | HO process optimization | 4 | For each bit location, a value of 0 indicates the associated reentry MAC control messages shall be required, a value of 1 indicates the reentry MAC control message may be omitted. Bit #0: Omit AAI_SBC-REQ/RSP MAC control messages during reentry processing Bit #1: Omit PKM Authentication phase Bit #2: Omit AAI_REG-REQ/RSP message and higher layer protocol triggering (for IP address refresh) during reentry processing. Bit #3: If Bit #3 = 1, Full service and operational state transfer or sharing between Serving BS and Target BS (All static and dynamic context, e.g., ARQ windows, state machines | Shall be included for each target ABS when Mode=0b00 When Bit #2 is set to 0 and IP address refresh is needed, the AAI_REG-REQ/RSP includes Host configuration capabilities and its parameters to provide IP address information. |
| O | Seamless HO | 1 | Indicates whether seamless HO is supported 0: Seamless HO is no supported 1: Seamless HO is supported |  |
| O | Ranging initiation deadline | 8 |  | Shall be included for each target ABS when Mode=0b00 |
| O | Service level prediction | 4 | Indicates the level of service the AMS can expect from this ABS. The following encodings apply: 0 = No service possible for this AMS 1 = Some service is available for one or several service flows authorized for the AMS 2 = For each authorized service flow, a MAC connection can be established with QoS specified by the AuthorizedQoSParamSet. 3 = No service level prediction available. | Shall be included for each target ABS when Mode=0b00 |
| O | Physical_Carrier_Index | 6 | Physical carrier index of the Target ABS where AMS will perform network reentry procedure. | Should be included for each target ABS which is multi-carrier ABS when Mode = 0b00 |
| O | Carrier Preassignment Indication | 1 | 1: information of pre-assigned secondary carriers by the target ABS is included in this message; 0: no pre-assigned secondary carrier | Shall be included for an AMS with multi-carrier capability when Mode = 0b00 |
| O | N_Preassigned_Carriers | 6 | Number of pre-assigned secondary carriers at the target ABS. A value of 0 means no secondary is pre-assigned. | Shall be included when Carrier Preassignment Indication = 1 |
| O | Pre-assigned secondary carrier information | TBD | Carrier information of each pre-assigned secondary carrier, which includes: Carrier Status Bitmap: indicating whether this pre-assigned carrier will be activated immediately after HO procedure is done; Physical carrier index of pre-assigned secondary carriers; Logical carrier index, which is assigned implicitly in the order of Carrier information of each pre-assigned secondary carriers. | Shall be included when N_Preassigned_Carriers > 0 |
| O | LZone Preamble Index | 8 | LZone Preamble Index for AMS to switch from MZone to LZone | Shall be included when Mode = 0b01. |
| O | Pre allocated Basic CID | 16 | Used by the AMS to derive its own primary CID and transport CID in target YBS. | May be included when HO to a LZone or a YBS |

-- ASN1START -- HO command MAC control message AAI-HO-CMD ::= SEQUENCE { messageType OCTET STRING (SIZE(1)), } -- ASN1STOP

------------------------------------------------------  Text End  ---------------------------------------------------------------