| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | ASN.1 Encoding for Handover Messages (16.2.3) |  |
| Date Submitted | 2010-03-12 |  |
| Source(s) | Kelvin Chou, Yih-Shen Chen, I-Kang Fu and Paul Cheng MediaTek Inc. | Kelvin.Chou@mediatek.com |
| Re: | Letter Ballot #31a on the Draft Amendment (IEEE P802.16m/D4) |  |
| Abstract | Text proposal on the format of handover messages in ASN.1 |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

ASN.1 Encoding for Handover Messages (16.2.3)

Kelvin Chou, Yih-Shen Chen, I-Kang Fu and Paul Cheng

MediaTek Inc.

Introduction

This contribution proposes the ASN.1 encoding formats for the following messages:

- AAI_HO-IND

- AAI_HO-REQ

- AAI_HO-CMD

- AAI_NBR-ADV

The original parameters for these messages have been restructured to fit into the ASN.1 coding structure. The presented ASN.1 codes have been syntax checked using the open ANS.1 compiler (asn1c).

Proposed Text

Remedy 1:

In the P802.16m/D4, page 79, line 51, replace Table 684 to the follow table and ASN.1 encoding format:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 684 — AAI_HO-IND Parameters

| M/O | Attributes / Array of attributes | Value / Note | Conditions |
| --- | --- | --- | --- |
| AAI-HO-IND |  |  |  |
| M | hoEvent | This is used to distinguish AAI_HO-IND among different scenarios: - targetABSSelection: Target ABS selection in case of multiple candidate T-ABSs. See TargetABSSelection - targetABSUnreachable: All target ABSs in AAI_HO-CMD are unreachable. In this case, the AMS shall include a new target ABS that was not included in AAI_HO-CMD. See TargetABSUnreachable - servingABSDisconnected: AMS unable to stay connected to serving ABS until expiration of disconnect time - hoCancel: HO cancel. See HandoverCancel | N/A |
| TargetABSSelection |  |  |  |
| M | targetABSID | Indicates the selected target ABS; | N/A |
| O | physicalCarrierIndex | Physical carrier index of the Target ABS where an AMS will perform network reentry procedure | May be presented when the target ABS is a multi-carrier ABS |
| TargetABSUnreachable |  |  |  |
| O | targetABSID | Indicates the preferred new target ABS | May be presented when AMS has a new candidate Target ABS |
| O | physicalCarrierIndex | Physical carrier index of the Target ABS where an AMS will perform network reentry procedure | May be presented when the target ABS is a multi-carrier ABS |
| HandoverCancel |  |  |  |
| M | AK_COUNT | The AMS’s current value of the AK_COUNT, which is used to generate the security keys in the target ABS. | N/A |

-- ASN1START

-- HO indication MAC control message

AAI-HO-IND ::= SEQUENCE {

hoEvent			CHOICE {

targetABSSelection		TargetABSSelection,

targetABSUnreachable	TargetABSUnreachable,

servingABSDisconnected	NULL,

hoCancel				HandoverCancel

}

}

TargetABSSelection ::= SEQUENCE {

targetABSID				INTEGER (0..7),

physicalCarrierIndex	INTEGER (0..7)		OPTIONAL

}

TargetABSUnreachable ::= SEQUENCE {

targetABSID				INTEGER (0..7)		OPTIONAL,

physicalCarrierIndex	INTEGER (0..7)		OPTIONAL

}

HandoverCancel ::= SEQUENCE {

akCount					AKCount

}

-- ASN1STOP

------------------------------------------------------  Text End  ---------------------------------------------------------------

Remedy 2:

In the P802.16m/D4, page 80, line 11, replace Table 685 to the following table and ASN.1 encoding format:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 685 — AAI_HO-REQ parameters

| M/O | Attributes / Array of attributes | Value / Note | Conditions |
| --- | --- | --- | --- |
| AAI-HO-REQ |  |  |  |
| M | reportMetrix | Bitmap indicator of trigger metrics that the AMS reports in this message. Bit #0: BS CINR mean Bit #1: BS RSSI mean Bit #2: Relative delay Bits #3-6: Reserved; shall be set to zero. | N/A |
| M | numberOfNewABSIndices | Number of neighboring ABSs to be considered for HO, which are included in AAI_NBR-ADV message. | N/A |
| M | neighborReportList | List of NeighborReport. See NeighborReport | N/A |
| O | carrierPreassignmentIndicator | Indicates whether AMS needs pre-assignment of secondary carriers at the Target ABS. | May be included when AMS support carrier aggregation |
| NeighborReport |  |  |  |
| M | absIndex | Index of ABS in AAI_NBR-ADV | N/A |
| O | cinrMean |  | Shall be included if reportMetrix #0 is set to 1 |
| O | rssiMean |  | Shall be included if reportMetrix #1 is set to 1 |
| O | relativeDelay |  | Shall be included if reportMetrix #2 is set to 1 |

-- ASN1START

-- HO request MAC control message

AAI-HO-REQ ::= SEQUENCE {

reportMetrix					BIT STRING {

cinr-Mean(0),

rssi-Mean(1),

relative-Delay(2)

},

numberOfNewABSIndices			INTEGER(0..255),

neighborReportList				SEQUENCE OF NeighborReport,

carrierPreassignmentIndicator	BOOLEAN							OPTIONAL

}

NeighborReport ::= SEQUENCE {

absIndex						INTEGER(0..255),

cinrMean						INTEGER(0..255)					OPTIONAL,

rssiMean						INTEGER(0..255)					OPTIONAL,

relativeDelay					INTEGER(0..255)					OPTIONAL

}

-- ASN1STOP

------------------------------------------------------  Text End  ---------------------------------------------------------------

Remedy 3:

In the P802.16m/D4, page 80, line 49, replace Table 686 to the following table and ASN.1 encoding format:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 686 — AAI_HO-CMD parameters

| M/O | Attributes / Array of attributes | Value / Note | Conditions |
| --- | --- | --- | --- |
| AAI-HO-CMD |  |  |  |
| M | Mode | Choice of the following structure according to the HO mode: - HO command (hoCmd): see HandoverCommand; - Zone switch command from MZone to LZone (zsCmd): see ZoneSwitchCommand; - AMS HO request rejected (ABS in list unavailable) (hoReject). In this case, AAI_HO-CMD message shall not include any target ABS. | N/A |
| HandoverCommand |  |  |  |
| M | hoReentryMode | 1: the AMS maintains communication with Serving ABS while performing network reentry with the Target ABS 0: the AMS disconnects from the Serving ABS before performing network reentry with the Target ABS | N/A |
| O | hoReentryInterleavingInterval | If hoReentryInterleavingInterval > 0, the AMS performs network reentry to the target ABS within the HO Reentry Interleaving Interval and continues data transmission with the Serving ABS in the remaining time. If hoReentryInterleavingInterval =0, the AMS performs multi-carrier EBB (Established Before Break) HO procedure per 16.2.8.2.9.2.2 | Presented when hoReentryMode is set to 1 |
| O | hoReentryIteration | The requested number of iterating HO Reentry Intervals by an AMS. | Presented when hoReentryInterleavingInterval > 0 |
| M | disconnectTimeOffset | Difference between Disconnect time and Action time in units of frames. The value of disconnect time shall be calculated for each T-ABS by adding/ subtracting this value with the value of Action time specified for this T-ABS. For hoReentryMode = 0, Disconnect time will be (Action time - Disconnect time offset); For hoReentryMode = 1, Disconnect time will be (Action time + Disconnect time offset). | N/A |
| M | resourceRetainTime | Indicates the time when the Serving ABS will discard AMS’s context upon expiration of this timer. | N/A |
| O | preallocatedBasicCID | Used by the AMS to derive its own primary CID and transport CID in target YBS. | May be included when HO to a LZone or a YBS |
| TargetBSInfo |  |  |  |
| M | targetBSID | The BS ID of the target ABS/YBS | N/A |
| M | preambleIndex | Preamble Index of Target ABS | N/A |
| M | faIndex | FA Index of Target ABS | N/A |
| M | s-sfhChangeCount | The change count of this BS's S-SFH IE | N/A |
| M | actionTime | The absolute frame number at the serving ABS (Mode=0b01) This value is defined as the frame number that AMS starts zone switch. Action Time included in this message is indicated by frame number | N/A |
| M | cdmaRngFlag | Indicates whether CDMA ranging may be omitted or not. 0: The AMS may omit ranging 1: The AMS shall perform ranging | N/A |
| O | dedicatedCDMARngCode | The assigned dedicated ranging code | May be presented when cdmaRngFlag = 1 |
| O | dedicatedRngOpp | The assigned dedicated ranging opportunity. See DedicatedRngOpp | May be presented when cdmaRngFlag = 1 |
| M | hoProcessOptimization | For each bit location, a value of 0 indicates the associated reentry MAC control messages shall be required, a value of 1 indicates the reentry MAC control message may be omitted. Bit #0: Omit AAI_SBC-REQ/RSP MAC control messages during reentry processing Bit #1: Omit PKM Authentication phase Bit #2: Omit AAI_REG-REQ/RSP message and higher layer protocol triggering (for IP address refresh) during reentry processing. Bit #3: If Bit #3 = 1, Full service and operational state transfer or sharing between Serving BS and Target BS (All static and dynamic context, e.g., ARQ windows, state machines | N/A |
| M | seamlessHO | Indicates whether seamless HO is supported 0: Seamless HO is no supported 1: Seamless HO is supported | N/A |
| M | rngInitiationDeadline |  | N/A |
| M | serviceLevelPrediction | Indicates the level of service the AMS can expect from this ABS. The following encodings apply: 0 = No service possible for this AMS 1 = Some service is available for one or several service flows authorized for the AMS 2 = For each authorized service flow, a MAC connection can be established with QoS specified by the AuthorizedQoSParamSet. 3 = No service level prediction available. | N/A |
| O | physicalCarrierIndex | Physical carrier index of the Target ABS where AMS will perform network reentry procedure. | Presented when the target ABS is a multi-carrier ABS |
| O | preassignedCarrierInfo | Information of pre-assigned secondary carriers by the target ABS. See PreassignedCarrierInfo | May be presented for the AMS which support carrier aggregation. |
| ZoneSwitchCommand |  |  |  |
| M | hoReentryMode | 1: the AMS maintains communication with MZone while performing network reentry with LZone in the same frame or on another carrier. 0: the AMS disconnects from the MZone before performing network reentry with the LZone. | N/A |
| M | resourceRetainTime | Indicates the time when the Serving ABS will discard AMS’s context upon expiration of this timer. | N/A |
| M | actionTime | The frame number that AMS starts zone switch. Action Time included in this message is indicated by frame number | N/A |
| M | preambleIndex-LZone | LZone Preamble Index for AMS to switch from MZone to LZone | N/A |
| M | cdmaRngFlag | Indicates whether CDMA ranging may be omitted or not. 0: The AMS may omit ranging 1: The AMS shall perform ranging | If cdmaRngFlag =0, CDMA based ranging shall be skipped. Otherwise, if set to 1, it indicates that ranging shall be done. |
| DedicatedRngOpp |  |  |  |
| M | rngOppIndex | The assigned dedicated ranging opportunity index | N/A |
| M | subframeIndex | Indicates the subframe index where the assigned dedicated ranging opportunity is allocated | N/A |
| PreassignedCarrierInfo |  |  |  |
| M | numberOfPreassignedCarriers | Number of pre-assigned secondary carriers at the target ABS. | N/A |
| M | carrierStatusBitmap | Indicating whether this pre-assigned carrier will be activated immediately after HO procedure is done. Size of the bitmap is numberOfPreassignedCarriers | N/A |
| M | physicalCarrierIndexList | Physical carrier index of pre-assigned secondary carriers. The Logical carrier index, which is assigned implicitly in the order of carrier information of each pre-assigned secondary carriers. | N/A |

-- ASN1START

-- HO command MAC control message

AAI-HO-CMD ::= SEQUENCE {

mode		CHOICE {

hoCmd 		HandoverCommand,

zsCmd		ZoneSwitchCommand,

hoReject	NULL

}

}

HandoverCommand ::= SEQUENCE {

hoReentryMode					BOOLEAN,

hoReentryInterleavingInterval	INTEGER(0..255)					OPTIONAL,

hoReentryIteration				INTEGER(0..255)					OPTIONAL,

disconnectTimeOffset			INTEGER(0..255),

resourceRetainTime				INTEGER(0..255),

targetBSList					SEQUENCE OF TargetBSInfo,

preallocatedBasicCID			INTEGER(0..65535)				OPTIONAL

}

ZoneSwitchCommand ::= SEQUENCE {

hoReentryMode					BOOLEAN,

resourceRetainTime				INTEGER(0..255),

actionTime						INTEGER(0..3),

preambleIndex-LZone				INTEGER(0..113),

cdmaRngFlag						BOOLEAN

}

TargetBSInfo ::= SEQUENCE {

targetBSID						ABSID,

preambleIndex					INTEGER(0..255),

faIndex							INTEGER(0..255),

s-sfhChangeCount				INTEGER(0..255),

actionTime						INTEGER(0..3),

cdmaRngFlag						BOOLEAN,

dedicatedCDMARngCode			INTEGER(0..31)					OPTIONAL,

dedicatedRngOpp					DedicatedRngOpp					OPTIONAL,

hoProcessOptimization			BIT STRING (SIZE (4)),

seamlessHO						BOOLEAN,

rngInitiationDeadline			INTEGER(0..31),

serviceLevelPrediction			ServiceLevelPrediction,

physicalCarrierIndex			INTEGER(0..63)					OPTIONAL,

preassignedCarrierInfo			PreassignedCarrierInfo			OPTIONAL

}

DedicatedRngOpp ::= SEQUENCE {

rngOppIndex						INTEGER(0..7),

subframeIndex					INTEGER(0..7)

}

ServiceLevelPrediction ::= ENUMERATED {

noServiceAvailable(0),

partiallyAvailable(1),

fullyAvailable(2),

noPrediction(3) }

PreassignedCarrierInfo ::= SEQUENCE {

numberOfPreassignedCarriers		INTEGER(1..63),

carrierStatusBitmap				BIT STRING,

physicalCarrierIndexList		SEQUENCE OF INTEGER(0..63)

}

-- ASN1STOP

------------------------------------------------------  Text End  ---------------------------------------------------------------

Remedy 4:

In the P802.16m/D4, page 85, line 1, replace Table 687 to the following table and ASN.1 encoding format:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 687 — AAI_NBR-ADV parameters

| M/O | Attributes / Array of attributes | Value / Note | Conditions |
| --- | --- | --- | --- |
| AAI-NBR-ADV |  |  |  |
| M | changeCount | Change count for this message | N/A |
| M | cellType | Cell type in this message 0b000 macro 0b001 micro 0b010 macro hotzone 0b011 femto 0b100 relay | N/A |
| M | totalNumberOfSegments | Total number of segments of AAI_NBR-ADV for this cell type | N/A |
| M | segmentIndex | Indicates current segment index of this message in the specific cell type | N/A |
| M | numberOfBSs | Total number of BSs to be included in this AAI_NBR-ADV segment | N/A |
| M | startingABSIndex | Starting ABS Index is the index offset from the last ABS of the previous AAI_NBR-ADV segment. If this is the first AAI_NBR-ADV segment, the Starting ABS Index will be 0. Hence, each AAI_NBR-ADV segment has one Index which corresponds to the first ABS in that AAI_NBR-ADV segment. | N/A |
| M | neighborInfoList | List of neighbor information. See NeighborInfo | N/A |
| NeighborInfo |  |  |  |
| M | absInfo | ABS information. See ABSInfo | N/A |
| M | sfhEncodingFormat | Choice of the following SFH encoding format information: - full subpacket information: See FullSubpktInfo; - delta encoding (the 1st BS in this cell type shall use full Subpkt encoding ): See DeltaEncoding; - no SFH included. For macrocell ABS, either full or delta information shall be included. | N/A |
| ABSInfo |  |  |  |
| M | absID | ABS ID | N/A |
| M | numberOfCarriers | Number of carriers of the BS | N/A |
| M | carrierInfoList | List of carrier information. See CarrierInfo | N/A |
| CarrierInfo |  |  |  |
| M | saPreambleIndex |  | N/A |
| M | aPreambleTxPower |  | N/A |
| M | physicalCarrierIndex | Refer to the physical carrier index in AAI_Global-Config message | N/A |
| M | macProtocolVersion | MAC protocol version of the BS Consistent with REV.2 definition, with new MAC protocol version 9 defined for 16m. | N/A |
| FullSubpktInfo |  |  |  |
| O | sfh-subpkt1 | SFH subpacket 1 information excluding those fields already in cell type info | Presented when needed |
| O | sfh-subpkt2 | SFH subpacket 2 information excluding those fields already in cell type info | Presented when needed |
| O | sfh-subpkt3 | SFH subpacket 3 information excluding those fields already in cell type info | Presented when needed |
| DeltaEncoding |  |  |  |
| M | deltaInfo | Delta information w.r.t the reference BS | N/A |

-- ASN1START

-- Neighbor advertisement MAC control message

AAI-NBR-ADV ::= SEQUENCE {

changeCount					INTEGER (0..7),

cellType					ENUMERATED {macro, micro, macro-hotzone, femto, relay},

totalNumberOfSegments		INTEGER (0..15),

segmentIndex				INTEGER (0..15),

numberOfBSs					INTEGER (0..255),

startingABSIndex			INTEGER (0..255),

neighborInfoList			SEQUENCE OF NeighborInfo

}

NeighborInfo ::= SEQUENCE {

absInfo						ABSInfo,

sfhEncodingFormat			CHOICE {

fullSubpktInfo	FullSubpktInfo,

deltaEncoding	DeltaEncoding,

noSFHIncluded	NULL }

}

FullSubpktInfo ::= SEQUENCE {

sfh-subpkt1					BIT STRING (SIZE (88))				OPTIONAL,

sfh-subpkt2					BIT STRING (SIZE (88))				OPTIONAL,

sfh-subpkt3					BIT STRING (SIZE (88))				OPTIONAL

}

DeltaEncoding ::= SEQUENCE {

deltaInfo					BIT STRING

}

-- ASN1STOP

------------------------------------------------------  Text End  ---------------------------------------------------------------

Appendix

A complete version of ASN.1 code for the proposed messages in this contribution is provided. The code has been successfully compiled using open ASN.1 compiler (asn1c).

MacControlMsg DEFINITIONS AUTOMATIC TAGS ::= BEGIN

-- MAC Control Messages

MAC-Control-Message ::= SEQUENCE {

message MAC-Control-Msg-Type

}

MAC-Control-Msg-Type ::= CHOICE {

aaiRngReq	AAI-RNG-REQ,

aaiRngRsp	AAI-RNG-RSP,

aaiRngAck	AAI-RNG-ACK,

aaiNbrAdv	AAI-NBR-ADV,

aaiHoInd	AAI-HO-IND,

aaiHoReq	AAI-HO-REQ,

aaiHoCmd	AAI-HO-CMD

}

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

AKCount	::= 		INTEGER (0..7)

CMACTuple ::= 		BIT STRING (SIZE (92))

ABSID ::=			BIT STRING (SIZE (48))

AMSIDStar ::=		BIT STRING (SIZE (48))

MACVersion ::=		BIT STRING (SIZE (8))

StationID ::=   	BIT STRING (SIZE (12))

TemporarySTID ::=	BIT STRING (SIZE (12))

-------------------------------------------------------------------------------

------------------------ AAI-HO-IND ------------------------------------------

-------------------------------------------------------------------------------

-- HO indication MAC control message

AAI-HO-IND ::= SEQUENCE {

hoEvent			CHOICE {

targetABSSelection		TargetABSSelection,

targetABSUnreachable	TargetABSUnreachable,

servingABSDisconnected	NULL,

hoCancel				HandoverCancel

}

}

TargetABSSelection ::= SEQUENCE {

targetABSID				INTEGER (0..7),

physicalCarrierIndex	INTEGER (0..7)		OPTIONAL

}

TargetABSUnreachable ::= SEQUENCE {

targetABSID				INTEGER (0..7)		OPTIONAL,

physicalCarrierIndex	INTEGER (0..7)		OPTIONAL

}

HandoverCancel ::= SEQUENCE {

akCount					AKCount

}

-------------------------------------------------------------------------------

------------------------ AAI-HO-REQ ------------------------------------------

-------------------------------------------------------------------------------

-- HO request MAC control message

AAI-HO-REQ ::= SEQUENCE {

reportMetrix					BIT STRING {

cinr-Mean(0),

rssi-Mean(1),

relative-Delay(2)

},

numberOfNewABSIndices			INTEGER(0..255),

neighborReportList				SEQUENCE OF NeighborReport,

carrierPreassignmentIndicator	BOOLEAN							OPTIONAL

}

NeighborReport ::= SEQUENCE {

absIndex						INTEGER(0..255),

cinrMean						INTEGER(0..255)					OPTIONAL,

rssiMean						INTEGER(0..255)					OPTIONAL,

relativeDelay					INTEGER(0..255)					OPTIONAL

}

-------------------------------------------------------------------------------

------------------------ AAI-HO-CMD ------------------------------------------

-------------------------------------------------------------------------------

-- HO command MAC control message

AAI-HO-CMD ::= SEQUENCE {

mode		CHOICE {

hoCmd 		HandoverCommand,

zsCmd		ZoneSwitchCommand,

hoReject	NULL

}

}

HandoverCommand ::= SEQUENCE {

hoReentryMode					BOOLEAN,

hoReentryInterleavingInterval	INTEGER(0..255)					OPTIONAL,

hoReentryIteration				INTEGER(0..255)					OPTIONAL,

disconnectTimeOffset			INTEGER(0..255),

resourceRetainTime				INTEGER(0..255),

targetBSList					SEQUENCE OF TargetBSInfo,

preallocatedBasicCID			INTEGER(0..65535)				OPTIONAL

}

ZoneSwitchCommand ::= SEQUENCE {

hoReentryMode					BOOLEAN,

resourceRetainTime				INTEGER(0..255),

actionTime						INTEGER(0..3),

preambleIndex-LZone				INTEGER(0..113),

cdmaRngFlag						BOOLEAN

}

TargetBSInfo ::= SEQUENCE {

targetBSID						ABSID,

preambleIndex					INTEGER(0..255),

faIndex							INTEGER(0..255),

s-sfhChangeCount				INTEGER(0..255),

actionTime						INTEGER(0..3),

cdmaRngFlag						BOOLEAN,

dedicatedCDMARngCode			INTEGER(0..31)					OPTIONAL,

dedicatedRngOpp					DedicatedRngOpp					OPTIONAL,

hoProcessOptimization			BIT STRING (SIZE (4)),

seamlessHO						BOOLEAN,

rngInitiationDeadline			INTEGER(0..31),

serviceLevelPrediction			ServiceLevelPrediction,

physicalCarrierIndex			INTEGER(0..63)					OPTIONAL,

preassignedCarrierInfo			PreassignedCarrierInfo			OPTIONAL

}

DedicatedRngOpp ::= SEQUENCE {

rngOppIndex						INTEGER(0..7),

subframeIndex					INTEGER(0..7)

}

ServiceLevelPrediction ::= ENUMERATED {

noServiceAvailable(0),

partiallyAvailable(1),

fullyAvailable(2),

noPrediction(3) }

PreassignedCarrierInfo ::= SEQUENCE {

numberOfPreassignedCarriers		INTEGER(1..63),

carrierStatusBitmap				BIT STRING,

physicalCarrierIndexList		SEQUENCE OF INTEGER(0..63)

}

-------------------------------------------------------------------------------

------------------------ AAI-RNG-REQ ------------------------------------------

-------------------------------------------------------------------------------

-- Ranging request MAC control message

AAI-RNG-REQ ::= SEQUENCE {

rngPurposeIndication		BIT STRING {

reentry-ho-idle(0),

location-update-dcr-ext(1),

emergency-call-setup(2),

location-update-embs(3),

location-update-dcr(4),

reentry-dcr(5),

reentry-coverage-loss(6),

reentry-legacy(7) }				OPTIONAL,

csgIDList					SEQUENCE OF CSGID					OPTIONAL,

rngInfo						CHOICE {

nwReentryInfo	NWReentryInfo,

zoneSwitchInfo	ZoneSwitchInfo,

directHOInfo	DirectHOInfo

}

}

CSGID ::= 						BIT STRING (SIZE (10))

NWReentryInfo ::= SEQUENCE {

servingBSID					BIT STRING (SIZE (48))				OPTIONAL, -- HO reentry

crID						BIT STRING (SIZE (16))				OPTIONAL, -- reentry from CR or DCR

stationID					StationID							OPTIONAL, -- HO reentry

akCount						AKCount								OPTIONAL, -- reentry, locateion update, HO

amsIDStar					AMSIDStar							OPTIONAL,

macVersion					MACVersion							OPTIONAL,

pagingInfo					PagingInfo							OPTIONAL, -- reentry from idle

pagingCycleChange			INTEGER (0..15)						OPTIONAL,

powerDownIndicator			BOOLEAN								OPTIONAL,

amsMobilityInfo				ENUMERATED {slow, medium, fast}		OPTIONAL,

cmacTuple					CMACTuple							OPTIONAL

}

PagingInfo ::= SEQUENCE {

pagingControllerID			BIT STRING (SIZE (48)),

deregistrationID			BIT STRING (SIZE (10)),

pagingGroupID				BIT STRING (SIZE (16)),

pagingCycle					INTEGER (0..15),

pagingOffset				INTEGER (0..15)

}

ZoneSwitchInfo ::= SEQUENCE {

nonceAMS					BIT STRING (SIZE (64)),

nonceABS					BIT STRING (SIZE (64))

}

DirectHOInfo ::= SEQUENCE {

servingBSID					BIT STRING (SIZE (48)),

legacyCID					INTEGER(0..4095),

legacyCMACKeyCount			INTEGER(0..65535),

legacyCMACTuple				BIT STRING (SIZE (104))

}

-------------------------------------------------------------------------------

------------------------ AAI-RNG-RSP ------------------------------------------

-------------------------------------------------------------------------------

-- Ranging response MAC control message

AAI-RNG-RSP ::= SEQUENCE {

rangingStatus				RangingStatus,

stID						CHOICE {

temporarySTID	TemporarySTID,

stationID		StationID

}									OPTIONAL,

amsIDStar					AMSIDStar							OPTIONAL,

locationUpdateResponse		LocationUpdateResponse				OPTIONAL,

pagingInfo					PagingInfo							OPTIONAL,

hoProcessOptimization		BIT STRING (SIZE (4))				OPTIONAL,

nbrMeasurementReport		BOOLEAN								OPTIONAL,

zoneSwitchInfo				ZoneSwitchInfo						OPTIONAL,

redirectionInfoList			SEQUENCE OF ABSInfo					OPTIONAL,

unsolicitedBWIndicator		BOOLEAN								OPTIONAL,

clcInfo						CLCInfo								OPTIONAL,

crID						BIT STRING (SIZE (16))				OPTIONAL,

rangingRequestBit			BOOLEAN								OPTIONAL

}

LocationUpdateResponse ::= ENUMERATED {

success,

failed,

success-DL-traffic-pending,

dcr-mode-initiation-allowed }

ABSInfo ::= SEQUENCE {

absID						ABSID,

numberOfCarriers			INTEGER(1..15),

carrierInfoList				SEQUENCE OF CarrierInfo

}

CarrierInfo ::= SEQUENCE {

saPreambleIndex				INTEGER(0..767),

aPreambleTxPower			INTEGER(0..255),

physicalCarrierIndex		INTEGER(0..63),

macProtocolVersion			INTEGER(0..255)

}

CLCInfo ::= SEQUENCE {

infoType					ENUMERATED {

clc-limit,

clc-request,

clc-response,

clc-report }

--	response					CHOICE {

--								}

}

-------------------------------------------------------------------------------

------------------------ AAI-RNG-ACK ------------------------------------------

-------------------------------------------------------------------------------

-- Ranging acknowledgement MAC control message

AAI-RNG-ACK ::= SEQUENCE {

frameIdentifier				FrameIdentifier,

rangingAck					CHOICE {

unicastRngAck	UnicastRngAck,

broadcastRngAck	BroadcastRngAck

}

}

FrameIdentifier ::= SEQUENCE {

superFrameNumber			INTEGER (0..3),

frameIndex					INTEGER (0..3)

}

UnicastRngAck ::= SEQUENCE {

rangingStatus				RangingStatus,

adjustmentParameters		AdjustmentParameters		OPTIONAL

}

RangingStatus ::= ENUMERATED {	continue(0), abort(1), success(2) }

AdjustmentParameters ::= SEQUENCE {

timeingOffsetAdjustment		INTEGER (0..1023)			OPTIONAL,

powerLevelAdjustment		INTEGER (0..7)				OPTIONAL,

frequencyOffsetAdjustment	INTEGER (0..255)			OPTIONAL

}

BroadcastRngAck ::=	SEQUENCE {

rngAckBitmap				BIT STRING,

detectedSlotsList			SEQUENCE OF DetectedSlot	OPTIONAL

}

DetectedSlot ::= SEQUENCE {

numberOfReceivedCodes		INTEGER (0..31),

receivedCodesList			SEQUENCE OF ReceivedCode

}

ReceivedCode ::= SEQUENCE {

codeIndex					INTEGER (0..127),

unicastRngAck				UnicastRngAck

}

-------------------------------------------------------------------------------

------------------------ AAI-NBR-ADV ------------------------------------------

-------------------------------------------------------------------------------

-- Neighbor advertisement MAC control message

AAI-NBR-ADV ::= SEQUENCE {

changeCount					INTEGER (0..7),

cellType					ENUMERATED {macro, micro, macro-hotzone, femto, relay},

totalNumberOfSegments		INTEGER (0..15),

segmentIndex				INTEGER (0..15),

numberOfBSs					INTEGER (0..255),

startingABSIndex			INTEGER (0..255),

neighborInfoList			SEQUENCE OF NeighborInfo

}

NeighborInfo ::= SEQUENCE {

absInfo						ABSInfo,

sfhEncodingFormat			CHOICE {

fullSubpktInfo	FullSubpktInfo,

deltaEncoding	DeltaEncoding,

noSFHIncluded	NULL }

}

FullSubpktInfo ::= SEQUENCE {

sfh-subpkt1					BIT STRING (SIZE (88))				OPTIONAL,

sfh-subpkt2					BIT STRING (SIZE (88))				OPTIONAL,

sfh-subpkt3					BIT STRING (SIZE (88))				OPTIONAL

}

DeltaEncoding ::= SEQUENCE {

deltaInfo					BIT STRING

}

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

END