| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Handover Messages Clean Up (16.2.3) |  |
| Date Submitted | 2010-03-16 |  |
| Source(s) | Kelvin Chou, Yih-Shen Chen, I-Kang Fu and Paul Cheng MediaTek Inc. | Kelvin.Chou@mediatek.com |
| Re: | Letter Ballot #31a on the Draft Amendment (IEEE P802.16m/D4) |  |
| Abstract | Text proposal on the format of handover messages in ASN.1 |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

ASN.1 Encoding for Handover Messages (16.2.3)

Kelvin Chou, Yih-Shen Chen, I-Kang Fu and Paul Cheng

MediaTek Inc.

Proposed Text

Remedy 1:

In the P802.16m/D4, page 79, line 51, replace Table 684 to the follow table:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 684 — AAI_HO-IND Parameters

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | HO Event Code | 8 | Used to distinguish AAI_HO-IND among different scenarios 0b000: Target ABS selection in case of multiple candidate T-ABSs in HO-CMD. The serving ABS does not meet the “ABS unreachable” triggers. 0b001: All target ABSs in AAI_HO-CMD are unreachable. The AMS includes a new target ABS that was not included in AAI_HO-CMD. The serving ABS does not meet the “ABS unreachable” triggers. 0b010: The serving ABS meets the “ABS unreachable” triggers. The AMS will attempt network re-entry at the indicated T-ABS. 0b011: HO cancel. 0b100: Indicate HO cancel due to mismatch of SFH | NA |
| O | Target ABS ID | 48 | NA | Shall be included when HO Event Code = 0b000 or 0b001 |
| O | Physical_Carrier_Index | 8 | Physical carrier index of the Target ABS where an AMS will perform network reentry procedure | May be included when the target ABS is a multi-carrier ABS and HO Event Code = 0b000 |

------------------------------------------------------  Text End  ---------------------------------------------------------------

Remedy 2:

In the P802.16m/D4, page 80, line 11, replace Table 685 to the following table:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 685 — AAI_HO-REQ parameters

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Report Metric | 8 | Bitmap indicator of trigger metrics that the AMS reports in this message Bit #0: BS CINR mean Bit #1: BS RSSI mean Bits #3-7: Reserved; shall be set to zero | NA |
| M | AAI_NBR-ADV Change count | 8 | AAI_NBR-ADV change count last received from the serving ABS | NA |
| M | N_New_ABS_Index | 8 | Number of neighboring ABSs to be considered for HO, which are included in AAI_NBR-ADV message. | NA |
| O | N_New_ABS_Full | 48 | The number of neighboring ABSs to be are identified by full considered for handover, which is 48 bit ABSID. |  |
| O | Neighbor_ABS_Index | 8 | ABS index corresponds to AAI_NBR-ADV. | Shall be included when N_New_ABS_Index is not 0. |
| O | Neighbor_ABS_ID | 48 | - | Shall be included when N_New_BS_Full is not 0 |
| O | ABS CINR mean | 8 | - | Shall be included if Report Metric Bit #0 is set to 0 |
| O | ABS RSSI mean | 8 | - | Shall be included if Report Metric Bit #1 is set to 1 |
| O | Relative delay | 8 | - | Shall be included if Report Metric Bit #2 is set to 1 |
| O | Carrier Preassignment Indication | 1 | Indicates whether AMS needs pre-assignment of secondary carriers at the Target ABS. | May be included when Multi-Carrier Support = 1 |

------------------------------------------------------  Text End  ---------------------------------------------------------------

Remedy 3:

In the P802.16m/D4, page 80, line 49, replace Table 686 to the following table:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 686 — AAI_HO-CMD parameters

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Mode | 2 | 0b00: HO command; 0b01: Zone switch command from MZone to LZone; 0b10: AMS HO request rejected (ABS in list unavailable). In this case, AAI_HO-CMD message shall not include any target ABS. | NA |
| O | HO Reentry Mode | 1 | 1: the AMS maintains communication with Serving ABS while performing network reentry with the Target ABS (Mode=0b00); or the AMS maintains communication with MZone while performing network reentry with LZone in the same frame or on another carrier (Mode=0b01). 0: the AMS disconnects from the Serving ABS before performing network reentry with the Target ABS (Mode=0b00); or the AMS disconnects from the MZone before performing network reentry with the LZone (Mode=0b01). | Should be included when Mode = 0b00. (Note: This should be changed to SHALL) Shall be included when Mode = 0b01. |
| O | HO Reentry Interleaving Interval | 8 | If HO Reentry Interleaving Interval > 0, the AMS performs network reentry to the target ABS within the HO Reentry Interleaving Interval and continues data transmission with the Serving ABS in the remaining time. If HO Reentry Interleaving Interval =0, the AMS performs multi-carrier EBB (Established Before Break) HO procedure per 16.2.8.2.9.2.2 | Shall be included when HO Reentry Mode is set to 1 |
| O | HO Reentry Iteration | 3 | The requested number of iterating HO Reentry Intervals by an AMS. | Shall be included when HO Reentry Interleaving Interval > 0 |
| O | Disconnect time offset | 8 | Difference between Disconnect time and Action time in units of frames. The value of disconnect time shall be calculated for each T-ABS by adding/ subtracting this value with the value of Action time specified for this T-ABS. For HO_Reentry_Mode = 0, Disconnect time will be (Action time - Disconnect time offset); For HO_Reentry_Mode = 1, Disconnect time will be (Action time + Disconnect time offset). | Should be included when Mode = 0b00. |
| O | Resource_Retain_Time | 8 | The time when the serving BS discards MS’s context. | Should be included when Mode = 0b00. Shall be included when Mode = 0b01. |
| O | targetBSID | 48 | - | Shall be included for each target ABS when Mode=0b00 |
| O | Preamble Index | 10 |  | Shall be included for each target ABS when Mode=0b00 |
| O | FA index | 8 |  | Shall be included for each target ABS when Mode=0b00 |
| O | Action Time | 8 | (Mode=0b00) Action Time included in this message is the absolute frame number at the serving ABS (Mode=0b01) This value is defined as the frame number that AMS starts zone switch. Action Time included in this message is indicated by frame number | Shall be included for each target ABS when Mode=0b00. Shall be included when Mode = 0b01. |
| O | CDMA_RNG_FLAG | 1 | If CDMA_RNG_FLAG=0, CDMA based ranging shall be skipped. Otherwise, if set to 1, it indicates that ranging shall be done. This parameter shall be included for each target ABS when Mode=0b00.; It shall be included when Mode = 0b01. If CDMA_RNG_FLAG is set to 0, pre-assigned STID at the target BS shall be assigned. | Shall be included for each target ABS when Mode=0b00 |
| O | Dedicated Ranging Code Flag | 1 | If set to 0, it indicates that no dedicated ranging code is provided. If set to 1, it indicates that no dedicated ranging code is provided. | May be included for each target ABS when CDMA_RNG_FLAG=1 |
| O | Dedicated Ranging Opportunity Flag | 1 | If set to 0, it indicates that no dedicated ranging opportunity is provided. If set to 1, it indicates that dedicated ranging opportunity is provided. | May be included for each target ABS when CDMA_RNG_FLAG=1 |
| O | Dedicated CDMA ranging code | 5 | Indicates the dedicated ranging code. | Shall be included when Dedicated Ranging Code Flag =1 |
| O | Ranging opportunity index | 3 | Indicates the index of the allocated ranging opportunity. Ranging opportunity shall be located in the first UL subframe. | Shall be included when Dedicated Opportunity Code Flag=1 |
| O | HO process optimization | 4 | For each bit location, a value of 0 indicates the associated reentry MAC control messages shall be required, a value of 1 indicates the reentry MAC control message may be omitted. Bit #0: Omit AAI_SBC-REQ/RSP MAC control messages during reentry processing Bit #1: Omit PKM Authentication phase Bit #2: Omit AAI_REG-REQ/RSP message and higher layer protocol triggering (for IP address refresh) during reentry processing. Bit #3: If Bit #3 = 1, Full service and operational state transfer or sharing between Serving BS and Target BS (All static and dynamic context, e.g., ARQ windows, state machines | Shall be included for each target ABS when Mode=0b00 When Bit #2 is set to 0 and IP address refresh is needed, the AAI_REG-REQ/RSP includes Host configuration capabilities and its parameters to provide IP address information. |
| O | Seamless HO | 1 | Indicates whether seamless HO is supported | Shall be included for each target ABS when Mode=0b00. If CDMA_RNG_FLAG is set to 1, and both Dedicated Ranging Code Flag and Dedicated Ranging Opportunity Flag is set to 0, Data packets shall be exchanged after receiving the AAI-RNG-REQ from the AMS. |
| O | Ranging initiation deadline | 8 | An AMS shall send the AAI_RNG-REQ message during HO until Ranging initiation deadline | Shall be included for each target ABS when Mode=0b00 |
| O | Pre-assigned STID | 12 | STID assigned to the AMS by the target ABS | Should be included for each target ABS when Mode = 0b00 |
| O | Service level prediction | 2 | Indicates the level of service the AMS can expect from this ABS. The following encodings apply: 0 = No service possible for this AMS 1 = Some service is available for one or several service flows authorized for the AMS 2 = For each authorized service flow, a MAC connection can be established with QoS specified by the AuthorizedQoSParamSet. 3 = No service level prediction available. | Shall be included for each target ABS when Mode=0b00 |
| O | Physical_Carrier_Index | 8 | Physical carrier index of the recommended Target ABS | Should be included for each target ABS which is multi-carrier ABS when Mode = 0b00 and target ABS is not included in AAI_NBR-ADV message or is multi-carrier ABS |
| O | Channel bandwidth | 8 | Channel bandwidth of recommended target ABS in units of 125 KHz. | Shall be included for each target ABS when Mode=0b00 and target ABS is not included in AAI_NBR-ADV message |
| O | CP length | 2 | CP length of the recommended target ABS 00: 1/8 01: 1/16 10: 1/4 | Shall be included for each target ABS when Mode=0b00 and target ABS is not included in AAI_NBR-ADV message |
| O | Carrier Preassignment Indication | 1 | 1: information of pre-assigned secondary carriers by the target ABS is included in this message; 0: no pre-assigned secondary carrier | Shall be included for an AMS with multi-carrier capability when Mode = 0b00 |
| O | N_Preassigned_Carriers | 3 | Number of pre-assigned secondary carriers at the target ABS. A value of 0 means no secondary is pre-assigned. | Shall be included when Carrier Preassignment Indication = 1 |
| O | Pre-assigned secondary carrier information | TBD | Carrier information of each pre-assigned secondary carrier, which includes: Carrier Status Bitmap: indicating whether this pre-assigned carrier will be activated immediately after HO procedure is done; Physical carrier index of pre-assigned secondary carriers; Logical carrier index, which is assigned implicitly in the order of Carrier information of each pre-assigned secondary carriers. | Shall be included when N_Preassigned_Carriers > 0 |
| O | LZone Preambile Index | 7 | LZone Preamble Index for AMS to switch from MZone to LZone | Shall be included when Mode = 0b01. |
| O | Pre allocated Basic CID | 16 | Used by the AMS to derive its own primary CID and transport CID in target YBS. | May be included when HO to a LZone or a YBS |
| O | SFH delta information | Variable | Delta encoding with reference to the serving ABS. | May be included for each target ABS when Mode = 0b00 and SFH delta information is not included in the AAI_NBR-ADV message for the target ABS |
| O | S-SFH change count | 4 | S-SFH change count of the reference for the included SFH delta information.. | Shall be included for each target ABS when SFH delta information is included |

------------------------------------------------------  Text End  ---------------------------------------------------------------

Remedy 4:

In the P802.16m/D4, page 85, line 1, replace Table 687 to the following table:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 687 — AAI_NBR-ADV parameters

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Change Count | 3 | NBR-ADV Change Count | NA |
| M | Cell type | 3 | Cell type in this message 0b000 macro 0b001 micro 0b010 macro hotzone 0b011 femto 0b100 relay 0b101-0b111 reserved | NA |
| M | Total Number of AAI_NBR-ADV Segments | 4 | Total number of segments of AAI_NBR-ADV for this cell type | NA |
| M | AAI_NBR-ADV Segment Index | 4 | Indicates current segment index of this message in the specific cell type | NA |
| M | BS number M | 8 | Total number of BSs to be included in this AAI_NBR-ADV segment | NA |
| M | Starting ABS Index | 8 | Starting ABS Index is the index offset from the last ABS of the previous AAI_NBR-ADV segment. If this is the first AAI_NBR-ADV segment, the Starting ABS Index will be 0. Hence, each AAI_NBR-ADV segment has one Index which corresponds to the first ABS in that AAI_NBR-ADV segment. The order of the ABS within a segment serves as an offset from the 'Starting ABS index'. The sum of the offset and the 'Starting ABS index' serves as the ABS index. | NA |
| O | List of PHY carrier indices | 6x64 (maximum 64 phy indices) | Refer to the physical carrier index in AAI_Global-Config message | For ABS type whose system info are not included in AAI_NBR-ADV |
| O | IDCell range start | 10 |  | For ABS type whose system info are not included in AAI_NBR-ADV |
| O | IDCell range end | 10 |  | For ABS type whose system info are not included in AAI_NBR-ADV |
| M | BSID | 48 | - | Shall be included for each neighbor BS |
| M | MAC protocol versions | 8 | MAC protocol version of the BS Consistent with REV.2 definition, with new MAC protocol version 10 defined for 16m. | Shall be included for each neighbor BS |
| M | CP time | 2 | CP time of the BS 00: 1/8 01: 1/16 10: 1/4 | Shall be included for each neighbor BS |
| M | Number of carriers | 3 | Number of carriers of the BS | Shall be included for each neighbor BS |
| M | SA-PREAMBLE index | 10 | - | Shall be included for each carrier of each neighbor BS |
| M | A-PREAMBLE transmit power | 8 | - | Shall be included for each carrier of each neighbor BS |
| M | Physical carrier index | 6 | Refer to the physical carrier index in AAI_Global-Config message | Shall be included for each carrier of each neighbor BS |
| M | S-SFH change count | 4 | Indicates the value of S-SFH change count of this neighbor ABS | Shall be included for each carrier of each neighbor BS |
| M | SFH_encoding_format | 2 | 0b00: full Subpkt information 0b01: delta encoding with reference to the information of current carrier which transmits this AAI_NBR-ADV message. 0b10: no SFH included 0b11: delta encoding with reference to the information of the preceding carrier For macrocell ABS, the bitmap shall be either 0b00 or 0b01 or 0b11. | Shall be included for each neighbor BS |
| M | Control_bitmap | 3 | Each bit maps to one SFH subpacket and AAI_SCD information Bit[0]: SFH SP1 Bit[1]: SFH SP2 Bit[2]: SFH SP3 Bit[3]: reserved | Shall be included for each neighbor BS |
| O | SFH Subpkt 1 | Variable depending on FFT size | - | Shall be included for each neighbor BS if SFH_encoding_format is set to 0b00, and Control_bitmap Bit #0 is set to 1 |
| O | SFH Subpkt 2 | Variable depending on FFT size | - | Shall be included for each neighbor BS if SFH_encoding_format is set to 0b00, and Control_bitmap Bit #1 is set to 1 |
| O | SFH Subpkt 3 | variable | - | Shall be included for each neighbor BS if SFH_encoding_format is set to 0b00, and Control_bitmap Bit #2 is set to 1 |
| O | Delta information | Var. | Delta encoding, w.r.t. the reference carrier the current carrier transmitting this message if SFH_encoding format = 01, the preceding carrier if SFH_encoding_format=11) | Shall be included for each neighbor BS if SFH_encoding_format is set to 0b01 or 0b11 |
| O | Neighbor-specific trigger TLVs | Var. | Optional neighbor-specific triggers with encoding defined in Table 744 | May be included for each neighbor BS |
| O | NBR-ADV period | 10 | The period of MOB_NVR-ADV is sent in LZone. Unit: Frame. | It shall be included when the existence of neighbor WirelessMAN-OFDMA BS or LZone of ABS are indicated in AAI_NBR-ADV. |
| O | NBR-ADV offset | 4 | The frame number offset the MOB_NBR-ADV is sent from the start point of frame counting. Unit: Frame. | It shall be included when the existence of neighbor WirelessMAN-OFDMA BS or LZone of ABS are indicated in AAI_NBR-ADV. |
| O | NBR-ADV interval: | 4 | The interval of MOB_NBR-ADV is sent in LZone. Unit: Frame. | It shall be included when the existence of neighbor WirelessMAN-OFDMA BS or LZone of ABS are indicated in AAI_NBR-ADV. |

------------------------------------------------------  Text End  ---------------------------------------------------------------