| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text Changes for the Single BS MIMO Feedback (16.2.3.37, 16.3.6.5.2.4.11) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Wooram Shin, Jaesun Cha, Kwang Jae Lim ETRI 138, Gajeongno, Yuseong-gu, Daejeon 305-700, Korea | Voice: +82-42-860-6687 E-mail: w.shin@etri.re.kr |
| Re: | “P802.16m/D4,” in response to the IEEE 802.16 Working Group Letter Ballot #31: Announcement, IEEE 802.16-10/0011 |  |
| Abstract | Single BS MIMO feedback on the IEEE 802.16m Amendment Draft Standard |  |
| Purpose | To discuss and adopt the proposed text in the next revision of 802.16m draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text Changes for the Single BS MIMO Feedback (16.2.3.37, 16.3.6.5.2.4.11)

Wooram Shin, Jaesun Cha, Kwang Jae Lim

ETRI

# Introduction

In IEEE 802.16m Amendment Draft Standard [1], AAI_SingleBS_MIMO_FBK message in Section 16.2.3.37 is defined to support DL single BS MIMO feedback, whose transmission would be scheduled with the help of Feedback Polling A-MAP IE in Section 16.3.6.5.2.4.11. Meanwhile, its usage is restricted to support only MIMO feedback modes (MFMs) 2, 3, 5 and 6 excluding MFMs 0, 1, 4, and 7. MIMO feedback extend header (MFEH) is available for all the MFMs, but, it could not be transmitted in a stand-alone fashion if there is no payload packed with it in a MAC PDU. In this context, technical and editorial modifications in AAI_SingleBS_MIMO_FBK supporting all the MFMs are proposed with two options. In Option 1, MFM_bitmap size is simply extended to 8 bits to cover all the MFMs; accordingly, additional elements for MFMs 0, 1, 4, and 7 are added. For its consistency with Feedback Polling A-MAP IE, there need to modify some texts in Feedback Polling A-MAP IE. In Option 2, MFMs are segregated into two groups with respect to its resource unit (RU) type. One group consists of MFMs 0, 1, 4, and 7 for diversity allocation on DLRU or NLRU, and the other group MFMs 2, 3, 5, and 6 for localized allocation on SLRU. Using this grouping, logically, there could be two types of AAI_SingleBS_MIMO_FBK by defining RU type indication element with 1 bit in Feedback Polling A-MAP IE. Therefore, without extending MFM_bitmap size, all the MFMs can be supported, but, not simultaneously for different RU types.

# Text Proposal for inclusion in the IEEE 802.16m Amendment

Note:

The text in black: the existing text in the 802.16m Amendment Draft Standard

The text in red with strike-out: the removal of existing 802.16m Amendment Draft Standard

The text in blue with under-line: the new text added to the 802.16m Amendment Draft Standard

Option 1:

[----------------------------------------------Start of Text Proposal for Option 1----------------------------------------------]

[Change texts for AAI_SingleBS_MIMO_FBK in Section 16.2.3.37 as the following.]

The AAI_SingleBS_MIMO_FBK message format is defined in Table 713. This message is used by AMS as a response to a Feedback Polling A-MAP IE requesting to feedback the subband information for MFM 0, 1, 2, 3, 4, 5, 6 or 67. It is also used for the feedback of the transmit correlation matrix when the ABS is equipped with 2, 4, or 8 transmit antennas. Variables MaxMt, Codebook_subset, Codebook_mode, Num_best_subbands and Measurement Method Indication are indicated in the Feedback Polling A-MAP IE. Nt is the number of transmit antennas at the ABS, indicated in S-SFH SP3. Best_subbands_index is encoded as described in section 16.3.9.3.1.4. The feedback information reported in the AAI_SingleBS_MIMO_FBK message depends on currently assigned feedback processes and reporting times.

Table 713—AAI_SingleBS_MIMO_FBK message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AAI_SingleBS_MIMO_FBK_Message_Format { |  |  |
| Management Message Type | 8 |  |
| If (((MFM == 3) and (q > 0)) or ((MFM == 6) and (q > 0)) or ((MFM == 4) and (q > 0)) or ((MFM == 7) and (q > 0))){ If ((q > 0) and ((MFM == 3) or (MFM == 4) or (MFM == 6) or (MFM == 7))){ |  | MFM and long period q are indicated in Feedback Polling A-MAP IEs relevant to currently assigned feedback processes. |
| For (i=1; i <= Nt; i++){ |  |  |
| i-th diagonal entry of correlation matrix | 1 | As defined in section 16.3.7.2.5.6 |
| For (j=i+1; j <= Nt; j++){ |  |  |
| (i,j)-th entry of correlation matrix | 4 | As defined in section 16.3.7.2.5.6 |
| } |  |  |
| } |  |  |
| } |  |  |
| MFM_bitmap | 48 | Bitmap to indicate the MFMs for which the AMS is sending feedback. It shall be consistent with current feedback allocations corresponding to the MFM requested by Feedback Polling IE. LSB #0: MFM 20 LSB #1: MFM 31 LSB #2: MFM 52 LSB #3: MFM 63 LSB #4: MFM 4 LSB #5: MFM 5 LSB #6: MFM 6 LSB #7: MFM 7 |
| If (LSB #0 in MFM_bitmap == 1){ |  | MFM 0 as specified in Feedback Polling A-MAP IE |
| STC_Rate | Variable | MaxMt = 2: 1 bit MaxMt = 3 or 4: 2 bits This field shall be omitted if MaxMt = 1 |
| Wideband CQI | 4 | MCS of wideband |
| } |  |  |
| If (LSB #1 in MFM_bitmap == 1){ |  | MFM 1 as specified in Feedback Polling A-MAP IE |
| Wideband CQI | 4 | MCS of wideband |
| } |  |  |
| If (LSB #02 in MFM_bitmap == 1){ |  | MFM 2 as specified in Feedback Polling A-MAP IE |
| Best_subbands_index | Variable |  |
| STC_Rate | Variable | If Measurement Method Indication=0b0: MaxMt = 2: 1 bit MaxMt = 3 or 4: 2 bits MaxMt > 4: 3 bits This field shall be omitted if Measurement Method Indication=0b1 or if MaxMt = 1. |
| For (m=0; m < Num_best_subbands; m ++){ |  | The subbands are sorted in order of increasing logical index |
| Subband CQI | 4 | MCS of m-th subband indicated by Best_subbands index |
| } |  |  |
| } |  |  |
| If (LSB #13 in MFM_bitmap == 1){ |  | MFM 3 as specified in Feedback Polling A-MAP IE |
| Best_subbands_index | Variable |  |
| STC_Rate | Variable | MaxMt = 2: 1 bit MaxMt = 3 or 4: 2 bits MaxMt > 4: 3 bits This field shall be omitted if MaxMt = 1. |
| For (m=0; m < Num_best_subbands; m ++){ |  | The subbands are sorted in order of increasing logical index |
| Subband CQI | 4 | MCS of m-th subband indicated by Best_subbands_index |
| Subband PMI | Variable | PMI of m-th subband indicated by Best_subbands_index Nt = 2: 3 bits Nt = 4 and CS = 0b0: 6 bits Nt = 4 and CS = 0b1: 4 bits Nt = 8: 4 bits |
| } |  |  |
| } |  |  |
| If (LSB #4 in MFM_bitmap == 1){ |  | MFM 4 as specified in Feedback Polling A-MAP IE |
| STC_rate | Variable | MaxMt = 2: 1 bit MaxMt = 3 or 4: 2 bits MaxMt > 4: 3 bits This field shall be omitted if MaxMt = 1. |
| Wideband CQI | 4 | MCS of wideband |
| Wideband PMI | Variable | PMI of wideband Nt = 2: 3 bits Nt = 4 and CS = 0b0: 6 bits Nt = 4 and CS = 0b1: 4 bits Nt = 8: 4 bits |
| } |  |  |
| If (LSB #25 in MFM_bitmap == 1){ |  | MFM 5 as specified in Feedback Polling A-MAP IE |
| Best_subbands_index | variable |  |
| For (m=0; m < Num_best_subbands; m ++){ |  | The subbands are sorted in order of increasing logical index |
| Subband CQI | 4 | MCS of m-th subband indicated by Best_subbands_index |
| Stream index | Variable | Best stream index of m-th subband indicated by Best_subbands_index If Measurement Method Indication=0b0: MaxMt = 2: 1 bit MaxMt = 3 or 4: 2 bits If Measurement Method Indication=0b1: 1 bit |
| } |  |  |
| } |  |  |
| If (LSB #36 in MFM_bitmap == 1){ |  | MFM 6 as specified in Feedback Polling A-MAP IE |
| Best_subbands_index | Variable |  |
| For (m=0; m < Num_best_subbands; m ++){ |  | The subbands are sorted in order of increasing logical index |
| Subband CQI | 4 | MCS of m-th subband indicated by Best_subbands_index |
| Subband PMI | Variable | PMI of m-th subband indicated by Best_subbands_index Nt = 2: 3 bits Nt = 4 and CS = 0b0: 6 bits Nt = 4 and CS = 0b1: 4 bits Nt = 8: 4 bits |
| } |  |  |
| } |  |  |
| If (LSB #7 in MFM_bitmap == 1){ |  | MFM 7 as specified in Feedback Polling A-MAP IE |
| Wideband CQI | 4 | MCS of wideband |
| Wideband PMI | Variable | PMI of wideband Nt = 2: 3 bits Nt = 4 and CS = 0b0: 6 bits Nt = 4 and CS = 0b1: 4 bits Nt = 8: 4 bits |
| } |  |  |
| } |  |  |

[Change texts for Feedback Polling Allocation A-MAP IE in Section 16.3.6.5.2.4.11 as the following.]

The information element shown in Table 827 is used by the ABS to schedule MMOMIMO feedback transmission by the AMS. The AMS sends the MIMO feedback using a MAC control message or an extended header, depending on the requested feedback content.

When the Polling_sub_type bit is set to 0b0, a dedicated UL allocation is included in this IE. The dedicated UL allocation shall be used by the AMS to transmit feedback at the designated feedback transmission frame defined by this IE. When the Polling_sub_type bit is set to 0b1, no dedicated UL allocation is included. Instead, at the designated transmission frame defined by this IE, the AMS shall compose the feedback, and the ABS shall either include a UL allocation for the transmission using UL Basic Assignment A-MAP IE or UL Subband Assignment A-MAP IE, or the AMS shall transmit in a dedicated UL allocation assigned by a previous Feedback Polling A-MAP IE with feedback periods designating the same transmission frames.

The AMS shall send the feedback in an extended header when the following feedback information is requested:

- wideband information for MIMO feedback modes 0, 1, 4 and 7

- subband information for 1 subband for MIMO feedback modes 2, 3, 5 and 6

The AMS shall send the feedback in a MAC control message when the following feedback information is requested:

- wideband information for MIMO feedback modes 0, 1, 4 and 7

- subband information for more than one subband for MIMO feedback modes 2, 3, 5 and 6

- multi-BS feedback

The coefficients of the quantized transmit correlation matrix should be feedback in Correlation Matrix Feedback Extended Header (CMFEH) when no AAI_SingleBS_MIMO_FBK message is sent in the same packet if the ABS is equipped with 2 or 4 transmit antennas. Otherwise, the coefficients of the quantized transmit correlation matrix should be feedback in AAI_SingleBS_MIMO_FBK message. The coefficients of the quantized transmit correlation matrix shall be feedback in AAI_SingleBS_MIMO_FBK message if the ABS is equipped with 8 transmit antennas.

In case of feedback for MIMO feedback modes 0 with Measurement Method Indication = 0b0, and MIMO feedback modes 4 or 7, the AMS shall feedback the CQI for FP0 if FPS0>0, or for FPk if FPS0 > 0 FPS0 = 0, where FPk is determined by k = floor(IDCell/256) )+1 for FPCT=3 or k = floor(IDcell/384)+1 for FPCT=2.

MIMO feedback reported by an AMS in frame N pertains to measurements performed at least up to frame N-1. The first MIMO feedback information if the MIMO feedback is sent in the frame immediately following the frame in which the Feedback Polling A-MAP IE was received.

Table 827—Feedback Polling A-MAP IE*

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| Feedback_Polling_A-MAP_IE(){ |  |  |
| A-MAP IE Type | 4 | Feedback Polling A-MAP IE |
| Polling_sub_type | 1 | 0b0: uplink resource allocation or de-allocation. 0b1: feedback mode allocation or de-allocation. |
| If (Polling_sub_type == 0b0){ |  |  |
| Allocation Duration (d) | 3 | The allocation is valid for 2(d–1) superframes starting from the superframe defined by allocation relevance. If d == 0b000, the pre-scheduled feedback header transmission is released. If d == 0b111, the pre-scheduled feedback header transmission shall be valid until the ABS commands to release it. |
| If (d ==0b000){ |  | Feedback de-allocation |
| Resource Index | 11 | Confirmation of the resource index for a previously assigned persistent resource that has been deallocated 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Polling_deallocation_bitmap | 3 |  |
| HFA | 36 | HARQ feedback channel allocation to acknowledge the successful detection of this IE. |
| } else if (d != 0b000){ |  | Feedback allocation |
| ISizeOffset | 5 | Offset used to compute burst size index |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| MEF | 2 | MIMO encoder format for uplink feedback transmission Non-adaptive precoding shall be used at the AMS. 0b00: SFBC 0b01: Vertical encoding with Mt = 2, or Mt =1 if AMS has 1 tranmsit antenna 0b10: CSM with TNS = 2, Mt =1, SI =1 0b11: CSM with TNS = 2, Mt =1, SI =2 |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: AAI subframe (default) 0b1: 4 UL AAI subframes for FDD or all UL AAI subframes for TDD If number of DL AAI subframes, D, is less than number of UL AAI subframes, U, Long TTI Indicator = 0b1 |
| HFA | 3 | HARQ feedback channel allocation for Feedback Channel De-allocation confirmation |
| ACID | 4 | HARQ channel identifier |
| MFM_allocation_index | 2 | 0b00: MFM 0 with Measurement Method Indication = 0 0b01: MFM 3 with all subbands 0b10: MFM 6 with all subbands 0b11: MFM is defined in Feedback Polling A-MAP IE with Polling_sub-type = 0b1 |
| MaxMt | 1~2 |  |
| Period | 4 | Resource is allocated at frames designated by every short and long period. The short feedback period is p frames. The long feedback period is q superframes. The first allocation shall start two frames later. The AAI subframe index is defined as in 16.2.14.2.2 and the AAI frame index is given by i+2, where i is the index of the frame where the Feedback Polling A-MAP IE is transmitted. The feedback of MIMO feedback modes in MFM_allocation_index is allocated on the short period. The feedback of the transmit correlation matrix is allocated on the long period if q > 0. Short and long period reports shall start at the first allocation. When short and long period feedback reports coincide in the same frame, both short period feedback content and long period feedback content shall be sent in the same burst. 0b0000: p = 1, q = 0 0b0001: p = 2, q = 0 0b0010: p = 4, q = 0 0b0011: p = 8, q = 0 0b0100: p = 16, q = 0 0b0101: p = 1, q = 1 0b0110: p = 2, q = 1 0b0111: p = 1, q = 2 0b1000: p = 2, q = 2 0b1001: p = 4, q = 2 0b1010: p = 1, q = 4 0b1011: p = 2, q = 4 0b1100: p = 4, q = 4 0b1101: p = 0, q = 1 0b1110: p = 0, q = 2 0b1111: p = 0, q = 4 |
| } else { |  | Polling_sub_type == 0b1 |
| ACK Allocation Flag | 1 |  |
| If (ACK Allocation Flag == 0b1){ |  |  |
| HFA | 6 | HARQ feedback channel allocation to acknowledge the successful detection of this IE. |
| } |  |  |
| Allocation Duration (d) | 3 | The allocation is valid for 2(d–1) superframes starting from the superframe defined by allocation relevance. If d == 0b000, the pre-scheduled feedback header transmission is released. If d == 0b111, the pre-scheduled feedback header transmission shall be valid until the BS commands to release it. |
| If (d == 0b000){ |  | Feedback de-allocation |
| Polling_deallocation_bitmap | 3 |  |
| } else { |  | Feedback allocation |
| MIMO_feedback_IE_type | 1 | 0b0: feedback allocation for single-BS MIMO operation 0b1: feedback allocation for multi-BS MIMO operation |
| If (MIMO_feedback_IE_type == 0b0){ |  | Single-BS MIMO feedback request 0b0: feedback allocation for single-BS MIMO operation 0b1: feedback allocation for multi-BS MIMO operation |
| MFM_bitmap | 8 | If a currently allocated MFM is indicated in the MFM_bitmap, it indicates a deallocation and reallocation of this MFM. ACK Allocation Flag shall be set to 0b1 in this case. |
| Period | 4 | 16 combination of short feedback period p (frames) and long feedback period q (superframes). The first report shall start two frames later. The AAI subframe index is defined as in 16.2.14.2.2 and the AAI frame index is given by i+2, where i is the index of the frame where the Feedback Polling A-MAP IE is transmitted. The short period feedback shall be included in the first report. The long period feedback shall also be included in the first report if it is assigned. When short and long period feedback reports coincide in the same frame, both short period feedback content and long period feedback content shall be sent in the same burst. 0b0000: p = 1, q = 0 0b0001: p = 2, q = 0 0b0010: p = 4, q = 0 0b0011: p = 8, q = 0 0b0100: p = 16, q = 0 0b0101: p = 1, q = 1 0b0110: p = 2, q = 1 0b0111: p = 1, q = 2 0b1000: p = 2, q = 2 0b1001: p = 4, q = 2 0b1010: p = 1, q = 4 0b1011: p = 2, q = 4 0b1100: p = 4, q = 4 0b1101: p = 0, q = 1 0b1110: p = 0, q = 2 0b1111: p = 0, q = 4 |
| If (LSB #0 in MFM_bitmap == 1){ |  | MFM 0 |
| MaxMt | 1~2 |  |
| Measurement Method Indication | 1 | 0b0: Use the midamble for CQI measurements 0b1: Use pilots in OL region with MaxMt streams for CQI measurements |
| } |  |  |
| If (LSB #1 in MFM_bitmap == 1){ |  | MFM 1 |
| MaxMt | 2 |  |
| Measurement Method Indication | 1 | 0b0: Reserved 0b1: Use pilots in OL region with MaxMt streams for CQI measurements |
| } |  |  |
| If (LSB #2 in MFM_bitmap == 1){ |  | MFM 2 |
| MaxMt | 1~2 |  |
| Num_best_subbands | 2 | 0b00: report all subbands 0b01: 1 best subband 0b10: min{6, YSB} best subbands 0b11: min{12, YSB} best subbands 1≤ Num_best_subbands ≤ YSB |
| Measurement Method Indication | 1 | 0b0: Use the midamble for CQI measurements 0b1: Use pilots in OL region with MaxMt streams for CQI measurements |
| } |  |  |
| If (LSB #3 in MFM_bitmap == 1){ |  | MFM 3 |
| MaxMt | 1~2 |  |
| Num_best_subbands | 2 | 0b00: report all subbands 0b01: 1 best subband 0b10: min{6, YSB} best subbands 0b11: min{12, YSB} best subbands 1≤ Num_best_subbands ≤ YSB |
| If (q == 0){ |  |  |
| Codebook_coordination | 1 | 0b0: base mode with codebook coordination disabled 0b1: base mode with codebook coordination enabled |
| } |  |  |
| If (Nt == 4){ |  |  |
| Codebook_subset | 1 | 0b0: report PMI from the base codebook or transformed base codebook 0b1: report PMI from codebook subset or transformed codebook subset |
| } |  |  |
| } |  |  |
| If (LSB #4 in MFM_bitmap == 1){ |  | MFM 4 |
| MaxMt | 1~2 |  |
| If (q == 0){ |  |  |
| Codebook_coordination | 1 | 0b0: base mode with codebook coordination disabled 0b1: base mode with codebook coordination enabled |
| } |  |  |
| If (Nt == 4){ |  |  |
| Codebook_subset | 1 | 0b0: report PMI from the base codebook 0b1: report PMI from the codebook subset |
| } |  |  |
| } |  |  |
| If (LSB #5 in MFM_bitmap == 1){ |  | MFM 5 |
| MaxMt | 1~2 |  |
| Num_best_subbands | 2 | 0b00: report all subbands 0b01: 1 best subband 0b10: min{6, YSB} best subbands 0b11: min{12, YSB} best subbands 1≤ Num_best_subbands ≤ YSB |
| Measurement Method Indication | 1 | 0b0: Use the midamble for CQI measurements 0b1: Use pilots in OL region with MaxMt streams for CQI measurements |
| } |  |  |
| If (LSB #6 in MFM_bitmap == 1){ |  | MFM 6 |
| MaxMt | 1~2 |  |
| Num_best_subbands | 2 | 0b00: report all subbands 0b01: 1 best subband 0b10: min{6, YSB} best subbands 0b11: min{12, YSB} best subbands 1≤ Num_best_subbands ≤ YSB |
| If (q == 0){ |  |  |
| Codebook_coordination | 1 | 0b0: base mode with codebook coordination disabled 0b1: base mode with codebook coordination enabled |
| } |  |  |
| If (Nt == 4){ |  |  |
| Codebook_subset | 1 | 0b0: report PMI from the base codebook or transformed base codebook 0b1: report PMI from codebook subset or transformed codebook subset |
| } |  |  |
| } |  |  |
| If (LSB #7 in MFM_bitmap == 1){ |  | MFM 7 |
| MaxMt | 1~2 |  |
| If (q == 0){ |  |  |
| Codebook_coordination | 1 | 0b0: base mode with codebook coordination disabled 0b1: base mode with codebook coordination enabled |
| } |  |  |
| If (Nt == 4){ |  |  |
| Codebook_subset | 1 | 0b0: report PMI from the base codebook 0b1: report PMI from the codebook subset |
| } |  |  |
| } |  |  |
| } else { |  | Multi-BS MIMO feedback request |
| Period (p) | 3 | Transmit feedback header every 4×p frames. The first report shall start at the next frame. |
| TRU | 2 | Target RU indicating which RUs or which type of RU to work on for feedback 0b00: Latest best subbands reported for single BS MIMO 0b01: Whole bandwidth 0b10: FFR partition 0 0b11: boosted FFR partition |
| ICT | 2 | 0b00: PMI restriction for single-BS precoding; 0b01: PMI recommendation for single-BS precoding; 0b10: CL-MD for multi-BS precoding; 0b11: Co-MIMO for multi-BS precoding; |
| If (Nt == 4){ |  |  |
| Codebook_subset | 1 | 0b0: report PMI from the base codebook 0b1: report PMI from the base codebook subset |
| } |  |  |
| N_multiBS_reports | 3 | N_multiBS_reports indicates the number of reports. |
| If (ICT = 0b11) { |  |  |
| MaxUser | 2 | Maximum number of users supported in Co-MIMO in the same resource. 0b00: 2 users 0b01: 3 users 0b10: 4 users 0b11: reserved |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| Padding | Variable | Padding to reach byte boundary |
| } |  |  |

* A 16 bit CRC is generated based on the contents of the Feedback Polling A-MAP IE. The CRC is masked by the Station ID

Parameters in the Feedback Polling A-MAP IE should be interpreted as for the Feedback Allocation A-MAP IE.

MaxMt: The possible values of MaxMt follow the rules of the Feedback Allocation A-MAP IE.

ACID: HARQ channel identifier. If q=0 or p=0, a single ACID is reserved. Otherwise, two ACIDs are reserved. The second ACID is obtained as ACID+1 modulo 1516. When two feedback periods are allocated, the AMS shall send packets at the long period feedback reports with the first ACID, and the AMS shall send packets at the short period feedback reports with the second ACID.

For uplink retransmissions of MIMO feedback, the ABS shall allocate an uplink resource with the UL Basic Assignment A-MAP IE and the ACID corresponding to the packet to retransmit. Retransmissions shall obey the following rules:

  - If the retransmission process for the previous HARQ burst is not finished before a new HARQ burst with the same ACID is transmitted, the retransmission process for the previous HARQ burst is terminated and the new HARQ burst overrides it.

  - If the retransmission process for the previous HARQ burst reporting short-period feedback is not finished before a new HARQ burst reporting long-period feedback is transmitted, the retransmission process for the previous HARQ burst reporting short-period feedback is terminated and the new HARQ burst overrides it.

  - If the retransmission process for the previous HARQ burst reporting long-period feedback is not finished before a new HARQ burst reporting short-period feedback is transmitted, the retransmission process for the previous HARQ burst reporting shortlong-period feedback is carried on, but the short-period feedback content of the retransmitted packet shall be discarded by the ABS and the new HARQ burst reporting short-period feedback overrides it.

Polling_deallocation_bitmap: An AMS supports a maximum of 3 distinct concurrent feedback allocations, including one or several MFMs, the transmit correlation matrix, and multiBS MIMO feedback. Ordering of concurrent allocations in the Polling_deallocation_bitmap shall follow the order MFM i, MFM j (j>i), MFM k (k>j), transmit correlation matrix, multiBS MIMO feedback, as shown in Table 837.

Table 837—Polling Deallocation Bitmap

| Polling_deallocation_bitmap bit #2 (MSB) | Polling_deallocation_bitmap bit #1 | Polling_deallocation_bitmap bit #0 (LSB) |
| --- | --- | --- |
| MFM i | MFM j (j>i) | MFM k (k>j) |
| MFM i | MFM j (j>i) | Transmit correlation matrix |
| MFM i | MFM j (j>i) | multiBS MIMO feedback |
| MFM i | Transmit correlation matrix | multiBS MIMO feedback |
| MFM i | MFM j (j>i) | N.A. |
| MFM i | Transmit correlation matrix | N.A. |
| MFM i | multiBS MIMO feedback | N.A. |
| Transmit correlation matrix | multiBS MIMO feedback | N.A. |
| MFM i | N.A. | N.A. |
| Transmit correlation matrix | N.A. | N.A. |
| multiBS MIMO feedback | N.A. | N.A. |

Period: The setting of short and long periods p and q when Polling_sub_type == 0b0 and MIMO_feedback_IE_type == 0b0 shall obey the rules below.

For MFM 0 or 1: When p > 0, q = 0, the wideband CQI and/or wideband STC rate shall be transmitted using short feedback period. p > 0 and q > 0 shall not be set at the same time with MFM 0 or 1.

For MFM 2 or 5: When p > 0, q = 0, the Best_subbands_index, subband CQI, and STC rate or Stream index shall be transmitted using short feedback period. p > 0 and q > 0 shall not be set at the same time with MFM 2 or 5.

For MFM 3: when p > 0 and q = 0, the Best_subbands_index, STC_Rate, subband CQI and subband PMI from the base codebook shall be transmitted using short period. When p = 0 and q > 0 p > 0 and q = 0, only the correlation matrix shall be transmitted using long feedback period. When p > 0 and q > 0, the Best_subbands_index, STC_Rate, subband CQI and subband PMI from the transformed codebook shall be transmitted using short period, while the correlation matrix shall be transmitted using long period.

For MFM 6: when p > 0 and q > 0, the Best_subbands_index, STC_Rate, subband CQI and subband PMI from the transformed codebook shall be transmitted using short period, while the correlation matrix shall be transmitted using long period. When p = 0 and q > 0, only the correlation matrix shall be transmitted using long feedback period. The ABS shall not set p > 0 and q = 0 simultaneously unless it has previously allocated feedback for the correlation matrix, in which case the Best_subbands_index, STC_Rate, subband CQI and subband PMI from the transformed codebook shall be transmitted using short period.

For MFM 4: when p > 0 and q = 0, the wideband CQI, STC rate and PMI from the base codebook shall be transmitted using short feedback period. When p > 0 and q > 0, the wideband CQI and STC rate shall be transmitted using short feedback period, and the correlation matrix shall be transmitted using long feedback period. When p = 0 and q > 0, only the correlation matrix shall be transmitted using long feedback period.

For MFM 7: when p > 0 and q = 0, the wideband CQI and PMI from the base codebook shall be transmitted using short feedback period. When p > 0 and q > 0, the wideband CQI shall be transmitted using short feedback period, and the correlation matrix shall be transmitted using long feedback period. When p = 0 and q > 0, only the correlation matrix shall be transmitted using long feedback period.

[-----------------------------------------------End of Text Proposal for Option 1----------------------------------------------]

Option 2:

[----------------------------------------------Start of Text Proposal for Option 2----------------------------------------------]

[Change texts for AAI_SingleBS_MIMO_FBK in Section 16.2.3.37 as the following.]

The AAI_SingleBS_MIMO_FBK message format is defined in Table 713. This message is used by AMS as a response to a Feedback Polling A-MAP IE requesting to feedback the subband information for MFM 0, 1, 2, 3, 4, 5, 6 or 67. Hereupon, MFMs is divided into two groups, one with MFMs 0, 1, 4, and 7, and the other with MFMs 2, 3, 5, and 6, depending on RU_type_indication defined in Feedback Polling A-MAP IE. The feedback information for the MFMs having the same RU_type_indication value is simultaneously fed back via this message. It is also used for the feedback of the transmit correlation matrix when the ABS is equipped with 2, 4, or 8 transmit antennas. Variables MaxMt, Codebook_subset, Codebook_mode, Num_best_subbands and Measurement Method Indication are indicated in the Feedback Polling A-MAP IE. Nt is the number of transmit antennas at the ABS, indicated in S-SFH SP3. Best_subbands_index is encoded as described in section 16.3.9.3.1.4. The feedback information reported in the AAI_SingleBS_MIMO_FBK message depends on currently assigned feedback processes and reporting times.

Table 713—AAI_SingleBS_MIMO_FBK message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AAI_SingleBS_MIMO_FBK_Message_Format { |  |  |
| Management Message Type | 8 |  |
| If (((MFM == 3) and (q > 0)) or ((MFM == 6) and (q > 0)) or ((MFM == 4) and (q > 0)) or ((MFM == 7) and (q > 0))){ If ((q > 0) and ((MFM == 3) or (MFM == 4) or (MFM == 6) or (MFM == 7))){ |  | MFM and long period q are indicated in Feedback Polling A-MAP IEs relevant to currently assigned feedback processes. |
| For (i=1; i <= Nt; i++){ |  |  |
| i-th diagonal entry of correlation matrix | 1 | As defined in section 16.3.7.2.5.6 |
| For (j=i+1; j <= Nt; j++){ |  |  |
| (i,j)-th entry of correlation matrix | 4 | As defined in section 16.3.7.2.5.6 |
| } |  |  |
| } |  |  |
| } |  |  |
| MFM_bitmap | 48 | Bitmap to indicate the MFMs for which the AMS is sending feedback. It shall be consistent with current feedback allocations corresponding to the MFM requested by Feedback Polling IE. If RU_type_indication = 0b0: LSB #0: MFM 0 LSB #1: MFM 1 LSB #2: MFM 4 LSB #3: MFM 7 If RU_type_indication = 0b1: LSB #1: MFM 2 LSB #2: MFM 3 LSB #3: MFM 5 LSB #4: MFM 6 |
| If (RU_type_indication == 0){ |  | As defined in Feedback Polling A-MAP IE |
| If (LSB #0 in MFM_bitmap == 1){ |  | MFM 0 as specified in Feedback Polling A-MAP IE |
| STC_Rate | Variable | MaxMt = 2: 1 bit MaxMt = 3 or 4: 2 bits This field shall be omitted if MaxMt = 1 |
| Wideband CQI | 4 | MCS of wideband |
| } |  |  |
| If (LSB #1 in MFM_bitmap == 1){ |  | MFM 1 as specified in Feedback Polling A-MAP IE |
| Wideband CQI | 4 | MCS of wideband |
| } |  |  |
| If (LSB #4 in MFM_bitmap == 1){ |  | MFM 4 as specified in Feedback Polling A-MAP IE |
| STC_rate | Variable | MaxMt = 2: 1 bit MaxMt = 3 or 4: 2 bits MaxMt > 4: 3 bits This field shall be omitted if MaxMt = 1. |
| Wideband CQI | 4 | MCS of wideband |
| Wideband PMI | Variable | PMI of wideband Nt = 2: 3 bits Nt = 4 and CS = 0b0: 6 bits Nt = 4 and CS = 0b1: 4 bits Nt = 8: 4 bits |
| } |  |  |
| If (LSB #7 in MFM_bitmap == 1){ |  | MFM 7 as specified in Feedback Polling A-MAP IE |
| Wideband CQI | 4 | MCS of wideband |
| Wideband PMI | Variable | PMI of wideband Nt = 2: 3 bits Nt = 4 and CS = 0b0: 6 bits Nt = 4 and CS = 0b1: 4 bits Nt = 8: 4 bits |
| } |  |  |
| } else { |  |  |
| If (LSB #0 in MFM_bitmap == 1){ |  | MFM 2 as specified in Feedback Polling A-MAP IE |
| Best_subbands_index | Variable |  |
| STC_Rate | Variable | If Measurement Method Indication=0b0: MaxMt = 2: 1 bit MaxMt = 3 or 4: 2 bits MaxMt > 4: 3 bits This field shall be omitted if Measurement Method Indication=0b1 or if MaxMt = 1. |
| For (m=0; m < Num_best_subbands; m ++){ |  | The subbands are sorted in order of increasing logical index |
| Subband CQI | 4 | MCS of m-th subband indicated by Best_subbands index |
| } |  |  |
| } |  |  |
| If (LSB #1 in MFM_bitmap == 1){ |  | MFM 3 as specified in Feedback Polling A-MAP IE |
| Best_subbands_index | Variable |  |
| STC_Rate | Variable | MaxMt = 2: 1 bit MaxMt = 3 or 4: 2 bits MaxMt > 4: 3 bits This field shall be omitted if MaxMt = 1. |
| For (m=0; m < Num_best_subbands; m ++){ |  | The subbands are sorted in order of increasing logical index |
| Subband CQI | 4 | MCS of m-th subband indicated by Best_subbands_index |
| Subband PMI | Variable | PMI of m-th subband indicated by Best_subbands_index Nt = 2: 3 bits Nt = 4 and CS = 0b0: 6 bits Nt = 4 and CS = 0b1: 4 bits Nt = 8: 4 bits |
| } |  |  |
| } |  |  |
| If (LSB #25 in MFM_bitmap == 1){ |  | MFM 5 as specified in Feedback Polling A-MAP IE |
| Best_subbands_index | variable |  |
| For (m=0; m < Num_best_subbands; m ++){ |  | The subbands are sorted in order of increasing logical index |
| Subband CQI | 4 | MCS of m-th subband indicated by Best_subbands_index |
| Stream index | Variable | Best stream index of m-th subband indicated by Best_subbands_index If Measurement Method Indication=0b0: MaxMt = 2: 1 bit MaxMt = 3 or 4: 2 bits If Measurement Method Indication=0b1: 1 bit |
| } |  |  |
| } |  |  |
| If (LSB #36 in MFM_bitmap == 1){ |  | MFM 6 as specified in Feedback Polling A-MAP IE |
| Best_subbands_index | Variable |  |
| For (m=0; m < Num_best_subbands; m ++){ |  | The subbands are sorted in order of increasing logical index |
| Subband CQI | 4 | MCS of m-th subband indicated by Best_subbands_index |
| Subband PMI | Variable | PMI of m-th subband indicated by Best_subbands_index Nt = 2: 3 bits Nt = 4 and CS = 0b0: 6 bits Nt = 4 and CS = 0b1: 4 bits Nt = 8: 4 bits |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

[Change texts for Feedback Polling Allocation A-MAP IE in Section 16.3.6.5.2.4.11 as the following.]

The information element shown in Table 827 is used by the ABS to schedule MMOMIMO feedback transmission by the AMS. The AMS sends the MIMO feedback using a MAC control message or an extended header, depending on the requested feedback content.

When the Polling_sub_type bit is set to 0b0, a dedicated UL allocation is included in this IE. The dedicated UL allocation shall be used by the AMS to transmit feedback at the designated feedback transmission frame defined by this IE. When the Polling_sub_type bit is set to 0b1, no dedicated UL allocation is included. Instead, at the designated transmission frame defined by this IE, the AMS shall compose the feedback, and the ABS shall either include a UL allocation for the transmission using UL Basic Assignment A-MAP IE or UL Subband Assignment A-MAP IE, or the AMS shall transmit in a dedicated UL allocation assigned by a previous Feedback Polling A-MAP IE with feedback periods designating the same transmission frames.

The AMS shall send the feedback in an extended header when the following feedback information is requested:

- wideband information for MIMO feedback modes 0, 1, 4 and 7

- subband information for 1 subband for MIMO feedback modes 2, 3, 5 and 6

The AMS shall send the feedback in a MAC control message when the following feedback information is requested:

- wideband information for MIMO feedback modes 0, 1, 4 and 7 if RU_type_indication is 0b0

- subband information for more than one subband for MIMO feedback modes 2, 3, 5 and 6 if RU_type_indication is 0b1

- multi-BS feedback

Note that RU_type_indication designates a type of RU composed of two groups, one with MFMs 0, 1, 4, and 7, and the other with MFMs 2, 3, 5, and 6, according whether it is diversity allocation or localized allocation. It is used to efficiently feed back information by grouping MFMs depending on the type of RU. Information for MFMs 0, 1, 4, and 7 for DLRU and NLRU is fed back if RU_type_indication is set to 0b0, and that for MFMs 2, 3, 5, and 6 for SLRU otherwise. The ABS can set the RU_type_indication value in consideration with resource allocation favorably requiring a specific type of RU for each AMS.

The coefficients of the quantized transmit correlation matrix should be feedback in Correlation Matrix Feedback Extended Header (CMFEH) when no AAI_SingleBS_MIMO_FBK message is sent in the same packet if the ABS is equipped with 2 or 4 transmit antennas. Otherwise, the coefficients of the quantized transmit correlation matrix should be feedback in AAI_SingleBS_MIMO_FBK message. The coefficients of the quantized transmit correlation matrix shall be feedback in AAI_SingleBS_MIMO_FBK message if the ABS is equipped with 8 transmit antennas.

In case of feedback for MIMO feedback modes 0 with Measurement Method Indication = 0b0, and MIMO feedback modes 4 or 7, the AMS shall feedback the CQI for FP0 if FPS0>0, or for FPk if FPS0 > 0 FPS0 = 0, where FPk is determined by k = floor(IDCell/256) )+1 for FPCT=3 or k = floor(IDcell/384)+1 for FPCT=2.

MIMO feedback reported by an AMS in frame N pertains to measurements performed at least up to frame N-1. The first MIMO feedback information if the MIMO feedback is sent in the frame immediately following the frame in which the Feedback Polling A-MAP IE was received.

Table 827—Feedback Polling A-MAP IE*

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| Feedback_Polling_A-MAP_IE(){ |  |  |
| A-MAP IE Type | 4 | Feedback Polling A-MAP IE |
| Polling_sub_type | 1 | 0b0: uplink resource allocation or de-allocation. 0b1: feedback mode allocation or de-allocation. |
| If (Polling_sub_type == 0b0){ |  |  |
| Allocation Duration (d) | 3 | The allocation is valid for 2(d–1) superframes starting from the superframe defined by allocation relevance. If d == 0b000, the pre-scheduled feedback header transmission is released. If d == 0b111, the pre-scheduled feedback header transmission shall be valid until the ABS commands to release it. |
| If (d ==0b000){ |  | Feedback de-allocation |
| Resource Index | 11 | Confirmation of the resource index for a previously assigned persistent resource that has been deallocated 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Polling_deallocation_bitmap | 3 |  |
| HFA | 36 | HARQ feedback channel allocation to acknowledge the successful detection of this IE. |
| } else if (d != 0b000){ |  | Feedback allocation |
| ISizeOffset | 5 | Offset used to compute burst size index |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| MEF | 2 | MIMO encoder format for uplink feedback transmission Non-adaptive precoding shall be used at the AMS. 0b00: SFBC 0b01: Vertical encoding with Mt = 2, or Mt =1 if AMS has 1 tranmsit antenna 0b10: CSM with TNS = 2, Mt =1, SI =1 0b11: CSM with TNS = 2, Mt =1, SI =2 |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: AAI subframe (default) 0b1: 4 UL AAI subframes for FDD or all UL AAI subframes for TDD If number of DL AAI subframes, D, is less than number of UL AAI subframes, U, Long TTI Indicator = 0b1 |
| HFA | 3 | HARQ feedback channel allocation for Feedback Channel De-allocation confirmation |
| ACID | 4 | HARQ channel identifier |
| MFM_allocation_index | 2 | 0b00: MFM 0 with Measurement Method Indication = 0 0b01: MFM 3 with all subbands 0b10: MFM 6 with all subbands 0b11: MFM is defined in Feedback Polling A-MAP IE with Polling_sub-type = 0b1 |
| MaxMt | 1~2 |  |
| Period | 4 | Resource is allocated at frames designated by every short and long period. The short feedback period is p frames. The long feedback period is q superframes. The first allocation shall start two frames later. The AAI subframe index is defined as in 16.2.14.2.2 and the AAI frame index is given by i+2, where i is the index of the frame where the Feedback Polling A-MAP IE is transmitted. The feedback of MIMO feedback modes in MFM_allocation_index is allocated on the short period. The feedback of the transmit correlation matrix is allocated on the long period if q > 0. Short and long period reports shall start at the first allocation. When short and long period feedback reports coincide in the same frame, both short period feedback content and long period feedback content shall be sent in the same burst. 0b0000: p = 1, q = 0 0b0001: p = 2, q = 0 0b0010: p = 4, q = 0 0b0011: p = 8, q = 0 0b0100: p = 16, q = 0 0b0101: p = 1, q = 1 0b0110: p = 2, q = 1 0b0111: p = 1, q = 2 0b1000: p = 2, q = 2 0b1001: p = 4, q = 2 0b1010: p = 1, q = 4 0b1011: p = 2, q = 4 0b1100: p = 4, q = 4 0b1101: p = 0, q = 1 0b1110: p = 0, q = 2 0b1111: p = 0, q = 4 |
| } else { |  | Polling_sub_type == 0b1 |
| ACK Allocation Flag | 1 |  |
| If (ACK Allocation Flag == 0b1){ |  |  |
| HFA | 6 | HARQ feedback channel allocation to acknowledge the successful detection of this IE. |
| } |  |  |
| Allocation Duration (d) | 3 | The allocation is valid for 2(d–1) superframes starting from the superframe defined by allocation relevance. If d == 0b000, the pre-scheduled feedback header transmission is released. If d == 0b111, the pre-scheduled feedback header transmission shall be valid until the BS commands to release it. |
| If (d == 0b000){ |  | Feedback de-allocation |
| Polling_deallocation_bitmap | 3 |  |
| } else { |  | Feedback allocation |
| MIMO_feedback_IE_type | 1 | 0b0: feedback allocation for single-BS MIMO operation 0b1: feedback allocation for multi-BS MIMO operation |
| If (MIMO_feedback_IE_type == 0b0){ |  | Single-BS MIMO feedback request 0b0: feedback allocation for single-BS MIMO operation 0b1: feedback allocation for multi-BS MIMO operation |
| RU_type_indication | 1 | In AAI_SingleBS_MIMO_FBK, the MFMs are divided into two groups, one with MFMs 0, 1, 4, and 7, and the other with MFMs 2, 3, 5, and 6, depending on the type of RU. MFMs 0, 1, 4, and 7 for DLRU and NLRU can be fed back if RU_type_indication = 0b0. Otherwise, MFMs 2, 3, 5, and 6 for SLRU can be fed back. |
| MFM_bitmap | 8 | If a currently allocated MFM is indicated in the MFM_bitmap, it indicates a deallocation and reallocation of this MFM. ACK Allocation Flag shall be set to 0b1 in this case. |
| Period | 4 | 16 combination of short feedback period p (frames) and long feedback period q (superframes). The first report shall start two frames later. The AAI subframe index is defined as in 16.2.14.2.2 and the AAI frame index is given by i+2, where i is the index of the frame where the Feedback Polling A-MAP IE is transmitted. The short period feedback shall be included in the first report. The long period feedback shall also be included in the first report if it is assigned. When short and long period feedback reports coincide in the same frame, both short period feedback content and long period feedback content shall be sent in the same burst. 0b0000: p = 1, q = 0 0b0001: p = 2, q = 0 0b0010: p = 4, q = 0 0b0011: p = 8, q = 0 0b0100: p = 16, q = 0 0b0101: p = 1, q = 1 0b0110: p = 2, q = 1 0b0111: p = 1, q = 2 0b1000: p = 2, q = 2 0b1001: p = 4, q = 2 0b1010: p = 1, q = 4 0b1011: p = 2, q = 4 0b1100: p = 4, q = 4 0b1101: p = 0, q = 1 0b1110: p = 0, q = 2 0b1111: p = 0, q = 4 |
| If (LSB #0 in MFM_bitmap == 1){ |  | MFM 0 |
| MaxMt | 1~2 |  |
| Measurement Method Indication | 1 | 0b0: Use the midamble for CQI measurements 0b1: Use pilots in OL region with MaxMt streams for CQI measurements |
| } |  |  |
| If (LSB #1 in MFM_bitmap == 1){ |  | MFM 1 |
| MaxMt | 2 |  |
| Measurement Method Indication | 1 | 0b0: Reserved 0b1: Use pilots in OL region with MaxMt streams for CQI measurements |
| } |  |  |
| If (LSB #2 in MFM_bitmap == 1){ |  | MFM 2 |
| MaxMt | 1~2 |  |
| Num_best_subbands | 2 | 0b00: report all subbands 0b01: 1 best subband 0b10: min{6, YSB} best subbands 0b11: min{12, YSB} best subbands 1≤ Num_best_subbands ≤ YSB |
| Measurement Method Indication | 1 | 0b0: Use the midamble for CQI measurements 0b1: Use pilots in OL region with MaxMt streams for CQI measurements |
| } |  |  |
| If (LSB #3 in MFM_bitmap == 1){ |  | MFM 3 |
| MaxMt | 1~2 |  |
| Num_best_subbands | 2 | 0b00: report all subbands 0b01: 1 best subband 0b10: min{6, YSB} best subbands 0b11: min{12, YSB} best subbands 1≤ Num_best_subbands ≤ YSB |
| If (q == 0){ |  |  |
| Codebook_coordination | 1 | 0b0: base mode with codebook coordination disabled 0b1: base mode with codebook coordination enabled |
| } |  |  |
| If (Nt == 4){ |  |  |
| Codebook_subset | 1 | 0b0: report PMI from the base codebook or transformed base codebook 0b1: report PMI from codebook subset or transformed codebook subset |
| } |  |  |
| } |  |  |
| If (LSB #4 in MFM_bitmap == 1){ |  | MFM 4 |
| MaxMt | 1~2 |  |
| If (q == 0){ |  |  |
| Codebook_coordination | 1 | 0b0: base mode with codebook coordination disabled 0b1: base mode with codebook coordination enabled |
| } |  |  |
| If (Nt == 4){ |  |  |
| Codebook_subset | 1 | 0b0: report PMI from the base codebook 0b1: report PMI from the codebook subset |
| } |  |  |
| } |  |  |
| If (LSB #5 in MFM_bitmap == 1){ |  | MFM 5 |
| MaxMt | 1~2 |  |
| Num_best_subbands | 2 | 0b00: report all subbands 0b01: 1 best subband 0b10: min{6, YSB} best subbands 0b11: min{12, YSB} best subbands 1≤ Num_best_subbands ≤ YSB |
| Measurement Method Indication | 1 | 0b0: Use the midamble for CQI measurements 0b1: Use pilots in OL region with MaxMt streams for CQI measurements |
| } |  |  |
| If (LSB #6 in MFM_bitmap == 1){ |  | MFM 6 |
| MaxMt | 1~2 |  |
| Num_best_subbands | 2 | 0b00: report all subbands 0b01: 1 best subband 0b10: min{6, YSB} best subbands 0b11: min{12, YSB} best subbands 1≤ Num_best_subbands ≤ YSB |
| If (q == 0){ |  |  |
| Codebook_coordination | 1 | 0b0: base mode with codebook coordination disabled 0b1: base mode with codebook coordination enabled |
| } |  |  |
| If (Nt == 4){ |  |  |
| Codebook_subset | 1 | 0b0: report PMI from the base codebook or transformed base codebook 0b1: report PMI from codebook subset or transformed codebook subset |
| } |  |  |
| } |  |  |
| If (LSB #7 in MFM_bitmap == 1){ |  | MFM 7 |
| MaxMt | 1~2 |  |
| If (q == 0){ |  |  |
| Codebook_coordination | 1 | 0b0: base mode with codebook coordination disabled 0b1: base mode with codebook coordination enabled |
| } |  |  |
| If (Nt == 4){ |  |  |
| Codebook_subset | 1 | 0b0: report PMI from the base codebook 0b1: report PMI from the codebook subset |
| } |  |  |
| } |  |  |
| } else { |  | Multi-BS MIMO feedback request |
| Period (p) | 3 | Transmit feedback header every 4×p frames. The first report shall start at the next frame. |
| TRU | 2 | Target RU indicating which RUs or which type of RU to work on for feedback 0b00: Latest best subbands reported for single BS MIMO 0b01: Whole bandwidth 0b10: FFR partition 0 0b11: boosted FFR partition |
| ICT | 2 | 0b00: PMI restriction for single-BS precoding; 0b01: PMI recommendation for single-BS precoding; 0b10: CL-MD for multi-BS precoding; 0b11: Co-MIMO for multi-BS precoding; |
| If (Nt == 4){ |  |  |
| Codebook_subset | 1 | 0b0: report PMI from the base codebook 0b1: report PMI from the base codebook subset |
| } |  |  |
| N_multiBS_reports | 3 | N_multiBS_reports indicates the number of reports. |
| If (ICT = 0b11) { |  |  |
| MaxUser | 2 | Maximum number of users supported in Co-MIMO in the same resource. 0b00: 2 users 0b01: 3 users 0b10: 4 users 0b11: reserved |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| Padding | Variable | Padding to reach byte boundary |
| } |  |  |

* A 16 bit CRC is generated based on the contents of the Feedback Polling A-MAP IE. The CRC is masked by the Station ID

Parameters in the Feedback Polling A-MAP IE should be interpreted as for the Feedback Allocation A-MAP IE.

MaxMt: The possible values of MaxMt follow the rules of the Feedback Allocation A-MAP IE.

ACID: HARQ channel identifier. If q=0 or p=0, a single ACID is reserved. Otherwise, two ACIDs are reserved. The second ACID is obtained as ACID+1 modulo 1516. When two feedback periods are allocated, the AMS shall send packets at the long period feedback reports with the first ACID, and the AMS shall send packets at the short period feedback reports with the second ACID.

For uplink retransmissions of MIMO feedback, the ABS shall allocate an uplink resource with the UL Basic Assignment A-MAP IE and the ACID corresponding to the packet to retransmit. Retransmissions shall obey the following rules:

  - If the retransmission process for the previous HARQ burst is not finished before a new HARQ burst with the same ACID is transmitted, the retransmission process for the previous HARQ burst is terminated and the new HARQ burst overrides it.

  - If the retransmission process for the previous HARQ burst reporting short-period feedback is not finished before a new HARQ burst reporting long-period feedback is transmitted, the retransmission process for the previous HARQ burst reporting short-period feedback is terminated and the new HARQ burst overrides it.

  - If the retransmission process for the previous HARQ burst reporting long-period feedback is not finished before a new HARQ burst reporting short-period feedback is transmitted, the retransmission process for the previous HARQ burst reporting shortlong-period feedback is carried on, but the short-period feedback content of the retransmitted packet shall be discarded by the ABS and the new HARQ burst reporting short-period feedback overrides it.

Polling_deallocation_bitmap: An AMS supports a maximum of 3 distinct concurrent feedback allocations, including one or several MFMs, the transmit correlation matrix, and multiBS MIMO feedback. Ordering of concurrent allocations in the Polling_deallocation_bitmap shall follow the order MFM i, MFM j (j>i), MFM k (k>j), transmit correlation matrix, multiBS MIMO feedback, as shown in Table 837.

Table 837—Polling Deallocation Bitmap

| Polling_deallocation_bitmap bit #2 (MSB) | Polling_deallocation_bitmap bit #1 | Polling_deallocation_bitmap bit #0 (LSB) |
| --- | --- | --- |
| MFM i | MFM j (j>i) | MFM k (k>j) |
| MFM i | MFM j (j>i) | Transmit correlation matrix |
| MFM i | MFM j (j>i) | multiBS MIMO feedback |
| MFM i | Transmit correlation matrix | multiBS MIMO feedback |
| MFM i | MFM j (j>i) | N.A. |
| MFM i | Transmit correlation matrix | N.A. |
| MFM i | multiBS MIMO feedback | N.A. |
| Transmit correlation matrix | multiBS MIMO feedback | N.A. |
| MFM i | N.A. | N.A. |
| Transmit correlation matrix | N.A. | N.A. |
| multiBS MIMO feedback | N.A. | N.A. |

Period: The setting of short and long periods p and q when Polling_sub_type == 0b0 and MIMO_feedback_IE_type == 0b0 shall obey the rules below.

For MFM 0 or 1: When p > 0, q = 0, the wideband CQI and/or wideband STC rate shall be transmitted using short feedback period. p > 0 and q > 0 shall not be set at the same time with MFM 0 or 1.

For MFM 2 or 5: When p > 0, q = 0, the Best_subbands_index, subband CQI, and STC rate or Stream index shall be transmitted using short feedback period. p > 0 and q > 0 shall not be set at the same time with MFM 2 or 5.

For MFM 3: when p > 0 and q = 0, the Best_subbands_index, STC_Rate, subband CQI and subband PMI from the base codebook shall be transmitted using short period. When p > 0 and q = 0p = 0 and q > 0, only the correlation matrix shall be transmitted using long feedback period. When p > 0 and q > 0, the Best_subbands_index, STC_Rate, subband CQI and subband PMI from the transformed codebook shall be transmitted using short period, while the correlation matrix shall be transmitted using long period.

For MFM 6: when p > 0 and q > 0, the Best_subbands_index, STC_Rate, subband CQI and subband PMI from the transformed codebook shall be transmitted using short period, while the correlation matrix shall be transmitted using long period. When p = 0 and q > 0, only the correlation matrix shall be transmitted using long feedback period. The ABS shall not set p > 0 and q = 0 simultaneously unless it has previously allocated feedback for the correlation matrix, in which case the Best_subbands_index, STC_Rate, subband CQI and subband PMI from the transformed codebook shall be transmitted using short period.

For MFM 4: when p > 0 and q = 0, the wideband CQI, STC rate and PMI from the base codebook shall be transmitted using short feedback period. When p > 0 and q > 0, the wideband CQI and STC rate shall be transmitted using short feedback period, and the correlation matrix shall be transmitted using long feedback period. When p = 0 and q > 0, only the correlation matrix shall be transmitted using long feedback period.

For MFM 7: when p > 0 and q = 0, the wideband CQI and PMI from the base codebook shall be transmitted using short feedback period. When p > 0 and q > 0, the wideband CQI shall be transmitted using short feedback period, and the correlation matrix shall be transmitted using long feedback period. When p = 0 and q > 0, only the correlation matrix shall be transmitted using long feedback period.

[-----------------------------------------------End of Text Proposal for Option 2----------------------------------------------]

# Reference

[1] IEEE P802.16m/D4, “DRAFT Amendment to IEEE Standard for Local and Metropolitan Area Networks; Part 16: Air Interface for Broadband Wireless Access Systems; Advanced Air Interface,” February 2010