| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text Changes for the UL Single BS Precoding with Multi-BS Coordination |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Wooram Shin, Jihyung Kim, Kwang Jae Lim ETRI 138, Gajeongno, Yuseong-gu, Daejeon 305-700, Korea | Voice: +82-42-860-6687 E-mail: w.shin@etri.re.kr |
| Re: | “P802.16m/D4,” in response to the IEEE 802.16 Working Group Letter Ballot #31: Announcement, IEEE 802.16-10/0011 |  |
| Abstract | Single BS precoding with multi-BS coordination on the IEEE 802.16m Amendment Draft Standard |  |
| Purpose | To discuss and adopt the proposed text in the next revision of 802.16m draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text Changes for the UL Single BS Precoding with Multi-BS Coordination

Wooram Shin, Jihyung Kim, Kwang Jae Lim

ETRI

# Introduction

In IEEE 802.16m Amendment Draft Standard [1], UL single BS precoding with multi-BS coordination is not still clear, so there need some modifications and additional information in Sections 16.5.2.1, 16.2.3, and 16.2.3.47 for clarification.

# Text Proposal for inclusion in the IEEE 802.16m Amendment

Note:

The text in black: the existing text in the 802.16m Amendment Draft Standard

The text in red with strike-out: the removal of existing 802.16m Amendment Draft Standard

The text in blue with under-line: the new text added to the 802.16m Amendment Draft Standard

In Sections 16.2.3 and 16.2.3.47, the name of MAC control message for the UL single BS precoding with multi-BS coordination does not seem to represent its functionality, so changing the name of it is proposed. Also, we suggest some editorial text changes. Finally, the values for PMImin and PCR in Section 16.2.3.47 are specified with some clarification.

[----------------------------------------------------Start of Text Proposal----------------------------------------------------]

[Chage Table 675 in Section 16.2.3 as the following.]

Table 675—MAC Control Messages

| Message name | Message description | Security | Connection |
| --- | --- | --- | --- |
| … | … | … | … |
| PGID_Info | Paging Group Advertisement | N/A | Broadcast |
| AAI_MultiBS_PMI_COMAAI_UL_MultiBS_ MIMO_SBP | Multi-BS PMI combinationUL multi-BS MIMO for single BS precoding with multi-BS coordination | <TBD> | Unicast |
| AAI_GRP-CFG | Group Cofiguration | Encypted/ICV | Unicast |
| … | … | … | … |

[Change the texts in Section 16.2.3.47 as the following.]

16.2.3.47 AAI_MultiBS_PMI_COMAAI_ UL_MultiBS_MIMO_SBP

The AAI_MultiBS_PMI_COMAAI_UL_MultiBS_MIMO_SBP message format is defined in Table 730—. This message is usedsignaled to AMS by ABS to indicate AMS the PMImin and PMI combination ratio (PCR) for the single BS precoding with multi-BS coordination.

Table 730—AAI_MultiBS_PMI_COMAAI_UL_MultiBS_MIMO_SBP message format

| Name | Length (bits) | Description |
| --- | --- | --- |
| AAI_MultiBS_PMI_COMAAI_UL_MultiBS_MIMO_SBP _Message_Format { |  |  |
| Control Message Type = xx |  |  |
| PMIMmin | 4 to 6 | PMI from the rank-1 base codebook£¨ generates minimum interference forto the neighboring ABS |
| PCR | 2 | PMI combination ratio |
| } |  |  |

[Change Table 946 in Section 16.5.2.1 as the following.]

Table 946—Control parameters for UL Multi-BS MIMO

| Parameter | Description | Value | Control channel (IE) | Notes |
| --- | --- | --- | --- | --- |
| PMImin | PMI minimizing interference to the neighboring ABS | 4 bits if Nt=2 6 bits if Nt=4 | AAI_MultiBS_PMI_COM messeageAAI_UL_MultiBS_MIMO_SBP | Selected from the rank-1 base codebook defined in 16.3.10.3 |
| PCR | PMI combination ratio | 0b00: 0.125 0b01: 0.375 0b10: 0.625 0b11: 0.875 | AAI_MultiBS_PMI_COM messeageAAI_UL_MultiBS_MIMO_SBP |  |

[Change the text in line 10 on page 712 in Section 16.5.2.1 as the following.]

The serving ABS shall inform the PMI in the UL Assignment A-MAP IEs and PMImin and PCR (PMI combination ratio) in anthe AAI_MultiBS_PMI_COMAAI_UL_MultiBS_MIMO_SBP message to the ABS.

[----------------------------------------------------End of Text Proposal----------------------------------------------------]

# Reference

[1] IEEE P802.16m/D4, “DRAFT Amendment to IEEE Standard for Local and Metropolitan Area Networks; Part 16: Air Interface for Broadband Wireless Access Systems; Advanced Air Interface,” February 2010