| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text Changes for the UL Single BS Precoding with Multi-BS Coordination (16.2.3, 16.2.3.47, 16.5.2.1) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Wooram Shin, Jihyung Kim, Kwang Jae Lim ETRI 138, Gajeongno, Yuseong-gu, Daejeon 305-700, Korea | Voice: +82-42-860-6687 E-mail: w.shin@etri.re.kr |
| Re: | “P802.16m/D4,” in response to the IEEE 802.16 Working Group Letter Ballot #31: Announcement, IEEE 802.16-10/0011 |  |
| Abstract | Single BS precoding with multi-BS coordination on the IEEE 802.16m Amendment Draft Standard |  |
| Purpose | To discuss and adopt the proposed text in the next revision of 802.16m draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text Changes for the UL Single BS Precoding with Multi-BS Coordination (16.2.3, 16.2.3.47, 16.5.2.1)

Wooram Shin, Jihyung Kim, Kwang Jae Lim

ETRI

# Introduction

In IEEE 802.16m Amendment Draft Standard [1], UL single BS precoding with multi-BS coordination is not still clear, so there need some modifications and additional information in Sections 16.5.2.1, 16.2.3, and 16.2.3.47 for clarification. In Sections 16.2.3 and 16.2.3.47, the name of MAC control message for the UL single BS precoding with multi-BS coordination does not seem to represent its functionality well, so changing the name of it is proposed, and its security is set to NULL. Also, we suggest some editorial text changes. Finally, the values for PMImin and PCR in Section 16.2.3.47 are specified.

# Text Proposal for inclusion in the IEEE 802.16m Amendment

Note:

The text in black: the existing text in the 802.16m Amendment Draft Standard

The text in red with strike-out: the removal of existing 802.16m Amendment Draft Standard

The text in blue with under-line: the new text added to the 802.16m Amendment Draft Standard

[----------------------------------------------------Start of Text Proposal----------------------------------------------------]

[Change Table 675 in Section 16.2.3 as the following.]

Table 675—MAC Control Messages

| Message name | Message description | Security | Connection |
| --- | --- | --- | --- |
| … | … | … | … |
| PGID_Info | Paging Group Advertisement | N/A | Broadcast |
| AAI_MultiBS_PMI_COMAAI_UL_MultiBS_ MIMO_SBP | Multi-BS PMI combinationUL multi-BS MIMO for single BS precoding with multi-BS coordination | NULL | Unicast |
| AAI_GRP-CFG | Group Configuration | Encrypted/ICV | Unicast |
| … | … | … | … |

[Change the texts and Table 730 in Section 16.2.3.47 as the following.]

16.2.3.47 AAI_MultiBS_PMI_COMAAI_ UL_MultiBS_MIMO_SBP

The AAI_MultiBS_PMI_COMAAI_UL_MultiBS_MIMO_SBP message format is defined in Table 730—. This message is usedsignaled to AMS by ABS to indicate AMS the PMIminmin and PMI combination ratio (PCR) for the single BS precoding with multi-BS coordination.

Table 730—AAI_MultiBS_PMI_COMAAI_UL_MultiBS_MIMO_SBP message format

| Name | Length (bits) | Description |
| --- | --- | --- |
| AAI_MultiBS_PMI_COMAAI_UL_MultiBS_MIMO_SBP _Message_Format { |  |  |
| Control Message Type = xx |  |  |
| PMIMinmin | 4 to 6 | PMI from the rank-1 base codebook£¨ generates minimum interference forto the neighboring ABS |
| PCR | 2 | PMI combination ratio |
| } |  |  |

[Change the texts and Table 946 in Section 16.5.2.1 as the following.]

16.5.2.1 Single BS precoding with Multi-BS Coordination

This subclause describes interference mitigation techniques applicable withto UL MIMO modes 2 and 4 withof adaptive codebook precoding with rank-1, with additional inter-ABS coordination mechanisms and interference measurement support. The inter-ABS coordination mechanisms in this subclause do not require data forwarding between different ABSs.

16.5.2.1.1 DL Signaling

Table 946 shows the control parameters for UL multi-BS MIMO.

Table 946—Control parameters for UL Multi-BS MIMO

| Parameter | Description | Value | Control channel (IE) | Notes |
| --- | --- | --- | --- | --- |
| PMIminmin | PMI minimizing interference to the neighboring ABS | 4 bits if Nt=2 6 bits if Nt=4 | AAI_MultiBS_PMI_COM messeageAAI_UL_MultiBS_MIMO_SBP | Selected from the rank-1 base codebook defined in 16.3.10.3 |
| PCR | PMI combination ratio | 0b00: 0.125 0b01: 0.375 0b10: 0.625 0b11: 0.875 | AAI_MultiBS_PMI_COM messeageAAI_UL_MultiBS_MIMO_SBP |  |

16.5.2.1.2 Operation procedure

Single BS precoding with Multi-BS Coordination is performed by combining two PMIs in TDD or FDD UL transmission for mitigating inter-cell interference (ICI) when CL MIMO precoding is applied for the serving and neighboring cells. One of PMIs maximizes the transmission power of the serving cell; another minimizes interference generated to the neighboring cell.

The operation procedure of PMI combination follows the steps below.

- PMI combination may be triggered by an ABS in an unsolicited manner. By using UL sounding signals, the serving and neighboring ABSs may measure channels of AMSs. Upon the channel measurements, the serving and neighboring ABSs may determine PMI maximizing transmission power and PMImin minimizing interference, respectively. The neighboring ABS may inform the serving ABS of PMImin through the backhaul network.

Those Ttwo PMIs of the predefined codebook {wi},  and , should be selected based on the following procedure:

(312)

(312)

where HsHS and HIHI denote information of channels from the AMS to its serving and corresponding neighboring ABS, respectively, and vi is the i-th codeword of the base codebook.

- The serving ABS shall inform the PMI in the UL Basic Assignment A-MAP IE, UL Subband Assignement A MAP-IE, or UL CSM Beamforming A-MAP IE, and PMImin and PCR (PMI combination ratio)PCR in anthe AAI_MultiBS_PMI_COMAAI_UL_MultiBS_MIMO_SBP message to the AMS.

The transmitted precoder W combining two precoders  should be generated as follows:

(313)

(313)

(314)

[----------------------------------------------------End of Text Proposal----------------------------------------------------]

# Reference

[1] IEEE P802.16m/D4, “DRAFT Amendment to IEEE Standard for Local and Metropolitan Area Networks; Part 16: Air Interface for Broadband Wireless Access Systems; Advanced Air Interface,” February 2010