| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text change for Feedback Allocation A-MAP IE (16.3.6) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Jinyoung Chun, Wookbong Lee LG Electronics Luciano Sarperi Fujitsu | jiny.chun@lge.com, wookbong.lee@lge.com luciano.sarperi@uk.fujitsu.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirc #31 |  |
| Abstract |  |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m D5 Draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text change for Feedback Allocation A-MAP IE (16.3.6)

Jinyoung Chun, Wookbong Lee

LG Electronics

Luciano Sarperi

Fujitsu

- Introduction

In this contribution, we propose;

  - HARQ channel allocation for deallocation and reallocation case.

  - Additional feedback format for MFM 4 and 7.

  - Some correction/clarification

Here is the performance benefit of new additional feedback format.

Simulation condition is same as ITU-R recommendation. (UMa)

Table 1 Single User MIMO spectral efficiency (bps/Hz) (MFM4)

|  | p=1 (10 ms) | p=2 (20 ms) |
| --- | --- | --- |
| Performance gain of new feedback format | 3.8% | 4.5% |

Table 2 Single User MIMO cell edge throughput (kbps) (MFM4)

|  | p=1 (10 ms) | p=2 (20 ms) |
| --- | --- | --- |
| Performance gain of new feedback format | 10.8% | 8.6% |

Table 3 Multi User MIMO spectral efficiency (bps/Hz) (MFM7)

|  | p=1 (10 ms) | p=2 (20 ms) |
| --- | --- | --- |
| Performance gain of new feedback format | 5.6% | 7.3% |

Table 4 Multi User MIMO cell edge throughput (kbps) (MFM7)

|  | p=1 (10 ms) | p=2 (20 ms) |
| --- | --- | --- |
| Performance gain of new feedback format | 8.5% | 8.8% |

- References

[1] Draft Amendment, P802.16m/D4, Feb. 2010.

Proposed text has been underlined in blue and deleted text has been struck through in red. Existing draft text is shown in black.

Remedy #1. Line 17-40, page 494

Modify text as follows;

-------------------------------------------------- Text Change starts #1 ------------------------------------------------

# 16.3.6.5.2.4.5 Feedback Allocation A-MAP IE

Table 830 describes the fields in a Feedback Allocation A-MAP IE used for dynamically allocating or de-allocating UL fast feedback control channels (including both PFBCH and SFBCH) to an AMS. If an AMS has an existing fast feedback control channel for an active DL carrier and receives a new feedback channel allocation for the same active DL carrier, the original fast feedback channel is de-allocated automatically.. And the channel index of the newly allocated FBCH is the same as the channel index of the deallocated FBCH.

Definitions of the fields in the Feedback Allocation A-MAP IE are listed below in Table 830.

Channel Index: Uniquely identifies a fast feedback channel on which an AMS can transmit fast feedback information. With this allocation, a one-to-one relationship is established between channel index and the AMS.

ACK Allocation Flag: The ABS shall set ACK Allocation Flag to 0b1 if an AMS has an existing fast feedback control channel for an active DL carrier, then ACK Allocation Flag shall be set to 0b1. The ABS may set ACK Allocation Flag to 0b1 if Allocation Duration equals 0b000. ABS may set ACK Allocation Flag to 0b1 if Allocation Duration is not equal to 0b000 if an AMS does not have an existing fast feedback control channel for an active DL carrier. and the channel index of the newly allocated FBCH is the same as the channel index of the deallocated FBCH.

-------------------------------------------------- Text Change #1 ends ------------------------------------------------

Remedy #2. Table 830

Modify table as follows;

-------------------------------------------------- Text Change #2 starts ------------------------------------------------

Table 830 - Feedback Allocation A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| Feedback-Allocation-MAP_IE() { | - | - |
| - A-MAP IE Type | 4 | - Feedback Allocation A-MAP IE |
| - Channel Index | 6 | - Feedback channel index within the UL fast feedback control resource region - (Dependent on LFB,FPi defined in section 16.3.8.3.3.2). |
| Short-term Feedback Period (p) | 3 | A feedback is transmitted on the FBCH every 2p frames. |
| Long-term Feedback Period (q) | 2 | A long-term feedback is transmitted on the FBCH every 2q short-term feedback opportunity. If q = 0b00, either the short-term or the long-term feedback shall be reported by the AMS, depending on the feedback formats defined in 16.3.9.3.1.5. |
| - Frame_number | - 2 | The AMS starts reporting at the frame which number in the superframe is equal to Frame_number. If the current frame is specified, the AMS starts reporting in four frames. Frames are numbered from 0 to 3 in the superframe. MIMO feedback reported by an AMS in frame N pertains to measurements performed at least up to frame N–1. The first MIMO feedback report following the Feedback Allocation A-MAP IE as per the “Frame_number” may contain invalid MIMO feedback information if the MIMO feedback is sent in the frame immediately following the frame in which the Feedback Allocation A-MAP IE was received. |
| - Subframe index | - 3 | - Indicate the UL subframe index in the UL portion of the frame |
| - Allocation Duration(d) | 3 | A FBCH is transmitted on the FBCH channels indexed by Channel Index for 82d frames. If d = 0b000, the FBCH is deallocated. If d=0b111, the AMS reports until the ABS command for the AMS to stop. |
| - ACK Allocation Flag | 1 | Indicate if one ACK channel is allocated to acknowledge the successful detection of this IE. If an AMS has an existing fast feedback control channel for an active DL carrier, then ACK Allocation Flag shall be set to 0b1. |
| - If (ACK Allocation Flag == 0b1){ |  |  |
| - HFA | 6 | Explicit Index for HARQ Feedback Allocation to acknowledge receipt of thisdeallocation A-MAP IE |
| } |  |  |
| - MFM | - 3 | - MIMO Feedback Mode as defined in Table 831 |
| - If (MFM ≠ 1){ |  |  |
| MaxMt | Variable 1-2 | Variable number of bits - depends on number of transmit antenna Nt - If Nt =2 (Any MFM): - 0b0: 1 - 0b1: 2 - If Nt =4 (Any MFM): - 0b00: 1 - 0b01: 2 - 0b10: 3 - 0b11: 4 - If Nt =8 (SU-MIMO MFM 0, 2, 3, 4): - 0b00: 1 - 0b01: 2 - 0b10: 4 - 0b11: 8 - If Nt =8 (MU-MIMO MFM 5, 6, 7): - 0b00: 1 - 0b01: 2 - 0b10: 3 - 0b11: 4 |
| } |  |  |
| If (MFM = 2, 3, 5, 6) { |  |  |
| - Feedback Format | 2 |  |
| } else if (MFM = 4, 7) { |  |  |
| - Feedback Format | 1 |  |
| } |  |  |
| If (MFM=0, 4,7 & FPCT>1){ |  | Diversity Permutation and FFR is enabled |
| If (FPCT=2){ |  |  |
| FPI | 1 | Frequency partition indication: ABS indicate AMS to send wideband CQI and STC rate of the frequency partition and reuse factor in the future: 0b0: Frequency partition index 1 0b1: Frequency partition index 2 ABS should set FPI to a value that FPSFPI>0 |
| } else { |  |  |
| FPI | 2 | Frequency partition indication: ABS indicate AMS to send wideband CQI and STC rate of the frequency partition and reuse factor in the future: 0b00: Frequency partition index 0 0b01: Frequency partition index 1 0b10: Frequency partition index 2 0b11: Frequency partition index 3 ABS should set FPI to a value that FPSFPI>0 |
| } |  |  |
| } |  |  |
| If (MFM == 0 and q!=0b00 & FPCT>1) |  |  |
| { |  |  |
| If (FPCT=2){ |  | When FPCT=2, Long term FPI is implicitly signaled by FPI. ABS indicates to the AMS to send wideband CQI and STC rate for the second frequency partition using long term feedback. |
| Long-Short FPI Switch Flag | 1 | Used to inform the AMS to switch Short and Long term reporting based on the FPI of the latest data allocation. 0b0: FPI for Long & Short Term Period report remains constant for the Allocation Duration (d) 0b1: FPI for Long & Short Term Period interchange after every update of FPI of latest data allocation at the subsequent Long Term Feedback Opportunity. |
| } else { |  |  |
| Long term FPI | 2 | Frequency partition indication: ABS indicates to the AMS to send wideband CQI and STC rate for the second frequency partition using long term feedback: 0b00: Frequency partition index 0 0b01: Frequency partition index 1 0b10: Frequency partition index 2 0b11: Frequency partition index 3 ABS should set Long term FPI to a different value than FPI and FPSLong term FPI>0. |
| } |  |  |
| } |  |  |
| If (MFM == 3) { |  | CL SU MIMO |
| Codebook_mode | 2 | Codebook Feedback Mode and Codebook Coordination 0b00: base mode with codebook coordination disabled 0b01: transformation mode with codebook coordination disabled 0b10: differential mode with codebook coordination disabled 0b11: base mode with codebook coordination enabled |
| If (Nt == 4) { |  |  |
| Codebook_subset | 1 | 0b0: report PMI from the base codebook or transformed base codebook 0b1: report PMI from the codebook subset or transformed codebook subset |
| } |  |  |
| } |  |  |
| If (MFM == 6) { |  | CL MU MIMO |
| Codebook_mode | 2 | Codebook Feedback Mode and Codebook Coordination 0b00: reserved 0b01: transformation mode with codebook coordination disabled 0b10: differential mode with codebook coordination disabled 0b11: base mode with codebook coordination enabled |
| If (Nt == 4) { |  |  |
| Codebook_subset | 1 | 0b0: report PMI from the base codebook or transformed base codebook 0b1: report PMI from the codebook subset or transformed codebook subset |
| } |  |  |
| } |  |  |
| If (MFM == 4,7) { |  | CL SU and MU MIMO |
| Codebook_coordination | 1 | Codebook Feedback Mode and Codebook Coordination 0b0: base mode with codebook coordination disabled 0b1: base mode with codebook coordination enabled |
| If (Nt == 4) { |  |  |
| Codebook_subset | 1 | 0b0: report PMI from the base codebook 0b1: report PMI from the codebook subset |
| } |  |  |
| } |  |  |
| If (MFM==0, 1, 2, 5){ |  |  |
| Measurement Method Indication | 1 | 0b0: Use the midamble for CQI measurements 0b1: Use pilots in OL region with MaxMt streams for CQI measurements |
| } |  |  |
| Padding | Variable | Padding to reach byte boundary |
| } | - | - |

-------------------------------------------------- Text Change #2 ends ------------------------------------------------

Remedy #3. Line 21, page 646 to line 46, page 647

Modify text as follows;

-------------------------------------------------- Text Change #3 starts  ------------------------------------------------

Feedback format for MFM 0,1,4,7

Feedback formats for MFM 0, 4, and 7 are listed in Table 910. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE, long term report will puncture short term report according to long term feedback period in feedback allocation A-MAP IE. The long period report shall start by puncturing the first short period report if MFM = 4 or 7, or by puncturing the 2q-th short period report if MFM = 0, where q is defined in the Feedback Allocation A-MAP IE. When q = 0, only the short period reports shall be sent. If the PFBCH indicator in SFBCH is set to '1', PFBCH is transmitted by puncturing SFBCH in the next feedback opportunity regardless of short-term and long-term feedback using encoding type 0. If MFM = 4 or 7 and q = 0 for feedback format = 0b0, the ABS should allocate the feedback of the transmit correlation matrix using Feedback Polling A-MAP IE before allocating MFM 4 or 7 to the AMS, where q is defined in the Feedback Allocation A-MAP IE. If MFM = 4 or 7 for feedback format = 0b1, the ABS shall set q = 0, and only the long period reports shall be sent.

The wideband CQI is one average CQI corresponding to the MIMO mode signaled by the combination MFM and STC rate, with averaging over the whole band. In the case where the number of DL frequency partitions FPCT > 1, the wideband CQI is one average CQI over the corresponding frequency partition.

When MFM = 4 or 7 and q > 0, AMS should calculate CQI assuming that ABS will calculate the beamforming matrix utilizing the latest PMI which is fed back to the ABS. When MFM = 4 or 7 and q = 0 for feedback format = 0b0, AMS should calculate CQI assuming that ABS will calculate the beamforming matrix utilizing the latest successfully received transmit correlation matrix which is fed back to the ABS.

Table 910 shows feedback formats for MIMO feedback mode 0, 4, and 7 when Measurement Method Indication = 0b0 in Feedback Allocation A-MAP IE (operation outside the open-loop region):.

Table 910 Feedback formats for MFM 0, 4 and 7 (outside OL region)

| MFM | Feedback Format | FBCH | Number of reports | Report Period | Feedback Fields | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | PFBCH | 2 | Short | Wideband CQI and STC rate or any type of EDI | Joint encoding of CQI and STC rate Encoding type 0 |
|  |  |  |  | Long | Wideband CQI and STC rate | Joint encoding of CQI and STC rate Encoding type 0 Long term FPI for FFR No long term report when q=0 |
| 4 | 0 | PFBCH | 2 | Short | Wideband CQI and STC rate or any type of EDI | Encoding type 0 |
|  |  |  |  | Long | Wideband PMI | PMI Encoding type 2 No long term report when q=0 |
|  | 1 | SFBCH | 2 | Short | Wideband CQI, Wideband STC rate, Wideband PMI, PFBCH indicator |  |
| 7 | 0 | PFBCH | 2 | Short | Wideband CQI and STC rate or any type of EDI | STC rate = 1 Encoding type 0 |
|  |  |  |  | Long | Wideband PMI | PMI for rank 1 Encoding type 2 No long term report when q=0 |
|  | 1 | SFBCH | 2 | Short | Wideband CQI, Wideband PMI, PFBCH indicator |  |

Table 911 shows feedback formats for MIMO feedback mode 0 and 1 when Measurement Method Indication = 0b1 in Feedback Allocation A-MAP IE (operation inside the open-loop region). The ABS shall not assign a feedback channel on the short periods.

Table 911 Feedback formats for MFM 0 and 1 (inside OL region)

| MFM | FBCH | Number of reports | Report Period | Feedback Fields | Description/Notes |
| --- | --- | --- | --- | --- | --- |
| 0 | PFBCH | 1 | Long | Wideband CQI and STC rate or any type of EDI | Joint encoding of CQI and STC rate Encoding type 0 |
| 1 | PFBCH | 1 | Long | Wideband CQI or any type of EDI | CQI for STC rate = 1/2 Encoding type 3 |

-------------------------------------------------- Text Change #3 ends ------------------------------------------------